5.  Analyisis of Current Situation and Projections	78

5.1	Status of PHASE-I Implementation in Arges County	78

5.2	Waste Amounts	81

5.2.1	Methodology and Assumptions	81

5.2.2	General	81

5.2.3	Total Municipal Solid Waste	82

5.2.4	Biodegradable Waste	85

5.2.5	Packaging Waste	86

5.2.6	Other Waste (Hazardous Waste, ELV, WEEE)	88

5.3	Composition of Waste	93

5.3.1	Current Situation and projection	93

5.4	Facilities and Performance	93

5.4.1	Collection and Transport	93

5.4.2	Biological Treatment of Waste	95

5.4.3	Recycling and Recovery	96

5.4.4	Existing of non-compliant disposal sites	97

5.4.5	Tariffs and Costs of Waste Management	99

5.5	Conclusion	100

# 5.  Analyisis of Current Situation and Projections

## Status of PHASE-I Implementation in Arges County

The Phase-I of IWM Project in Arges County includes 5 of the 8 areas with the most urgent needs in terms of environment protection: area 3 - Campulung, area 4 - Rucar, area 5 - Mioveni, area 6 – Pitesti and area 7 - Topoloveni. 475.909 inhabitants, meaning that 73% of the Arges county population will benefit from this stage.

This stage is 75% financed by the European Commission through the ISPA Programme, based on the Financing Memorandum signed by the European Commission on the 29th of September 2005 and by the Romanian Government on the 10th of March 2006. At the national level, the Memorandum is ratified by Order no. 561/23.08.2006 published in the Official Gazette no. 721bis of 2006.

The ISPA Measure is being implemented through five contracts, as follows:

- Contract no. 2005 Ro 16 PPE 001 – 01: Technical Assistance for the management and supervision of the ISPA contracts in the solid waste sector from Arges county;

- Contract no. 2005 Ro 16 PPE 001 – 02: Closure of uncontrolled waste landfills and platforms, construction of a waste ecological landfill and transfer station, collection system in the urban and rural areas, Arges – Romania –;

- Contract no. 2005 Ro 16 PPE 001 – 03: Service contract for independent evaluators;

- Contract no. 2005 Ro 16 PPE 001 – 04: Technical assistance for the tariff study and tender preparation for PPP operator, selection and monitoring;

- Contract no. 2005 Ro 16 PPE 001 – 05: Equipment for pre-collection, collection and transport system.

The following activities are being be carried out in the Phase-I:

| - Closure of non-compliant waste landfills from Campulung and Mioveni. |  |
| --- | --- |
| Commencement date: | Campulung (15.10.2008) & Mioveni (05.2009) |
| Status: | - Campulung. Under work - Mioveni Underwork |
| Performed work: | - Old landfill in Campulung is closed with interim cover - Mioveni is being closed with interim cover |
| Work to be done | - campulung. Inner roads, dewatering system, final cover in landfill - Mioveni. The works to be achieved in Mioveni are carried on in the same time with the operation of the existing landfill.The existing la ndfill will be closed when the first cell in Albota will be ready to recieve new waste |
| Finish date: | 10/2010 |
| - Closure of the waste landfill from Albota and construction of the first cell of the ecological landfill, with a capacity of 750,000 m3 and 5 years' lifetime |  |
| Commencement date: | 22.09.2008 |
| Status: | - Old landfill Albota: closed - Cell-1: In operation - Landfill and facilities already with temporary operator |
| Performed work: | - Closure of the old Albota landfill and the construction of a new ecologic landfill and a Utility Area |
| Work to be done | - completion of Landfill Gas Plant and network - dewatering system for surface water - waste relocation from rural areas - final sealing system for the old landfill - signaling and control installation - inner roads |
| Finish date: | 10/2010 |
| - Relocation and remediation of Topoloveni landfill and non-compliant landfill from the rural area |  |
| Status: | - Finished |
| Performed work: | - Relocation of the Topoloveni landfill waste to Albota and all of uncontrolled landfills from the rural area through the relocation of all waste to the Cell-1 of Albota landfill |
| Work to be done | - -- |
| Finish date: | - -- |
| - Reduction of the organic waste content through compost production in the composting plants from Albota-Pitesti and Campulung |  |
| Status: | - Under work |
| Performed work: | - Albota. Infrastructure and equipment ready, ready for receiving input material for composting processes - Campulung. Infrastructure and equipment ready, ready for receiving input material for composting processes |
| Work to be done | - Albota. Start operations (11/2010) - Campulung. Start operations (11/2010) |
| Finish date: | - -- |
| - Introduction of a selective collection system in the urban area by setting up and providing the collection points with recipients, setting up 4 collection centres for recyclable waste ”Green collection points” (two in Pitesti and one in Mioveni and Campulung respectively), purchasing compatible machinery for the new collection system, constructing a transfer station in Campulung with a surface of 7,000 m2 |  |
| Status: | - Works. Finished - Supply. Under work |
| Performed work: | - Collection platforms are constructed and ready for receiving containers and start operations - Waste collection equipment is already supplied |
| Work to be done | - Distribution of containers and collection equipment (09/2010) - Selection of operator for waste collection (tender in Process) - Start operations (11/2010) |
| Finish date: | - -- |
| - Introduction of the selective collection system in the rural area – setting up and equipment supply for nearly 973 platforms, fitted with 1,1 m3 containers for residual and recyclable waste, (status: starting operations in 2011); |  |
| Status: | - Works. Finished - Supply. Under work |
| Performed work: | - Collection platforms are constructed and ready for receiving containers and start operations - Waste collection equipment is already supplied |
| Work to be done | - Distribution of containers and collection equipment (09/2010) - Selection of operator for waste collection (tender in Process) - Start operations (11/2010) |
| Finish date: | - -- |
| - Purchase of machinery for the operation of the ecological waste landfill |  |
| Status: | - Finished |
| Performed work: | - Machinery for landfill purchased and delivered - Equipment already in operation |
| Work to be done | - none |
| Finish date: | - -- |
| - Preparation of documents for the implementation of the Public Private Partnerships concerning the sanitation service provided by 4 operators and administration of the new ecological waste landfill |  |
| Status: | - In progress |
| Performed work: | - Documentation prepared, concerning the implementation of the Public Private Partnerships concerning the sanitation service provided by 4 operators and administration of the new ecological waste landfill - Tender processes launched |
| Work to be done | - Selection of operators |
| Finish date: | - (12/2010) |
| - Organization of publicity and public awareness campaigns with an emphasis on the promotion of the EU financial contribution, reduction of the waste quantities deposited by selectively collecting and sorting the waste at source |  |
| Status: | - In progress |
| Performed work: | - Documentation prepared, concerning project publicity and public awareness campaign - Tender organization for required material supply (in progress) |
| Work to be done | - Selection of material suppliers - Web site of project - Distribution of publicity and public awareness material |
| Finish date: | - (12/2010) |

The implementation of Phase-I (ISPA investments) is going to clearly improve the WM in the covered zones; this will contribute to the County targets, towards the Romania’s commitments for environmental acquis. However, the fully implementation of ongoing WM projects (see previous table) can not achieve the County targets alone, as there are several WM issues to solve in the County, i.e.,

- In Phase-I zones. The composting plants were designed to treat only green waste, thus it is necessary to increase their capacity in order to achieve the LD, regarding disposal biodegradable waste

- In PHARE areas. The three ongoing projects comprise only waste collection improvements, thus further WM stages, i.e. transfer,  waste treatment and waste disposal, are still in inadequate state

- In Phase-II zones. These zones lack of any WM project for improvement, the main issues to solve are:

- Improvement of separate waste collection in urban areas

- Introduction of separate waste collection in rural areas

- Closure of 2 urban disposal sites, which deadline is by end of 2010

- Introduction of treatment procedures for organic waste fraction

- Introduction of waste sorting infrastructure

- Introduction of separate waste collection infrastructure for special waste streams

- Implementation of sound waste disposal at county landfill in Albota

in this way, the Phase-II of the IWMP of Arges County arises as a complement measure to improve the WM in the entire Arges County and achieve the County targets in WM field, and moreover, to contribute accomplishing the Romania’s commitment for Environmental acquis.

## Waste Amounts

### Methodology and Assumptions

Waste data presented at Master Plan level are a summary of data gathered from the REPA, LEPA and the local authorities. Other data, like data about collected and uncollected waste are based on assumptions. Following these same assumptions, the Table 5.1-4 shows the forecast of the amount of waste generated in the Arges County for urban and rural areas, and for different years during the period of project.

### General

The main waste streams reported by the LSWMP in Arges county, and which are scope of the present feasibility study, are:

|  | Stream | Code |
| --- | --- | --- |
| 1,0 | Municipal waste and assimilable waste from commerce, industry and institutions, out of which: | 20 15 01 |
| 1,1 | Municipal waste mixed collected from the population, out of which: | 20 03 01 |
| 1,2 | Assimilable waste mixed collected from commerce, industry and institutions | 20 03 01 |
| 1,3 | Municipal and similar waste collected separately (except construction and demolition waste) out of which: | 20 01 15 01 |
|  | Paper and cardboard | 20 01 01 15 01 01 |
|  | Glass | 20 01 02 15 01 07 |
|  | Metal | 20 01 40 15 01 04 |
|  | Plastic | 20 01 39 15 01 02 |
|  | Wood | 20 01 38 15 01 03 |
|  | Biodegradable | 20 01 08 |
|  | Others | 20 01 15 01 |
| 1,4 | Bulky waste | 20 03 07 |
| 1,5 | Waste from gardens and parks | 20 02 |
| 1,6 | Market waste | 20 03 02 |
| 1,7 | Street waste** | 20 03 03 |
| 1,8 | Generated and uncollected waste | 20 01 15 01 |
| 2,0 | Municipal and assimilable waste sorted collected (exclusively waste from demolitions and constructions)* |  |
| 3,0 | Sludge from WWTP |  |

Currently, the collection and transport of solid waste is performed basically in urban areas and in the head of communes. This service is provided by private companies or by the municipality. In the most of rural areas (villages) of Phase-II areas, this service is deficient. There are some exceptions like Domensti commune and some villages, rural areas which have contracted waste collection services with a private company.

In regard the final disposal sites existing within the Arges county, there exist no precise number of these places, nor detailed data about the amount of waste disposed and their localization. All the disposal sites identified in Arges county do not comply with the requirements of the Ministry of Environment and Forest Order 757/2004, thus they are dumping sites.

The main dumpsites identified in Arges county are in the communes of Albota, Campulung, Mioveni, Curtea de Arges and Costesti.

### Total Municipal Solid Waste

#### Generated Waste

The estimated amounts of waste generated in Arges County for urban and rural areas, and for different years, are presented in Table 5.2-1.

Table 5.2-1 Total Waste generated in Argeş County

|  | Generated Waste (ton/year) | Code | 2005 | 2007 | 2009 | 2010 | Index2007 (Kg/Inh-y) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1,0 | Municipal waste and assimilable waste from commerce, industry and institutions, out of which: | 20 15 01 | 276.925 | 279.863 | 282.655 | 284.061 | 434,41 |
| 1,1 | Municipal waste mixed generated from the population, out of which: | 20 03 01 | 158.153 | 159.880 | 161.475 | 162.279 | 248,17 |
|  | 1.1.1 Urban |  | 106.785 | 107.616 | 108.689 | 109.230 | 348,61 |
|  | 1.1.2 Rural |  | 51.368 | 52.264 | 52.786 | 53.048 | 155,76 |
| 1,2 | Assimilable waste mixed generated from commerce, industry and institutions | 20 03 01 | 87.146 | 88.034 | 88.913 | 89.355 | 136,65 |
|  | 1.2.1 Urban |  | 65.360 | 65.868 | 66.525 | 66.856 | 213,37 |
|  | 1.2.2 Rural |  | 21786,5 | 22.166 | 22.388 | 22.499 | 66,06 |
| 1,3 | Municipal and similar waste generated separately (except construction and demolition waste) out of which: | 20 01 15 01 |  |  |  |  |  |
|  | Paper and cardboard | 20 01 01 15 01 01 | 9.461 | 11.136 | 12.750 | 13.387 | 17,29 |
|  | Glass | 20 01 02 15 01 07 | 8.740 | 10.287 | 11.777 | 12.366 | 15,97 |
|  | Plastic | 20 01 39 15 01 02 | 3.608 | 4.247 | 4.862 | 5.105 | 6,59 |
|  | Metal | 20 01 40 15 01 04 | 11.626 | 13.684 | 15.667 | 16.450 | 21,24 |
|  | Wood | 20 01 38 15 01 03 | 4.811 | 5.662 | 6.483 | 6.807 | 8,79 |
|  | Biodegradable | 20 01 08 | 118.406 | 124.544 | 131.009 | 150.028 | 193,32 |
|  | Others | 20 01 15 01 | 39.203 | 40.687 | 42.272 | 46.630 | 63,16 |
| 1,4 | Bulky waste | 20 03 07 | -- | -- | -- | -- | -- |
| 1,5 | Waste from gardens and parks | 20 02 | 9.174 | 9.268 | 9.360 | 9.407 | 14,39 |
|  | Urban |  | 6.881 | 6.934 | 7.003 | 7.038 | 22,46 |
|  | Rural |  | 2.294 | 2.334 | 2.357 | 2.369 | 6,95 |
| 1,6 | Market waste | 20 03 02 | 2.452 | 2.477 | 2.502 | 2.514 | 3,84 |
|  | Urban |  | 1.839 | 1.853 | 1.872 | 1.881 | 6,00 |
|  | Rural |  | 613 | 624 | 630 | 633 | 1,86 |
| 1,7 | Street waste** | 20 03 03 | 20.000 | 20.204 | 20.405 | 20.507 | 31,36 |
|  | Urban |  | 15.000 | 15.117 | 15.267 | 15.343 | 48,97 |
|  | Rural |  | 5.000 | 5.087 | 5.138 | 5.164 | 15,16 |
|  | Kg/inh-year |  | 428,5 | 434,4 | 441 | 445 | 434,41 |
|  | Kg/inh-day |  | 1,2 | 1,2 | 1,21 | 1,22 | 1,19 |
| 1,8 | Generated and uncollected waste, out of which: | 20 01 15 01 | 74.247 | 67.590 | 60.233 | 32.284 | 104,92 |
|  | Urban |  | 0 | 0 | 0 | 0 | 0,00 |
|  | Rural |  | 74.247 | 67.590 | 60.233 | 32.284 | 201,44 |
| 2,0 | Municipal and assimilable waste sorted generated (exclusively waste from demolitions and constructions)* |  | 34.749 | 35.103 | 35.453 | 35.630 | 54,49 |
|  | Urban |  | 26.062 | 26.264 | 26.527 | 26.658 | 85,08 |
|  | Rural |  | 8.687 | 8.839 | 8.927 | 8.971 | 26,34 |
| 3,0 | Sludge from WWTP (ton) |  | ND | ND | ND | 10.000 |  |

#### Collected Waste

The Table 5.1-2 presents the collection rates for different waste types and areas in Arges County, as well as the collected amount of waste.

Table 5.2-2 Share of population serviced with waste collection service in Arges County

| Zone | Total Population served in Zones2008 (Inh.) | % Served Population | Population served in urban area (Inh.) | % Served Population | Population served in rural area (Inh.) | % Served Population | Total Generated Waste 2008 | 1Total Collected Waste2008 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Total Zone |  | Urban area |  | Rural area | Ton | Ton |
| 1-Curtea de Arges | 33.154 | 44% | 33.143 | 100% | 10,729 | 25% | 31.690 | 24.676 |
| 2-Domnesti | 5.478 | 20% | -- | -- | 5.478 | 20% | 5.592 | 341 |
| 3-Campulung | 47.831 | 59% | 37.468 | 100% | 10.363 | 21% | 34.766 | 25,146 |
| 4-Rucar | 1.822 | 19% | -- | -- | 1.822 | 19% | 2.311 | 1,435 |
| 5-Mioveni | 40.881 | 57% | 33.998 | 100% | 6.883 | 16% | 30.862 | 20,832 |
| 6-Pitesti | 203.354 | 76% | 181.917 | 100% | 21.437 | 14% | 137.634 | 99,350 |
| 7-Topoloveni | 16.996 | 42% | 10.481 | 100% | 6.515 | 27% | 13.928 | 9,882 |
| 8-Costesti | 25.470 | 37% | 10.765 | 100% | 14.705 | 25% | 21.094 | 8.575 |
| Sum | 374.986 |  | 307.772 |  | 67.214 |  | 278.878 | 100.822 |

1 Data according to operators data

The amount of collected waste by formal operators in Arges County is reported as 100.882 tons by 2009, mainly in urban areas, however, in sight of the lack of weighbridges in the County by that year, the amount is taken with reserves. In Rural areas of Arges County and specially areas without collection services the informal sector is responsible for around 4.000 tons/year of sub-products from municipal waste if Arges County.

#### Projections on generated waste in Arges County

|  | Generated Waste (ton/year) | Code | 2010 | 2013 | 2016 | 2020 | 2025 | 2030 | 2035 | 2039 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,0 | Municipal waste and assimilable waste from commerce, industry and institutions, out of which: | 20 15 01 | 284.061 | 288.323 | 291.145 | 293.941 | 297.478 | 297.486 | 296.610 | 295.914 |
| 1,1 | Municipal waste mixed collected from the population, out of which: | 20 03 01 | 162.279 | 164.713 | 166.407 | 168.171 | 170.406 | 170.623 | 170.335 | 170.107 |
|  | 1.1.1 Urban |  | 109.230 | 110.869 | 111.863 | 112.754 | 113.878 | 113.645 | 113.075 | 112.621 |
|  | 1.1.2 Rural |  | 53.048 | 53.844 | 54.544 | 55.417 | 56.528 | 56.978 | 57.260 | 57.486 |
| 1,2 | Assimilable waste mixed collected from commerce, industry and institutions | 20 03 01 | 89.355 | 90.696 | 91.509 | 92.238 | 93.157 | 92.967 | 92.500 | 92.129 |
|  | 1.2.1 Urban |  | 66.856 | 67.859 | 68.468 | 69.013 | 69.701 | 69.558 | 69.209 | 68.931 |
|  | 1.2.2 Rural |  | 22.499 | 22.837 | 23.041 | 23.225 | 23.456 | 23.408 | 23.291 | 23.197 |
| 1,3 | Municipal and similar waste collected separately (except construction and demolition waste) out of which: | 20 01 15 01 |  |  |  |  |  |  |  |  |
|  | Paper and cardboard | 20 01 01 15 01 01 | 13.387 | 15.497 | 17.598 | 19.807 | 20.818 | 21.879 | 20.807 | 19.587 |
|  | Glass | 20 01 02 15 01 07 | 12.366 | 14.315 | 16.256 | 18.296 | 19.230 | 20.211 | 19.220 | 18.093 |
|  | Metal | 20 01 40 15 01 04 | 5.105 | 5.910 | 6.711 | 7.554 | 7.939 | 8.344 | 7.935 | 7.469 |
|  | Plastic | 20 01 39 15 01 02 | 16.450 | 19.043 | 21.625 | 24.339 | 25.581 | 26.886 | 25.568 | 24.068 |
|  | Wood | 20 01 38 15 01 03 | 6.807 | 7.880 | 8.948 | 10.071 | 10.585 | 11.125 | 10.580 | 9.959 |
|  | Biodegradable | 20 01 08 | 150.028 | 173.585 | 168.950 | 163.155 | 158.860 | 151.775 | 147.414 | 144.993 |
|  | Others | 20 01 15 01 | 46.630 | 52.140 | 51.914 | 51.442 | 50.883 | 49.761 | 48.613 | 47.700 |
| 1,4 | Bulky waste | 20 03 07 | -- | -- | -- | -- | -- | -- | -- | -- |
| 1,5 | Waste from gardens and parks | 20 02 | 9.407 | 9.548 | 9.652 | 9.768 | 9.915 | 9.944 | 9.943 | 9.943 |
|  | Urban |  | 7.038 | 7.144 | 7.222 | 7.309 | 7.418 | 7.440 | 7.440 | 7.439 |
|  | Rural |  | 2.369 | 2.404 | 2.430 | 2.460 | 2.496 | 2.504 | 2.504 | 2.504 |
| 1,6 | Market waste | 20 03 02 | 2.514 | 2.552 | 2.575 | 2.595 | 2.621 | 2.616 | 2.603 | 2.592 |
|  | Urban |  | 1.881 | 1.909 | 1.926 | 1.942 | 1.961 | 1.957 | 1.947 | 1.939 |
|  | Rural |  | 633 | 643 | 648 | 653 | 660 | 659 | 655 | 653 |
| 1,7 | Street waste** | 20 03 03 | 20.507 | 20.815 | 21.001 | 21.169 | 21.380 | 21.336 | 21.229 | 21.143 |
|  | Urban |  | 15.343 | 15.574 | 15.713 | 15.838 | 15.996 | 15.964 | 15.884 | 15.820 |
|  | Rural |  | 5.164 | 5.241 | 5.288 | 5.330 | 5.383 | 5.372 | 5.345 | 5.324 |
|  | Kg/inh-year |  | 445 | 456 | 464 | 474 | 487 | 495 | 501 | 506 |
|  | Kg/inh-day |  | 1,22 | 1,25 | 1,27 | 1,30 | 1,34 | 1,36 | 1,37 | 1,39 |
| 1,8 | Generated and uncollected waste, out of which: | 20 01 15 01 | 32.284 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Urban |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Rural |  | 32.284 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2,0 | Municipal and assimilable waste sorted collected (exclusively waste from demolitions and constructions)* |  | 35.630 | 36.164 | 36.561 | 36.999 | 37.554 | 37.665 | 37.663 | 37.662 |
|  | Urban |  | 26.658 | 27.058 | 27.356 | 27.683 | 28.098 | 28.181 | 28.180 | 28.179 |
|  | Rural |  | 8.971 | 9.106 | 9.206 | 9.316 | 9.456 | 9.484 | 9.483 | 9.483 |
| 3,0 | Sludge from WWTP (tons) |  | NA | 16.000 | 16.000 | 16.000 | 16.000 | 16.000 | 16.000 | 16.000 |

Figure 5.1-1 Household waste generation development in Arges County

The corresponding Waste indexes for the Table above, you may find it in Annex 5.1-1.

### Biodegradable Waste

#### Current Situation and Projection

At the current situation biodegradable waste is not collected separately. In urban areas, biodegradable waste is discarded with the other household waste streams, and ends up in a landfill or waste dump. Park and garden waste is also landfilled.

In rural areas, garden waste is burned in spring and autumn. A certain proportion of biodegradable household waste is used as follows:

- composted in the garden and used, together with wood ash, as a plant fertilizer,

- vegetable and fruit scraps and peelings are used as livestock and poultry fodder.

A residual biodegradable fraction is discarded with either domestic waste and ends up at a disposal sites.

Table 5.2-3 Share of collected Biodegradable waste in Arges County (see Annex 5.1-2)

| Year | 2010 | 2013 | 2016 | 2020 | 2025 | 2030 | 2035 | 2039 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Total (ton/y) | 149.834 | 174.437 | 167.679 | 161.569 | 156.988 | 149.612 | 144.736 | 142.159 |
| Household waste from the population: | 100.141 | 118.350 | 114.020 | 111.112 | 109.098 | 105.082 | 102.463 | 101.157 |
| Urban | 75.452 | 76.584 | 72.525 | 70.057 | 68.174 | 64.970 | 62.840 | 61.720 |
| food and garden waste | 62.261 | 63.195 | 59.974 | 55.357 | 51.460 | 46.178 | 42.750 | 40.930 |
| paper+cardboard; wood | 13.191 | 13.389 | 12.551 | 14.699 | 16.713 | 18.792 | 20.090 | 20.790 |
| Rural | 24.689 | 41.766 | 41.495 | 41.056 | 40.924 | 40.112 | 39.623 | 39.437 |
| food and garden waste | 22.503 | 38.068 | 37.211 | 35.976 | 35.071 | 33.447 | 32.414 | 31.909 |
| paper+cardboard; wood | 2.186 | 3.698 | 4.284 | 5.080 | 5.853 | 6.665 | 7.209 | 7.528 |
| Similar waste from commerce, industry and institutions (collected mixed and separately) | 36.160 | 40.813 | 38.229 | 34.867 | 32.098 | 28.724 | 26.498 | 25.253 |
| Urban | 30.085 | 30.537 | 28.603 | 26.088 | 24.016 | 21.491 | 19.826 | 18.894 |
| Rural | 6.075 | 10.276 | 9.626 | 8.779 | 8.082 | 7.232 | 6.672 | 6.359 |
| Garden and park waste | 8.036 | 9.070 | 9.170 | 9.280 | 9.419 | 9.447 | 9.446 | 9.446 |
| Market waste | 1.809 | 2.041 | 2.060 | 2.076 | 2.097 | 2.093 | 2.082 | 2.074 |
| Street waste | 3.688 | 4.163 | 4.200 | 4.234 | 4.276 | 4.267 | 4.246 | 4.229 |
| Index (Kg/inh-year) | 234,683 | 275,692 | 267,41 | 260,781 | 257,222 | 248,847 | 244,38 | 242,931 |

#### Definition of demands, needs and main deficiencies

Main deficiencies

- lack of knowledge of key parameters for producing compost high in nutrients and therewith very limited activities on composting and home composting of biodegradable waste

- dumping of biodegradable waste instead of upgrading soil parameters for better harvest

- misunderstanding of reuse of biodegradable waste in an ecological way

If we take into account the current capacity for biodegradable waste treatment in Arges County, i.e. composting plants at Campulung and Albota (total of 23.000 tons/year), the achievement of corresponding targets is as follows:

Table 5.2-4 Ladfilling of biodegradable waste  in Arges , considering current capacities

| Year | OW (ton) | Landfill amounts (Tons) |  | OW (ton) | Comments to target achievement |
| --- | --- | --- | --- | --- | --- |
|  | Target1 | Phase-I | Phase-II | Total |  |
| 2010 | 107.726 | 69.977 | 26.689 | 96.666 | Accomplished |
| 2013 | 71.818 | 54.180 | 35.148 | 89.327 | NotAccomplished |
| 2016 | 50.272 | 51.418 | 33.862 | 85.280 | NotAccomplished |

The restricted amount of biodegradable waste being disposed off in landfill is achieving the target by 2010, but falling in the next years. Therefore, an increase in treatment capacity is necessary in Arges County.

Needs and Demands

- Establishment of biodegradable waste reduction measures due to the EU Landfill Directive 99/31/EG and their target until 2016 to reduce the biodegradable waste share on the level of 35% of 1995

- Meet the Romanian requirements of the Landfill Directive Implementation Plan of October 2004

- Education in composting and extension of public awareness campaigns

- Home composting measures in rural areas

### Packaging Waste

#### Current Situation and Projection

At present at urban areas (e.g. Curtea de Arges, Pitesti) carry out some selective collection points are established. The separate collection of PET shows separate collection with less impurity, however the amount is considered as negligible as in total represents less than 5% of the required amount. The existing separate collection of recyclables is not sufficient to cover the targets of the Packaging Waste Directive.

In rural areas, packaging waste is discarded together with the other household waste streams, and ends up in a landfill or waste dump. The packaging waste being generated in Arges County is presented in the next table.

Table 5.2-5 Current and future development for waste packaging in Arges County (see Annex 5.1-3)

| Waste Packaging (ton/y) | 2009 | 2010 | 2013 | 2016 | 2020 | 2025 | 2030 | 2035 | 2039 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Increase rate | 7% | 5% |  | 3% |  | 1% |  | -1,0% | -1,5% |
| Total (ton) | 54.024 | 56.725 | 65.667 | 74.570 | 83.929 | 88.210 | 92.710 | 88.166 | 82.994 |
| Kg/inh-year | 84,4 | 88,8 | 103,8 | 118,9 | 135,5 | 144,5 | 154,2 | 148,9 | 141,8 |
| Household Contribution (60%) | 32.415 | 34.035 | 39.400 | 44.742 | 50.357 | 52.926 | 55.626 | 52.900 | 49.796 |
| Urban Areas | 15.532 | 16.309 | 18.879 | 21.439 | 24.130 | 25.360 | 26.654 | 25.348 | 23.861 |
| Paper and cardboard | 3.666 | 3.849 | 4.456 | 5.060 | 5.695 | 5.985 | 6.290 | 5.982 | 5.631 |
| Glass | 3.386 | 3.555 | 4.116 | 4.674 | 5.260 | 5.529 | 5.811 | 5.526 | 5.202 |
| Metal | 1.398 | 1.468 | 1.699 | 1.929 | 2.172 | 2.282 | 2.399 | 2.281 | 2.147 |
| Plastic | 4.504 | 4.730 | 5.475 | 6.217 | 6.998 | 7.355 | 7.730 | 7.351 | 6.920 |
| Wood | 1.864 | 1.957 | 2.266 | 2.573 | 2.896 | 3.043 | 3.198 | 3.042 | 2.863 |
| Others | 714 | 750 | 868 | 986 | 1.110 | 1.167 | 1.226 | 1.166 | 1.098 |
| Rural Areas | 16.882 | 17.727 | 20.521 | 23.303 | 26.228 | 27.565 | 28.972 | 27.552 | 25.935 |
| Paper and cardboard | 3.984 | 4.183 | 4.843 | 5.499 | 6.190 | 6.505 | 6.837 | 6.502 | 6.121 |
| Glass | 3.680 | 3.864 | 4.474 | 5.080 | 5.718 | 6.009 | 6.316 | 6.006 | 5.654 |
| Metal | 1.519 | 1.595 | 1.847 | 2.097 | 2.360 | 2.481 | 2.607 | 2.480 | 2.334 |
| Plastic | 4.896 | 5.141 | 5.951 | 6.758 | 7.606 | 7.994 | 8.402 | 7.990 | 7.521 |
| Wood | 2.026 | 2.127 | 2.462 | 2.796 | 3.147 | 3.308 | 3.477 | 3.306 | 3.112 |
| Others | 777 | 815 | 944 | 1.072 | 1.206 | 1.268 | 1.333 | 1.267 | 1.193 |
| Industry, Commerce& Institutions (40%) | 21.610 | 22.690 | 26.267 | 29.828 | 33.571 | 35.284 | 37.084 | 35.266 | 33.197 |
| Paper and cardboard | 5.100 | 5.355 | 6.199 | 7.039 | 7.923 | 8.327 | 8.752 | 8.323 | 7.835 |
| Glass | 4.711 | 4.946 | 5.726 | 6.502 | 7.319 | 7.692 | 8.084 | 7.688 | 7.237 |
| Metal | 1.945 | 2.042 | 2.364 | 2.685 | 3.021 | 3.176 | 3.338 | 3.174 | 2.988 |
| Plastic | 6.267 | 6.580 | 7.617 | 8.650 | 9.736 | 10.232 | 10.754 | 10.227 | 9.627 |
| Wood | 2.593 | 2.723 | 3.152 | 3.579 | 4.029 | 4.234 | 4.450 | 4.232 | 3.984 |
| Others | 994 | 1.044 | 1.208 | 1.372 | 1.544 | 1.623 | 1.706 | 1.622 | 1.527 |
| Indexes for Packaging Waste by Households |  |  |  |  |  |  |  |  |  |
| Waste Packaging Index (ton/y) | 2009 | 2010 | 2013 | 2016 | 2020 | 2025 | 2030 | 2035 | 2039 |
| Paper and cardboard | 7.650 | 8.032 | 9.298 | 10.559 | 11.884 | 12.491 | 13.128 | 12.484 | 11.752 |
| Kg/inh-year | 11,95 | 12,58 | 14,70 | 16,84 | 19,18 | 20,47 | 21,84 | 21,08 | 20,08 |
| Glass | 7.066 | 7.420 | 8.589 | 9.754 | 10.978 | 11.538 | 12.126 | 11.532 | 10.856 |
| Kg/inh-year | 11,03 | 11,62 | 13,57 | 15,55 | 17,72 | 18,90 | 20,17 | 19,47 | 18,55 |
| Metal | 2.917 | 3.063 | 3.546 | 4.027 | 4.532 | 4.763 | 5.006 | 4.761 | 4.482 |
| Kg/inh-year | 4,56 | 4,80 | 5,60 | 6,42 | 7,32 | 7,80 | 8,33 | 8,04 | 7,66 |
| Plastic | 9.400 | 9.870 | 11.426 | 12.975 | 14.604 | 15.349 | 16.131 | 15.341 | 14.441 |
| Kg/inh-year | 14,68 | 15,46 | 18,06 | 20,69 | 23,57 | 25,15 | 26,83 | 25,90 | 24,68 |
| Wood | 3.890 | 4.084 | 4.728 | 5.369 | 6.043 | 6.351 | 6.675 | 6.348 | 5.976 |
| Kg/inh-year | 6,07 | 6,40 | 7,47 | 8,56 | 9,75 | 10,41 | 11,10 | 10,72 | 10,21 |

#### Definition of demands, needs and main deficiencies

Main deficiencies

- The existing recyclable waste collection is inefficient and it exists only in some institutions of urban areas.

- No professional and comprehensive system on packaging waste recovery exists, except of certain activities in urban areas and unprofessional selection of few types of packaging waste by the informal sector

- The informal sector actually includes professional waste separators – in that they know the prevailing market prices and quality requirements of the recycling dealers. However, their contribution is not recognised by most public authorities and some practitioners/consultants working in the field. Internationally, and in particular in IFIs and international practitioner networks, the informal sector is gradually being recognised for its positive contribution to recycling. One current main deficiency is that the percentage contribution of the informal sector is not quantified and apportioned to the achievement of legislated targets.

Needs and Demands

- Meet the requirements of the Packaging and Packaging Waste Directive

- Extension of the recyclable waste collection to the entire county

- Education in separate collection and extension of public awareness campaigns

- The recyclable targets and the needed achievements by the project are presented in the table below.

- The several recycling targets could be reached easily but the overall recovery/ recycling rate is higher as the sum of the several compartments.

- The following table gives a detailed overview of the waste quantities and the several targets for each type of packaging waste.

The following table gives a detailed overview of the waste quantities needed to reach the Waste Packaging (WP) targets (see Annex 4.3-1). The concerning waste types are paper & cardboard, glass, plastics, and metal. Due to the increasing income of the population in the considered time span the amount of packaging waste is increasing, for urban areas as well as rural areas.. According to collecting targets of packaging waste the amount of collected packaging waste is increasing.

Table 5.2-6 Amounts of packaging waste targeted in Arges County (Source: LWMP Arges)

| Amounts (ton) | 2010 | 2011 | 2012 | 2013 |
| --- | --- | --- | --- | --- |
| H+C | 9.639 | 10.683 | 11.512 | 12.398 |
| Plastic | 2.303 | 2.764 | 3.265 | 4.189 |
| Glass | 5.441 | 6.232 | 7.362 | 8.589 |
| Metal | 3.267 | 3.860 | 4.165 | 4.492 |
| Wood | 817 | 1.072 | 1.276 | 1.497 |
| Total recycled | 21.467 | 24.611 | 27.580 | 31.165 |
| Total Collected | 27.228 | 31.567 | 35.647 | 39.400 |

Based on the amount of packaging waste by type of material expected to be generally quantified targets for recycling and recovery.

The recovery amount is presented in the following table.

Table 5.2-7 Amounts of recovered packaging waste in Arges  County

| Year | Households |  |  |  |  | Commerce & Institutions |  |  |  |  | Sum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (ton) |  |  |  |  | (ton) |  |  |  |  |  |
|  | Paper | Glass | Metal | Plastic | Wood | Paper | Glass | Metal | Plastic | Wood |  |
| 2010 | 7.229 | 7.012 | 4.166 | 8.883 | 3.676 | 4.819 | 4.452 | 2.777 | 5.922 | 2.451 | 51.387 |
| 2011 | 8.012 | 7.771 | 4.617 | 9.846 | 4.074 | 5.341 | 4.934 | 3.078 | 6.564 | 2.716 | 56.954 |
| 2012 | 8.856 | 8.589 | 5.103 | 10.882 | 4.503 | 5.904 | 5.453 | 3.402 | 7.255 | 3.002 | 62.949 |
| 2013 | 9.298 | 9.019 | 5.358 | 11.426 | 4.728 | 6.199 | 5.726 | 3.572 | 7.617 | 3.152 | 66.096 |
| 2014 | 9.763 | 9.470 | 5.626 | 11.997 | 4.964 | 6.509 | 6.012 | 3.751 | 7.998 | 3.310 | 69.401 |
| 2015 | 10.252 | 9.754 | 5.908 | 12.597 | 5.213 | 6.834 | 6.313 | 3.938 | 8.398 | 3.475 | 72.682 |
| 2016 | 10.559 | 10.046 | 6.085 | 12.975 | 5.369 | 7.039 | 6.502 | 4.057 | 8.650 | 3.579 | 74.862 |

The amounts material from Packaging waste that is recycled by the IWM System in Arges County (Phase-I + Phase-II) are as follows:

Table 5.2-8 Amounts of recycled packaging waste in Arges  County

| Year | Amounts |  |  |  |  | Sum |
| --- | --- | --- | --- | --- | --- | --- |
|  | (ton) |  |  |  |  |  |
|  | Paper | Glass | Metal | Plastic | Wood |  |
|  | 80% | 60% | 50% | 23% | 15% |  |
| 2010 | 10.710 | 7.642 | 3.857 | 3.701 | 1.021 | 26.932 |
| 2011 | 11.245 | 8.024 | 4.050 | 3.886 | 1.072 | 28.278 |
| 2012 | 11.808 | 8.426 | 4.253 | 4.081 | 1.126 | 29.692 |
| 2013 | 12.398 | 8.847 | 4.465 | 4.285 | 1.182 | 31.177 |
| 2014 | 13.018 | 9.289 | 4.689 | 4.499 | 1.241 | 32.736 |
| 2015 | 13.669 | 9.640 | 4.923 | 4.724 | 1.303 | 34.259 |
| 2016 | 14.079 | 9.929 | 5.071 | 4.866 | 1.342 | 35.287 |

In order to verify the achievement of the targets, the following table shows the targeted quantified amounts of packaging waste in the Local Waste Management Plan of Arges County. The following table gives the comparison between targeted amounts and the achievements by the ISWM-Arges.

| Amounts (ton) | 2010 | 2011 | 2012 | 2013 |
| --- | --- | --- | --- | --- |
| Total recycled (LWMP-Arges) | 21.467 | 24.611 | 27.580 | 31.165 |
| Total Collected (LWMP-Arges) | 27.228 | 31.567 | 35.647 | 39.400 |
| Total recycled (Achievement) | 26.932 | 28.278 | 29.692 | 31.177 |
| Total Collected (Achievement) | 51.387 | 56.954 | 62.949 | 66.096 |

As it can be observed in the last table, the targets stated in the LWMP-Arges are reached with the implementation of Phase-i+Phase-II of ISWM-Arges.

### Other Waste (Hazardous Waste, ELV, WEEE)

#### Hazardous Waste

Households routinely generate a number of types of waste which are classified as dangerous according to the EU List of Waste.

No organized measures have been taken to collect other hazardous household wastes separately in Arges County. Using crude generation coefficients which are thought to apply generally in the RWMP-Arges (urban areas 2.5 kg/person/year, rural areas 1.5kg/person/year) it can be estimated that the total amount of hazardous waste getting into the household waste stream in Arges County would be between 1.110 and 1.260 tones per year.

#### WEEE

The WEEE Directive requires that Romania should set up systems to allow holders of WEEE to return such waste separately. Municipalities began to implement this requirement in 2006, several towns (e.g. Curtea de Arges, Pitesti) now provide facilities to allow holders of waste to hand them over without costs. The volumes being collected are still small.

According to §3 HG 448/2005 producers must ensure the establishment

a) until 31 December 2005 at least:

(i) a collection point in each county;

(ii) a collection point in each city with 100,000 inhabitants;

(iii) to collect 6 points in Bucharest;

b) until 31 December 2006 at least one collection point in each city with over 20,000 inhabitants.

c) The EU regulation 2006/ 66/ EC on batteries and accumulators gives the following collection rates for there Member states:

          - 25% collection rate until 26th September 2012

          - 45% collection rate until 26th September 2016

After several meetings with local authorities and operators it was declared that WEEE is collected in the counties and then transferred to Bucharest where the WEEE is only stored but not treated. The final treatment is missing completely in Romania.

The RWMP gives a generation rate of 4 kg/ inh. x year until 2008 for urban and rural areas. This generation rate was adopted at the Master Plan and finally also to the present Feasibility Study.

#### ELVs, Bulky Waste and C&D Waste

The objective set out by the RWMP3 is the establishment of several ELV collection Points in Arges. In Arges County some authorized operators for treatment of ELV exist. End- of- Life vehicles are not subject to the Feasibility Study.

In the case of Bulky Waste, Arges does not yet collect bulky waste separately. The RWMP3 reported a generation of 1.311 tons in 2003, that means a rate of 2 Kg/inh-y.

The Construction & Demolition Waste, the current and future development in Arges County is presented in the next Table.

Table 5.2-9 Specific Waste streams quantities in Arges County (see Annex 5.1-4)

| Year | 2010 | 2013 | 2016 | 2020 | 2025 | 2030 | 2035 | 2039 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Household Hazardous Waste (ton) |  |  |  |  |  |  |  |  |
| Urban | 765 | 758 | 751 | 742 | 731 | 720 | 709 | 701 |
| Rural | 499 | 494 | 490 | 484 | 477 | 470 | 463 | 457 |
| Index Urban (kg/inh-year) | 2,5 |  |  |  |  |  |  |  |
| Index Rural (kg/inh-year) | 1,5 |  |  |  |  |  |  |  |
| Municipal and assimilable waste sorted collected (exclusively waste from demolitions and constructions)* |  |  |  |  |  |  |  |  |
| Generation County (tons) | 35.630 | 36.164 | 36.561 | 36.999 | 37.554 | 37.665 | 37.663 | 37.662 |
| Generation Urban | 26.658 | 27.058 | 27.356 | 27.683 | 28.098 | 28.181 | 28.180 | 28.179 |
| Generation Rural | 8.971 | 9.106 | 9.206 | 9.316 | 9.456 | 9.484 | 9.483 | 9.483 |
| County Index (Kg/inh-y) | 55,81 | 57,16 | 58,31 | 59,72 | 61,53 | 62,65 | 63,59 | 64,36 |
| Index Urban (Kg/inh-y) | 87,14 | 89,25 | 91,04 | 93,25 | 96,08 | 97,82 | 99,30 | 100,50 |
| Index Rura (Kg/inh-y)l | 26,98 | 27,63 | 28,19 | 28,87 | 29,75 | 30,29 | 30,74 | 31,11 |
| Bulky waste |  |  |  |  |  |  |  |  |
| Generation County (tons) | 1.302 | 1.290 | 1.274 | 1.259 | 1.240 | 1.218 | 1.199 | 1.185 |
| Generation Urban | 974 | 965 | 953 | 942 | 928 | 911 | 897 | 887 |
| Generation Rural | 328 | 325 | 321 | 317 | 312 | 307 | 302 | 298 |
| County Index (Kg/inh-y) | 2,04 | 2,04 | 2,03 | 2,03 | 2,03 | 2,03 | 2,03 | 2,03 |
| Index Urban (Kg/inh-y) | 3,18 | 3,18 | 3,17 | 3,17 | 3,17 | 3,16 | 3,16 | 3,16 |
| Index Rura (Kg/inh-y)l | 0,99 | 0,99 | 0,98 | 0,98 | 0,98 | 0,98 | 0,98 | 0,98 |
| End-of-Life-Vehicles |  |  |  |  |  |  |  |  |
| Arges County (Unit) | 3.831 | 5.315 | 6.584 | 7.907 | 9.941 | 12.499 | 15.714 | 18.873 |
| County Index (ELV/1000inh) | 6 | 8,4 | 10,5 | 12,8 | 16,3 | 20,8 | 26,5 | 32,3 |
| Waste from Electric and Electronic Equipment (WEEE) |  |  |  |  |  |  |  |  |
| Arges County (Tons) | 2.554 | 3.796 | 3.762 | 3.717 | 3.662 | 3.607 | 3.554 | 3.511 |
| County Index (Kg/inh-y) | 4 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |

C&D and Bulky waste are out of collection figures as the producer will be responsible for deliver their waste into the transfer Stations and/or landfill, where there are civic amenity facilities to store the bulky waste and equipment to transport the residual part to landfilling.

In regard ELV and WEEE as the European Legislations states, the producers will set a collection net in order to get back the discarded equipment and the last owner will deliver the equipment to these points. Nevertheless, the system will provide a container in the civic amenity facilities (Green points) for storing WEEE, in case these equipment reachs the system and to avoid the contamination of waste streams, able for recycling.

#### Definition of demands, needs and main deficiencies

- Household Hazardous Waste

Main deficiencies:

- no separate hazardous waste collection in urban and rural areas

- no treatment or disposal site for hazardous waste in Arges County (e.g. hazardous waste incineration)

- no treatment facility for hazardous waste in Arges County

- Landfilling of hazardous waste could affect the biological process in landfill, the leachate treatment and finally it might pollute the groundwater

Needs and Demands:

- implementation of a comprehensive system for hazardous waste collection in urban and rural areas, meeting the requirements of EU and national environmental and waste legislation

- implementation of solutions for hazardous waste disposal and/or treatment, either on regional basis, serving all counties of the development region, or on county basis, serving Arges county separately

- WEEE

Main deficiencies:

- separate collection of WEEE in 3 urban areas (6 collection points)

- no separate collection of WEEE in rural areas

- no WEEE recyclers in Romania, only storage of WEEE near to Bucharest

Needs and Demands:

- implementation of a comprehensive system for WEEE collection in urban and rural areas, meeting the requirements of EU and national environmental and waste legislation (The RWMP objective demands a WEEE collection rate of minimum 4kg/inh./year until 31.12.2008)

- implementation of solutions for WEEE treatment, either on regional basis, serving all counties of the development region or on county basis, serving Arges county separately

- ELV

Main deficiencies

- some operators for ELV, not enough collection points for ELV

- no collection points for ELV in rural areas

- quantity of treated ELV is low

- ELV are placed at the streets, only cars which are brought to the ELV collection point are treatedthere is no system for the cars which are not roadworthy

Needs and Demands

- implementation of a comprehensive system for ELV collection in urban and rural areas, meeting the requirements of EU and national environmental and waste legislation

- implementation of solutions for ELV treatment

- either on regional basis, serving all counties of the development region, or on county basis, serving Arges County separately

- Bulky Waste

Main deficiencies

- no separate collection of bulky waste in Arges County

- no disposal site or treatment of bulky waste in Arges County (e.g. incineration and therefore energy recovery)

Needs and Demands

- implementation of a comprehensive system for Bulky Waste collection in urban and rural areas, meeting the requirements of EU and national environmental and waste legislation

- implementation of solutions for Bulky Waste treatment

- either on regional basis, serving all counties of the development region or on county basis, serving Argescounty separately

- Construction and Demolition Waste

Main deficiencies

- no separate collection of C&D waste in Arges County

- existing of construction and demolition waste dumping on mixed municipal waste dumps

- Construction & Demolition waste is collected by private generators or construction companies, without any separation of mineral and other substances

- No reliable waste generation data exists

Needs and Demands

- implementation of a comprehensive system for Construction and Demolition Waste processing in urban and rural areas, meeting the requirements of EU and national environmental and waste legislation

- implementation of solutions for Construction & Demolition  Waste treatment, either on regional basis, serving all counties of the development or better on operators yard basis, serving of several communes per yard.

## Composition of Waste

### Current Situation and projection

The composition of the generated waste will evolve in Arges County during the project period; organic fractions will decrease in urban and rural areas, while other fractions as glass and plastics will increase their participation in the total waste composition. All these as result are due to the implementation of the European Waste legislation.

The following tables present the Household, Institution & Commercial waste composition that has been considered for the project period.

Table 5.3-1 Evolution of the HH Waste Composition during project period in Arges County (see Annex 5.1-5)

| Year | 2003-2013 |  | 2015 |  | 2020 |  | 2030 |  | 2039 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Waste Composition | Urban | Rural | Urban | Rural | Urban | Rural | Urban | Rural | Urban | Rural |
| Waste Packaging | 26,45% | 18,99% | 29,16% | 20,94% | 35,48% | 25,47% | 45,42% | 32,61% | 50,91% | 36,55% |
| Paper and Cardboard | 6,54% | 4,70% | 7,21% | 5,18% | 8,78% | 6,30% | 11,24% | 8,07% | 12,59% | 9,04% |
| Glass | 6,04% | 4,34% | 6,66% | 4,78% | 8,11% | 5,82% | 10,38% | 7,45% | 11,63% | 8,35% |
| Metals | 2,50% | 1,79% | 2,75% | 1,98% | 3,35% | 2,40% | 4,28% | 3,08% | 4,80% | 3,45% |
| Plastic | 8,04% | 5,77% | 8,86% | 6,36% | 10,79% | 7,74% | 13,81% | 9,91% | 15,48% | 11,11% |
| Wood | 3,33% | 2,39% | 3,67% | 2,63% | 4,46% | 3,20% | 5,71% | 4,10% | 6,40% | 4,60% |
| Biodegradable (food and garden waste) | 57,00% | 70,70% | 54,62% | 68,96% | 49,10% | 64,92% | 40,63% | 58,70% | 36,34% | 55,51% |
| Recycling Waste different than Packaging | 16,55% | 10,31% | 16,22% | 10,10% | 15,43% | 9,61% | 13,95% | 8,69% | 12,74% | 7,94% |
| Paper and Cardboard | 2,66% | 1,04% | 2,61% | 1,02% | 2,48% | 0,97% | 2,24% | 0,88% | 2,05% | 0,80% |
| Metals | 1,14% | 2,52% | 1,12% | 2,47% | 1,06% | 2,35% | 0,96% | 2,12% | 0,88% | 1,94% |
| Hazardous waste | 0,05% | 0,03% | 0,05% | 0,03% | 0,05% | 0,03% | 0,04% | 0,03% | 0,04% | 0,02% |
| WEEE | 1,00% | 0,50% | 0,98% | 0,49% | 0,93% | 0,47% | 0,84% | 0,42% | 0,77% | 0,39% |
| Others | 11,70% | 6,22% | 11,47% | 6,10% | 10,91% | 5,80% | 9,86% | 5,24% | 9,01% | 4,79% |
| Total | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |

In the last table the Household waste composition is based on the composition reported by the LWMP-Arges in 2008 for the period of 2003-2013. In the further years a slow fall down of the Organic fraction takes place and as a consequence a growth in the packaging waste composition.

## Facilities and Performance

### Collection and Transport

The level of service and coverage of service for collection and transport of waste separately for urban and rural areas in Arges County are presented in the next table.

Table 5.4-1 Current infrastructure for Waste collection and transport in Arges County

| Issue | Description |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Share of population connected to sanitation service | - 374.986 persons (59%) connected to collection services in Arges. - 307.772 inh in urban areas (close to 100%) - 278.878 inh. In rural areas (20% -25%) |  |  |  |  |  |  |  |  |
| Existing sanitation services | Total amount collected (tons) |  |  | - Zone 1:24.676 - Zone-2: 341 - Zone-3:25.246 - Zone-4:1.435 - Zone-5:20.832 - Zone-6: 99.350 - Zone-7:9.882 - Zone-8:8.575 - Total: 100.882 |  |  |  |  |  |
|  | Ownership (private or public) |  |  | Urban areas: Private and/or public Rural areas: Private and/or public |  |  |  |  |  |
|  | no. of served inhabitants |  |  | - Zone 1:33.154 - Zone-2: 5.478 - Zone-3:47.831 - Zone-4:1.822 - Zone-5:40.881 - Zone-6: 203.354 - Zone-7:16.996 - Zone-8:25.470 Total: 374.986 |  |  |  |  |  |
| Issue | Z1 | Z2 | Z3 |  | Z4 | Z5 | Z6 | Z7 | Z8 |
| Equipment for collection | - 2626 Cont. 120 -140L - 697 cont. 4m3 - 89 Cont. 4m3 (rented) - 4 Spare Cont. - 4 Compacting machines | - 20000 wheel bins | - 1000 Plastic cont. - 19 cont. 1 m3 - 140 cont. 4-5 m3 - 4 auto-cont. - 1 solid waste compacting machine - 2 compacting machine |  | - 100 bins 240 L - 9 cont. 1 m3 | - 20 Plastic cont. 120L - 20 euro-cont. 1100L - 72 cont. 4000L - 2 solid waste compacting machine - 1 cont. transport vehicle | - 635 Euorbins 240L - 596 cont. 4.5 m3 - 23 cont. 10m3 - 10 contaoiners 5 m3 - 118 Cont. 1m3 | - 10 cont. 1 m3 - 1 solid waste compacting machine | 5 Cont. 4m3 |
| Equipment for transport | - 3 vehicles for bins - 2 vehicles for cont. - 1 tow tractor - 1 Compactor truck | - 2 Compactor truck | - 6 Compactor truck |  | - 2 vehicles for cont. | - 2 Trailer tractor | - 11 compactor trucks - 23 cont. vehicles - 14 tow tractors - 2 lorries | - 4 Compactor truck | - 2 trucks |
| Collection frequency | 1-2 per week | 1-2 per week | 1-2 per week |  | 1-2 per week | 1-2 per week | 1-2 per week | 1-2 per week | 1-2 per week |
| Average distance to next landfill | 40 Km | 48 Km | 51 Km |  | 68 Km | 32 Km | 38 Km | 20 Km | 25 Km |

#### Definition of demands, needs and main deficiencies

Urban

The main deficiencies regarding the infrastructure for waste collection and transport in urban areas of Arges County are related to the lack of operations for separated collection. This fact causes a literally waste of all the recyclable materials, content into the waste streams of the entire County. In other cases, the deficiencies are related to the sufficiency and condition of the implemented equipment for waste collection and transport, as well as the road infrastructure.

In regard transfer services, the new transfer station in Campulung serves only Zones 3&4 for waste transfer services. The waste being collected in Zones 5,6 & 7, due to their close position to WMC, is directly transported to the WMC at Albota facilities.

However, the zones 1,2 & 8 are not covered by any transfer station and they are far away from WMC, thus they are currently transporting their collected waste directly to WMC at Albota.

| The distances travelled, from zones without transfer station (zones 1,2&8) to WMC at Albota, are presented in the next table. Distances (Km); Curtea de Arges city; Domnesti Commune; Costesti city; WMC at Albota. Curtea de Arges city; 0; 19,3; 64,7; 40,0. Domnesti Commune; 19,3; 0; 69,6; 48,5. Costesti city; 64,7; 69,6; 0; 23,1. WMC at Albota; 40,0; 48,5; 23,1; 0. The distance from these zones to WMC is from 23 to 50 Km, which currently represent high O&M costs to operators, besides environmental impacts. For this reason there is an urgency for transfer stations to serve these zones (1,2&8) in order to solve this situation | Figure 5.4-1 Travelled distances to WMC |
| --- | --- |

Rural

The condition for waste collection and transport in rural areas of Arges County is very similar. There is a lack of waste collection at all, especially in villages.

Table 5.4-2 Correspondent National and County Objectives

| Measure | Related objectives |  | Equivalence | Units |
| --- | --- | --- | --- | --- |
|  | Issue | Target |  |  |
| Waste collection and transport | - Extension of waste collection systems | Collection coverage: - 100% urban areas (2013) - 90% rural areas (2009) | 309.625 302.286 | Inh. |
|  | - Separated collection of HHW | Starting with 2007 |  |  |
|  | - Separated storage and collection | -100% by 2013 (urban) -100% by 2017 (rural) | 646.171 337.219 | Inh. |
|  | - Recovery of waste materials | - 43% of the population (2008) - 59% of the population (2011) - 70 % of the population (2013) | 277.160 380.860 452.320 | Inh. |

### Biological Treatment of Waste

According to the collected information, from Zones of Arges County, there is no organized biological treatment of waste in urban areas.

In rural areas, the householders carry out the composting of biodegradable materials in their backyards. The resulting product is used as manure for agricultural purposes in their land. There is no detailed information about the amounts of compost produced, nor the quality.

#### Definition of demands, needs and main deficiencies

The present situation for the biological treatment of waste in urban and rural areas of Arges County describes a lack of structured and designed biological treatment process of biodegradable fractions.  Therefore, the amounts and the efficiency of these processes remains unclear. The main demand is the implementation of a biological treatment of the biodegradable fraction in the Zones, in order to explode its potential for compost production, and its potential for diverging, an important volume of material, from confinement in landfills.

The specific targets for the waste collection and transport are presented as follows:

Table 5.4-3 County objectives and targets regarding waste management

| Field/Activity | Objectives | Sub-Objectives | Targets | Responsible |
| --- | --- | --- | --- | --- |
| 10. Biodegradable waste | 10.1. Reducing the stored biodegradable waste | 10.1.1. A reduction in the amount of biodegradable waste (in comparison to 1995) | Reduction until 75% until 2010 Reduction until 50% until 2013 Reduction with 35% until 2016 | CJ, CL; sanitary operators and of all landfills |
|  |  | 10.1.2. Making new investments in the field of treating the stored biodegradable waste | Starting with 2011 | CJ, CL, professional associations, economic operators |

Table 5.4-4 Quantified county objectives and targets regarding waste management

| Biodegradable municipal waste | UM | 2010 | 2013 | 2016 |
| --- | --- | --- | --- | --- |
| Quantity of biodegradable waste generated in 1995 | Ton | 143.635 |  |  |
| Maximum amount of biodegradable waste that can be stored | Ton | 107.726 | 71.817 | 50.272 |
| Quantity of biodegradable waste from landfill to be reduced | Ton | 35.288 | 74.657 | 98.555 |
| Quantity of biodegradable waste generated | ton | 143.014 | 146.474 | 148.827 |

### Recycling and Recovery

The recovery and recycling infrastructure in Arges county is incipient and un-organized. There exist commercial entities working independently and the main sub-products for recovery are metal, glass, paper and plastic bottles. The amount of recovery materials are not available, nor registered.

There exists also the participation of the inhabitants in the project areas, as private persons collecting iron and paper and hand them over to recycling entities on their own. Paper is sorted in schools, high schools and public institutions. In the case of old batteries, it is mandatory to turn back the old battery when buying a new one. The sorted sub-products are transferred to the main societies in Pitesti. There is no available data about recent amounts of sorted sub-products or materials from solid wastes.

Table5.4-5 Arges economical agents for recovery of sub-products

| Place | Address | Equipment type | Environmentally approved | The carried out activity /Project capacity. (t/year) | Type of processed waste |
| --- | --- | --- | --- | --- | --- |
| SC REMAT AG SA Pitesti | Depozitelor nr.55 Pitesti | Balling press, China type press, hydraulic scissor, alligator scissor, cable splitting machine, extracting device for pulling out the metal from the cable, plastics shredder, drop hammer for breaking cast iron. | yes | Collecting, capitalizing of metals – ferrous (30000t/year), non-ferrous (300 t/year), paper (1000 t/year) ; Collection - treatment VSU – 6000 pieces/year | Grinded iron, pressed, span, steel, cast iron, non-ferrous, plastics, paper, VSU |
| SC Proremat SA Pitesti | Negru Voda nr.2 | scale, electric elevator, front loader | yes | Collecting, capitalizing of metals – ferrous 3600 t/year; collection VSU – 250 pieces/year | iron, cast iron, VSU |
| SC Metalimpex Romania | Village Ciumesti | Shredder, balling press, hydraulic scissor, scale, water treatment plant, mobile ramp, portable computerized contamination measuring device | yes | Collecting, capitalizing scrap iron -36000 t/year, non-ferrous metals-120 t /year, H/C-540 t/year, plastics -432 t/ year, wood – 2160 t/ year ; collection- treatment VSU -500 pieces/year | Metallic waste, paper, plastics, VSU |
| SC Eco Reciclare SRL | Str Caporal Dogaru 1 | Scale | yes | Collecting, capitalizing PET – 720 t/year ; foil -180 t/year | Wrapping waste PET |
| SC Dac 2003 Trade Center SRL | Stefanesti | scale, balling press for plastics | yes | Collecting, capitalizing PET 500 t/year | Wrapping waste PET |
| SC Salubritate 2000 SA | Targul din Vale | Bins | yes | Collecting, capitalizing PET – 168 t/year | Wrapping waste PET |
| SC Replast Prod Plast SRL | Str Doaga 1C | balling press for plastics | yes | Collecting, capitalizing PET – 600 t/year; foil– 600 t/year ; PE,PP – 480 t/year | Wrapping waste PET |
| SC Estelina Construct Building SRL | Bascov Village Rotaresti | Concrete platform, frontal loader, scissor, scale | yes | Capitalization of the ferrous stainless steel – 350 t/year, batteries – 10 t/year | Ferrous metallic waste, stainless steel, used batteries |
| SC Ecoref 2002 SA | Str. Depozitelor 20 | balling press, grinding mill, scale, front loader, transport lanes and selected | yes | Collecting, capitalizing PET – 1200 t/year, foil – 240 t/year; plastics – 240 t/year | Wrapping waste PET, foil |
| SC Electro Ghetas SRL Pitesti | Str. Garii nr.2 | Concrete platform, frontal loader, scissor, scale, laboratory with a spectrometer | yes | Collecting, capitalizing iron, stainless steel – 100000t/year | Ferrous metallic waste |

The last table shows the companies established in Arges County, their target material and their capacity. The sum of the capacities for every sub-product can give an estimation of the market capacity in Arges County, thus it is as following:

- Ferrous material: 		69.600 	t/year

- Non-Ferrous material: 	100.770 	t/year

- Paper:			1000 		t/year

- Plastics:			4.560 		t/year

- Others:			2.710 		t/year

- ELV:				6750 		units/year

#### Definition of demands, needs and main deficiencies

Main Deficiencies

- Recycled Capacity is too low to cover the recycling targets set by the Packaging Waste Directive

- Worn out and old collection equipment is used

- The capacity installed for recovery, by private entities is low

- Problem areas with no road connection at special sessions and after heavy rains

- Current limited separate collection opportunities for recyclables

Needs and Demands

- implementation of a comprehensive system recyclable waste collection and treatment in urban and rural areas, meeting the requirements of EU and national environmental and waste legislation

- collection system has to be made profitable

- Either the sorted recyclables will be shipped to another county or country or new recycling companies will be settling down in Arges County

- implementation of solutions for treatment of recyclable waste disposal and/or treatment, either on regional basis, serving all counties of the development region  or on county basis

- Increase public awareness campaigns and education

- build on the existing formal recycling sector and integrate the informal recycling sector, already operating in Arges County

### Existing of non-compliant disposal sites

The places for final disposal that do not comply with the corresponding requirements, MO 757/2004, are called dumpsites. In some cases, dumpsites are operated by the municipality with some measures of control in their activities, however as they lack the most of required infrastructure, they are still denominated dumpsites.

The final disposal sites in Arges County are differentiated in one main type, the final disposal in controlled sites. Starting with 16th of july 2009, in Arges County there is no disposal site that fulfils the corresponding legal and technical requirements for landfills. The landfills that will be closed in the second phase of the project are:

- Curtea de Arges	(will be closed according to MO 757/2004 )

- Costesti		(will be relocated and remediated)

In rural areas, dumpsites were already closed and remediated, according to deadline (July-2009).

Figure 5.4-2 In the same time with the closure of the landfills in Curtea de Arges and Costesti will be developed the transfer stations in Curtea de Arges and Costesti similar with Campulung

#### Definition of demands, needs and main deficiencies

In Arges County there are open urban waste deposits, all presenting important deficiencies:

- Are not fulfilling the requirements of the environmental protection norms in force: lack of waterproofing and sealing up closing with rainwater infiltrations;

- The deposits have no collecting systems for leaching and gases, directly flowing in the environment;

- The placements of the deposits are located in open spaces, and subsequently, the people and animals access is not under control;

- The influential waste quantity and quality (composition) is estimated without measurement.

All existing sites represent a severe polluting factor for the environment, especially for human health and is not fulfilling the requirements of MO 757/2004.Thus, these sites represent big sources of environmental pollution, as well as risks for the public health.

The main current demands are those related to the urban dumpsites from Zones 1,2 & 8 in Arges County, which correspond to the Phase-II of the WM Project in Arges County. These are:

- proper 2 step closure of the urban disposal site of Curtea de Arges city,

- Relocation and remediation of the urban disposal site in Costesti

The rest of urban and rural sites, from Zones 3,4,5,6 & 7, are currently within the remediation measures of Phase-I of WM Project in Arges County (see Section 2.5).

The specific targets for final waste disposal, according to the Local Waste Management Plan, Arges County, are presented in the  next table.

Table 5.4-6 Specific targets for final disposal sites in Zones 1,2 & 8

| Measure | Related objectives |  | Equivalence |
| --- | --- | --- | --- |
|  | Issue | Target |  |
| Closure of urban dumpsites |  | - Curtea de Arges 31.12.2010 - Costesti 31.12.2010 | 2 steps closure Relocated and remediated |
| Closure of rural dumpsites | Reducing the number of dumpsites | 16-July -2009 | N.A |

*As this figure is based on the total amount of biodegradable waste, generated in 1995, and there is no registers of that amount. The basis is considered from the oldest amount in 2002, for the total amount of Zones 1,2 & 8 (24,973 tons).

### Tariffs and Costs of Waste Management

The Tariff situation in Arges County, regarding the waste management is shown in the next table.

Table 5.3-7 Employed tariffs for the population in 2007 and 2008.

| Tariffs in RON / person | Zone 1 C. de Arges | Zone 2 Domnesti | Zone 3 Campulung | Zone 4 Rucar | Zone 5 Mioveni | Zone 6 Pitesti | Zone 7 Topoloveni | Zone 8 Costesti |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tariff for the population in 2007 – monthly | 3,06 | 3,5 | 3,12 | 1,66 | 1,73 | 3,34 | 2,5 | 2,28 |
| Tariff for the population in 2007 – yearly | 36,72 | 42 | 37,44 | 19,92 | 20,76 | 40,08 | 30 | 27,36 |
| Tariff for the population in 2008 – monthly | 3,06 | 5,3 | 3,37 | 1,66 | 2,07 | 4,54 | 2,5 | 2,28 |
| Tariff for the population in 2008 – yearly | 36,72 | 63,6 | 40,44 | 19,92 | 24,84 | 54,48 | 30 | 27,36 |

#### Definition of demands, needs and main deficiencies

Main Deficiencies:

- At present there are different tariff systems in the county

- Subvention of the existing system at the moment

- Collection rate of tariffs is not 100% at all operators

Needs and Demands:

- Implementation of and profitable waste tariff system and

- Polluter pays principle has to be implemented in the system

## Conclusion

The current waste management system, being in implementation, in Arges County is deficient. The collection rate is reduced to mainly urban areas and some rural communes. The rest of communes in rural areas have no waste collection service. This fact favours the creation and/or development of dumpsites, with the consequent effects to the public health and environmental impacts.

An important amount of material is being buried in disposal sites, without being utilized for recycling. The main cause for this is the lack of a separated system of waste collection and temporal storage of solid waste. A big fraction, of the solid waste from these Zones, is consisting in organic waste. This fraction is practically wasted in disposal sites, as there exist no possibility to process this material, in order to obtain compost.

The main deficiencies and quantified needs of the current waste management system is summarized and quantified in the following tables:

Table 5.5-1 Main deficiencies and needs in the Waste Management System

| Component | Main deficiencies | Quantified needs |
| --- | --- | --- |
| Biodegradable Waste | - currently dumping of biodegradable waste - Ground water Pollution, landscape degradation - no separate collection of biodegradable | - Establishment of biodegradable waste reduction measures due to legislation - Extension of public awareness campaigns - home composting measures in rural areas |
| Packaging Waste | - The existing recyclable waste collection is not efficient - Recyclables are collected by informal sector out of landfilled waste decrease of the quality of the recyclables | - Meet the requirements of legislation - Extension of the recyclable waste collection to the entired county - Extension of public awareness campaigns - collection system has to be made profitable |
| Household hazardous waste | - no separate hazardous waste collection in urban and rural areas - no disposal site for hazardous waste - no treatment facility for hazardous waste - Pollution of groundwater | - meet requirements of EU and national environmental and waste legislation - implementation of solutions for hazardous waste disposal and/or treatment |
| WEEE | - separate collection of WEEE at different locations - no separate collection of WEEE in rural areas - no WEEE recyclers in Romania - Landfilling of WEEE | - meet requirements of EU and national environmental and waste legislation |
| ELV | - few operators for treatment of ELV exists - quantity of treated ELV is increasing - no collection network | - implementation of a system after requirements of EU and national environmental and waste legislation - implementation of solutions for ELV treatment |
| Bulky Waste | - no separate collection of bulky waste - no disposal site or treatment of bulky waste (e.g. incineration and therefore energy recovery) | - implementation of a system after requirements of EU and national environmental and waste legislation - implementation of solutions for Bulky Waste treatment |
| Construction and Demolition Waste | - existing of construction and demolition wastes and also dumping on mixed municipal waste dumps - No reliable waste generation data exists | - implementation of a system after requirements of EU and national environmental and waste legislation |
| Collection System | - Collection coverage rate is low - Limited collection opportunities - Worn out and old collection equipment is used - Problem areas with no road connection at special sessions and after heavy rains | - implementation of a system after requirements of EU and national environmental and waste legislation - implementation of solutions for treatment of residue, recyclable and biodegradable waste disposal and/or treatment |
| Non-compliant disposal sites | - No compliancy of the existing dumpsites and landfills with EU regulations - Groundwater pollution - Landscape degrading - No waste recording system - Lack of deposit capacity for landfilling of household waste | - Construction of a new landfill, meeting the requirements of EU and national environmental and waste legislation - Implementation of treatment facilities reduction of landfilled waste quantity and especially on biodegradable waste. |
| Tariffs and Costs of waste management | - At present there are different tariff systems in the county - Subvention of the existing system - Collection rate of tariffs is not efficient | - Implementation of and profitable waste tariff system and - Polluter pays principle has to be implemented in the system |

The quantified targets for WM in Arges County in the different Phases are presented in the next table.

Table 5.4-2 Quantified targets and Phases of IWMS in Arges County

| No. | Component | Unit | Original Situation | County Target | Quantified Needs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Being remedied by Phase-I (ISPA investment) | To be remedied by Phase-II |
| Separated Waste Storage |  |  |  |  |  |  |
| a | Coverage in urban areas | persons | 307.772 | 303.1832013 | 259.9302013 | 43.2532013 |
| b | Coverage in rural areas | persons | 67.214 | 329.5432013 | 202.3412013 | 127.2022013 |
| c | Biodegradable fraction+ | ton/y | negligible | 2010=33.662 2013=84.223 2016=101.114 | 2010=23.164 2013=61.635 2016=75.777 | 2010=10.498 2013=22.588 2016=25.557 |
| d | HHW | ton/y | negligible | -- | -- | -- |
| e | Plastic | ton/y | Negligible | 5.368 | 4.269 | 1.099 |
| f | Paper | ton/y | Negligible | 11.649 | 9.264 | 2.385 |
| g | Glass | ton/y | negligible | 10.761 | 8.558 | 2.203 |
| Waste Collection |  |  |  |  |  |  |
| a | Coverage in urban areas | % | 90 – 100% | 100% | 86% | 14% |
| b | Coverage in rural areas | % | 20 - 25% | 100% | 61% | 39% |
| Transfer stations |  |  |  |  |  |  |
| a | Coverage in urban areas | % | -- | 100% | 86% | 14% |
| b | Coverage in rural areas | % | -- | 100% | 61% | 39% |
| Composting plants |  |  |  |  |  |  |
| a | Processing of biodegradable waste | ton/y | negligible | 2010=33.662 2013=84.223 2016=101.114 | 2010=23.164 2013=61.635 2016=75.777 | 2010=10.498 2013=22.588 2016=25.557 |
| b | Biodegradable disposed in landfill | ton/y | 106.000 | 2010= 107.726 2013= 71.818 2016= 50.272 | 2010=84.085 2013=59.347 2016=41.589 | 2010= 16.309 2013= 12.246 2016= 8.263 |
| Closure and remediation of final disposal sites |  |  |  |  |  |  |
| a | Curtea de Arges | site | Operating | 31.12.2010 | Operating | For Closure |
| b | Campulung | Site | Closed | 31.12.2008 | Closured | -- |
| c | Mioveni | Site | Operating | 31.12.2017 | Operating | -- |
| d | Albota | Site | Closed | 31.12.2007 | Closured | -- |
| e | Topoloveni | site | Operating | 31.12.2010 | Relocated | -- |
| f | Costesti | site | Operating | 31.12.2010 | Operating | For Relocation |
| g | Rural dumpsites | site | Operating | 16.07.2010 | 100% closed | -- |
| Final disposal in regional landfill Albota |  |  |  |  |  |  |
| a | Curtea de Arges | Ton /year | 0 | 2009-2013 | 0 | 22.216 in Cell-2 |
| b | Domnesti | Ton /year | 0 |  | 0 | 3.271 in Cell-2 |
| c | Campulung | Ton /year | 36.874 |  | 24.589 | 24.589 in Cell-2 |
| d | Rucar | Ton /year | 4.435 |  | 1.147 | 1.147 in Cell-2 |
| e | Mioveni | Ton /year | 31.631 |  | 21.972 | 21.972 in Cell-2 |
| f | Pitesti | Ton /year | 97.719 |  | 103.908 | 103.908 in Cell-2 |
| g | Topoloveni | Ton /year | 10.871 |  | 8.964 | 8.964 in Cell-2 |
| h | Costesti | Ton /year | 0 |  | 0 | 12.576 in Cell-2 |

*This figure represents the total biodegradable to be diverted from landfill, it includes individual composting and biodegradable material at composting plants