6	Sludge management	102

6.1	The management of sludge from the treatment processes	102

6.2	Current sludge Management in Arges County	103

6.3	Legal frame	106

6.4	Options for sludge management in Arges County	107

6.4.1	Options for sludge management – Piteşti project area	108

6.5	Conclusions	111

# Sludge management

For all the water treatment stations it was foreseen the recovery of the waters used to wash the filters. Currently, these waters are discharged directly in the river, with out a previous treatment.

It was foreseen the sedimentation of the technologic waters (min. 1 h), the recovery of the  supernatant liquid (with the re-introduction in the brute water circuit at max. 3 – 5 % from the influenced debits in the treatment station) and dehydration of the sludge (w = 70%).

The systems for treating the sludge including the thickening in the brighteners, the pumping, the dosage with polymer and the dehydration will be designed as integrated systems equipped with evaluation of the efficiency for several components of the process.

After dehydration, the resulted sludge quantities will be included in the afferent circuit for sludge management, sludge resulted from the treatment of the used waters, or they will be stored and reutilized in the construction materials industry.

## The management of sludge from the treatment processes

General

Through the treatment process of the used waters there are separated both the pollutants which are concentrated in the sludge and also the material formed during the treatment process (example: exceed sludge).

The main types of sludge which result from the waste water treatment processes are:

- Primary sludge (mechanical step);

- Secondary sludge (biologic step);

- Mixed sludge (mechanical step with sludge from the biologic one) ;

- Sludge from the upper treatment step.

From the sludge management process, from the treatment stations the following aspects can be mentioned: the raw sludge (not treated), the stabilized sludge (aerobic or anaerobic), the dehydrated sludge (natural or artificial) and the stabilized sludge (resulted through solidification).

Sludge characteristics: humidity, specified weight, filtration level, thermal level, pH, MTS, mineral and volatile fraction, the fertilizing substances and the pathogenic potential.

Depending on the properties of the sludge resulted from the treatment processes resulted from the treatment processes and on the local conditions one of the processing and evacuation methods are presented in the following table.

Table 9.1-1 Types of methods for processing and discharge of sludge

| The type of process | The obtained results | Method |
| --- | --- | --- |
| Thickening | Water removal Reduction of the volume | Sedimentation Floating Filtration Centrifuging |
| Stabilization | Reduction of the pathogenic factors Production of fermentation gas | Anaerobic fermentation Anaerobic stabilization |
| Conditioning | Structure modification Improvement of the filtration and compaction on levels | Chemical conditioning Thermal conditioning Inert material conditioning |
| Disinfection | Reduction o the pathogenic factors | Pasteurization Lime treatment Chlorine treatment Composting |
| Dehydration | Removal of water | Drying Vacuuming Filtration Centrifuging |
| Drying | Advanced removal of water Sterilization of the material | Drying |
| Burning | Destruction of the organic material Sterilization | Incineration Pyrolyses |
| Elimination | Removal from the installation Capitalization on the useful material |  |

The main methods for sludge recovery are:

- Technologic capitalization (recovery of the useful products in the industrial processes);

- Energetic capitalization (as secondary resources and as regenerating energy);

- Agricultural purposes

## Current sludge Management in Arges County

The sludge management situation for each of the 7 urban localities from Arges County is presented in the following table

Table 6.2-1 Sludge management in Arges County

| Locality | Sludge management |
| --- | --- |
| Piteşti | Within the ISPA 2003/RO/16/P/PE/026 measure, currently being implemented, there are comprised the rehabilitation works from the Pitesti Treatment Station (including the reduction of the nitrogen and phosphorus components). For the sludge line, there are provisioned the following elements: The primary and biological sludge in excess (SAS), will be thickened separately in the existent gravitational separators and in the new mechanical ones. SAS and the primary thickened sludge will be pumped in two circular storage tanks provisioned with blenders for insuring the required degree of mixture. The thickened sludge will be pumped to the anaerobic fermentation devices, and from here it will be directed by gravity to two new covered storage tanks, after which it will be pumped to the dehydration installations with centrifugal extractors in order to remove part of the water from the sludge; before of the dehydration, the sludge is conditioned with polyelectrolyte. The resulted water will be returned to the admission in the pumping station through the new sewage system. The S.A.S. quantity which must be eliminated is equal to the accumulated amount of material found in suspension produced as a result of an increase in the bacterial increase and in the flocculation processes. The amount of dry sludge which will develop in the system will be equal to: Xs = Yb ( So-Se ) * Qd - kd * V * x + YN [( TKNo - TKNe ) - 0,05 ( So - Se) ] * Qd in kg substance/day where: Yb = BOD cellular efficiency coefficient = 0,94 YN = TKN cellular efficiency coefficient = 0,24 kd = bacteria disappearance factor at the designed temperature = 0,032 V = total volume of the aeration tanks Also, the chemical elimination of the phosphorus, obtained through dosage of the ferric chloride in the biologic treatment will produce a flux of sludge which has to be added to the biologic S.A.S. in order to obtain the Overall Surplus of Active Sludge. The dried percentage from the active surplus of sludge extracted through the secondary clarifier is supposed to be equal with 0.9% = 9 kg/m3. - primary sludge Two new pumping stations for primary sludge will be built, one for the DP3 and DP4 clarifiers and the other one for the DP5. Each clarifier will have underwater pumps powered on and in stand-by. The sludge will be directed in the two rehabilitated radial thickeners. Thickener of primary sludge The primary sludge is pumped directly at the two existent gravitational thickeners which will be rehabilitated at the right moment. A dosage point with polymer is foreseen for each thickener in order to achieve a concentration of dried substance in the thickened sludge of 5%. Mechanic thickener This new system is comprised out of two thickening equipments with cylinders completed with the flocculation reactor and it will be mounted in the existent dehydration building. With an influent with a SS concentration of 0.9%, the maximum discharge of sludge for each thickener with a cylinder is of 40 cm / h. The concentration of the dried matter after the dynamic thickening will be of 6%. The liquids from the thickener will be collected in the internal sewage system and re-circulated gravitationally towards the new storage tank for re-circulated liquids. Each mechanical thickener has its own transfer worm and wheel pump of the thickened sludge for fermentation. - tank for the mixture of the thickened sludge In order to obtain a homogenous mixture of primary sludge and sludge thickened in excess there will be built two covered circular thickeners, with a diameter of 14 m. - the anaerobic fermentation of the sludge The stabilization of the sludge will be made through an anaerobe digestion, by using a vertical mixing system with double propulsion. For the calculation of the volume of the fermented sludge the experimental Pöpel formula has been employed: V = Q * 175 * 10-0,03*T where Q is the sludge entered to be fermented and T the process temperature. The fermented sludge production The produced fermented sludge quantity will be calculated with the following formula: SSd = SNV + SVd = SNV+ SV*1.065*(1- GT /1,866) SSd; Material in suspension; kg/d. SNV; Non volatile solid matters; kg/d. SVd; Volatile solid matters; kg/d. - Buffer tanks for the fermented sludge The fermented sludge will be deposited in two new covered circular thickeners. This chamber plays the part of accumulator and mixer for the fermented sludge before the next step, the dehydration of the sludge step, and it will be equipped with a blender in order to insure the continuous mix of the sludge. - the mechanical dehydration of the sludge The new dehydration unit will have 3 centrifugal machines (2 on / 1 in stand-by), with a maximum capacity of 650 kg TSS / h. These will be arranged in the existing dehydration building, including 2 stations for preparing polymers, with a capacity of 3.500 l / h (for a total capacity of 7.000 l / h), with a dosage pump with an adjustable capacity. The sludge witch will enter in the blender will be pre-conditioned with polyelectrolyte. The polymer preparation station will be emplaced and arranged in a way that it will not obstruct the filling with polymer, the running and the maintenance. The polymer dosage ratio (10 g poly / kg SST) will be manually imputed and the feeding pumps for the centrifuge will control the running of the dosage pumps. The aging and dosage tanks will be made out of stainless steel. The dosage unit will run fully automated, together with a centrifuge and with the sludge feeding pumps. - Storage of the dehydrated sludge cakes The future storage surface will be of approximately 22.500 m2. Assuming a usual storage surface, there will be provisioned 33.750 m3 as storage volume, which will correspond with a storage period of approximately 470 days. Studies and researches regarding the management of sludge In close cooperation with the Agricultural Research Station (ARS), with the Office for Pedology and Agricultural Chemistry Sciences (OPACS), and with the National Authority for Scientific Research (NASR) the Study regarding the usage in agriculture of the processed sludge from the Municipality of Pitesti Treatment Station and the evaluation of the impact had on the agricultural production levels and on the soil was written. The conclusion drawn was that the sludge resulted from the treatment of the urban waters can be used in agriculture as a soil enhancer for soils, mainly for the acid ones, situation in which the pollution factor does not occurs: „The residual organic sludge obtained in the current stage knows both the anaerobic fermentation and also dehydration. In these conditions there will be a certain statistical increase in the harvest, but also an increase of the soils’ quality. In parallel with the elaboration of the previously mentioned studies the following actions were taken: - The visa of the Rural Agricultural and Development no. 2172/28.05.2008 was given; - It was elaborated and forwarded to APM Pitesti the documentation which constituted the basis for the Awarding Permit no. 3038-1/30.05.2008; - In 2009, actions regarding the seeding of a corn crop on a surface of 52 ha, on the land given by the Stolnici Land Improvements and Agricultural Association were taken, 1100 t of sludge being spread and incorporated. At the same time with the sludge usage actions in agriculture, another study is being run regarding the energetic capitalization of sludge. The following step will be for the strategy regarding the management of sludge from the treatment station to be set at a local/regional level inside the elaboration period of the Feasibility Study. |
| Câmpulung | The primary sludges and the ones in excess pumped are put in the homogenization tank (the former thickener for the primary sludge), being sucked and put in the mechanical thickener installation. The thickened sludge is pumped in the anaerobic fermentation tank (1 piece). After fermentation, the sludge is evacuated in the storage tank (the former thickener for the fermented sludge), from where it is sucked by the pumps of the mechanical dehydration installation and put in the mechanical dehydration installation. The dehydrated sludge has a humidity of approximately 78% and it is unloaded in special containers, being then transported by car to the city landfill. The existent pumping station is constructed out of a reinforced concrete coffer equipped with (1+1) submersible pumps EPEG; in the current scheme it plays a double role, to pump the primary sludge through primary clarification beds in the sludge concentrator, but also to pump the concentrated sludge in the fermentation tank or directly on the platforms set up for drying. The conclusions of the study done in collaboration with the Office for Pedology and Agricultural Chemistry Sciences (OPACS), pointed out the fact that because of the enhanced concentration of micropolutants existent in the sludge from the Campulung Treatment Station it can not be used in agriculture. |
| Mioveni | The sludge resulted in the mechanical treatment process it is directed the reception tank of the primary sludge station, from where it is pumped into a sludge thickener with a diameter of 8 m, for the purpose of lowering the humidity levels. From here it is brought gravitationally in the reception tank of the thickened sludge pumping station for the purpose of inserting it in the RFN, foreseen with heating and sludge recirculation installations. The excess activated sludge, resulted in the treatment installations during the biological step it’s taken gravitationally in the aspiration tank of the sludge recirculation pumping station from where through pumping it is partially sent in the aeration tank (recirculation sludge) and the other part is sent to the distributor, from where, by mixing it with water, it enters the primary clarification bed. The sludge fermented in the methane tank is sent gravitationally on the sludge drying platform. The conclusions of the study done in collaboration with the Office for Pedology and Agricultural Chemistry Sciences (OPACS), pointed out the fact that because of the enhanced concentration of micropolutants existent in the sludge from the Mioveni Treatment Station it can be used in agriculture. as a soil enhancer. |
| Curtea de Argeş | The sludge resulted from the Imhoff clarification beds is evacuated through a siphon directly on to the drying platforms. The sludge resulted from the secondary clarification beds, with the exception of the activated sludge, it is sent to the sludge fermentation tank, and after the mesophilous fermentation (30-35 0C) it is sent to the sludge drying platforms. The recirculation sludge it is introduced concentrated uphill for all the aeration tanks. The evacuation of the activated sludge is done through a siphonage action in the aspiration tank of the pumping station of the activated sludge. The evacuation sludge of the secondary clarification beds is done through syphonage in the „pocket„ united with the cable drag scraper bridge, from where the activated sludge is pumped, by a pump endowed with a vertical axle, in the above-ground activated sludge collection gutter (there are afferent to each group of two clarification beds a collection gutter). From the activated sludge collection gutter, it is directed gravitationally upstream from the activated sludge tanks, the difference being sent gravitationally, as excess sludge, upstream of the primary clarification beds. In case the evacuation of sludge can not be realized as described above, there have been foreseen, starting from each clarification bed, sludge evacuation pumps (through a hydrostatic difference in pressure) directly in the aspiration chamber of the pumping station, from where it is sent through pumping upstream of the primary clarification beds. The pumping equipments of the sludge resulted from the treatment processes: Station for pumping primary fermented sludge and supernatant Activated sludge pumping station for recirculation and excess Primary sludge pumping station in mixture with the one found in excess. The drying platforms receive the primary anaerobe fermented sludge from the technologic line I and also the anaerobe fermented sludge afferent to the technologic lines II and III (from the sludge fermentation tanks) in order for them to be dehydrated naturally. These platforms have been provisioned to be 10 pieces with the following geometric dimensions: width Bi = 22,0 m, length L = 34,0 m and working depth hu = 0,50 m. After drying, the sludge is transported to the landfill (just a small part can be used in agriculture). The conclusions of the study done in collaboration with the Office for Pedology and Agricultural Chemistry Sciences (OPACS), pointed out that the sludge from the Curtea de Arges Treatment Station can be used in agriculture as a soil enhancer. |
| Costeşti | The treatment process is not compatible with the quality indicators of the used waters. The overloading with organic materials, fact due to the used waters from the Pig farm determines a very low efficiency of the (CCOCrinfluent = 268,8 mg/l and CCOCrefluent = 1046,4 mg/l). The aeration tanks function as horizontal clarification beds in which the anaerobic/aerobic fermentation of the sludge takes place. The purging of the sludge from these tanks is done manually, because there aren’t provisioned hydraulic cleaning equipments. The conclusions of the study done in collaboration with the Office for Pedology and Agricultural Chemistry Sciences (OPACS), pointed out that the sludge from Costeşti Treatment Station can be used in agriculture as a soil enhancer. The following step is for the sludge management strategy regarding the sludge from the treatment station to be established at a local/regional level with in the Feasibility Study elaboration step. |
| Topoloveni | The treatment station is not efficient and the necessary endowments for insuring an adequate treatment process are lacking, reason why the quantities of sludge are reduced. The sludge resulted from the sedimentation and fermentation processes is sent to the drying platforms (6 pieces X 10 x 16 m). The dried sludge is periodically cleaned, being used in agriculture. The conclusions of the study done in collaboration with the Office for Pedology and Agricultural Chemistry Sciences (OPACS), pointed out that the sludge from Topoloveni Treatment Station can be used in agriculture as a soil enhancer. The following step is for the sludge management strategy regarding the sludge from the treatment station to be established at a local/regional level within the Feasibility Study elaboration step. |

Suource: Data from the local operators

The conclusions of the study performed in collaboration with the Office for Pedology and Agricultural Chemistry Sciences (OPACS), pointed out the following:

- Excepting the sludge from SE Câmpulung, the processed sludge quantities from the other treatment stations can be used in agriculture as soil enhancerers, especially for the acid ones, situation in which it does not generate pollution.

As a result of the reengineering and modernization, the treatment stations will have to comply with the rules imposed by the management of the sludge resulted from the treatment of the used waters (Directive 86/278/CCE regarding the environmental protection and mainly soil protection, when there is employed sludge from treatment stations, applied by MO 334/2004 completed with OM 27/2007).

## Legal frame

The Directive 86/278/CCE regarding the environmental protection and mainly soil protection, when there is used sludge from the treatment stations, was applied by OM 344/2004 in order to approve the Technical Norms regarding environmental protection and in special soil protection, when there is employed sludge from treatment stations in agriculture. This order was modified through the order no. 27/2007 regarding environmental protection and in special soil protection, when there is employed sludge from treatment stations.

The objectives of the Directive are: the regulation of treatment sludge in agriculture so that it will prevent harmful effects to occur on soil, vegetation, animals and public health, at the same time encouraging their proper use. It imposes limit values for heavy metals (cadmium, copper, nickel, lead, zinc, chrome, mercury) found in sludge and soil. The usage of sludge must be forbidden when the concentration of these metals in the ground surpasses the limit values. It encourages the capitalization of the treatment sludge in agriculture by showing that their usage will not harm the quality of the soil and the level of the harvest.

The MO 334/2004 establishes the concentrations of heavy metals (cadmium, copper, nickel, lead, zinc, chrome, mercury) in the soils on which the sludge is applied, the concentrations of heavy metals found in sludge and the maximum quantities of annual heavy metals which can be inserted in the soils intended for agriculture. It forbids the usage of sludge when one or more heavy metals found in the soils exceeds the established limit values and sets the necessity to take measures in order to ensure that these limit values will not be exceeded as a result of the usage of sludge. There are also set the obligations and responsibilities of the sludge producers.

In accordance with GEO no.78/2000 regarding the waste regime, amends and approves through law no. 426/2001, the sludge from the Treatment Stations which is classified as hazardous waste if it is not treated or treated adequately for their use in agriculture and if they have in their composition substances which have explosive, flammable, irritant, oxidative, toxic, corrosive, properties etc.

GO 856/2002 regarding the management of solid waste and the approval of the list of waste, including hazardous waste, designates code 1908 for the unspecified waste from the treatment stations. Out of these, the following are classified as hazardous:

- Saturated or dehydrated resins with ion exchange;

- Solutions or sludge resulted from regeneration processes of the resins with ion change;

- Waste which contain heavy metals;

- Mixes of oils and fat resulted from the separation processes, in other sectors than the sector for eatable oils and fats;

- Sludge which contains hazardous waste from the biological treatment of used industrial waters;

- Sludge which contains hazardous waste resulted from other industrial waters treatment processes;

Incineration of waste is regulated by:

- The order of the Ministry of Environment and Durable Development no. 756/2004 through which the technical norms are approved regarding waste incineration; according to this norm, urban sludge is sludge resulted from the treatment stations, no mater if the treated water is household or industrial;

- GD no. 128/2002, amended by GD no. 268/2005 regulates the incineration of waste; it also establishes the limits of the emissions the incinerators have to uphold.

## Options for sludge management in Arges County

In the treatment process of the used household and industrial waters, there will appear huge quantities of sludge, in different stages of wetness, sludge rich in organic substance, with high levels of nutrients necessary to agriculture. In some cases the resulted sludge has a high fermentation strength which will imply nasty smells and an increased virulence, at also a high degree of ions and heavy metals.

In order to reduce significantly the fermentation power and the sanitary risks involved through usage, the sludge is stored for a long time. The removal of these disadvantages is done through biologic processes, thus dealing, on short and average periods the possibility of storage of the sludge resulted at the water treatment station.

Because of the large quantities of sludge resulted from the water treatment process of the used waters, and also because of the similarity of the chemical composition with that of stable waste, with assimilation with this, it was proposed to be used as fertilizer and soil enhancer for different crops.

Its purpose is not only the removal of this waste from the natural circuit through biological degradation into the soil, degradation which releases nutrients necessary to plant growth, for the purpose of obtaining unprocessed vegetal products, according to GD 917 /13.09.2001, but also for the purpose of implementing an extended biological agriculture.

Through the biological decomposing in the field germinator bed, there is realized simultaneous not only a decomposing of some biologic components harmful for human health (bacteria, viruses and other microorganisms etc.) thus being realized an active cleaning based on soil natural biological processes, but also a fertilization of the soils, by directly giving to plants the nutrients necessary for their optimum development.

On the other hand, there must be pointed out that although the sludge represents a source of nutrients for plants, the microelements present in variable quantities, can be fitotoxic if they are excessively incorporated into the soil, over the critical norms.

### Options for sludge management – Piteşti project area

Since 23.01.2004, SC APǍ CANAL 2000 SA has signed a partnership protocol with Albota Agricultural Research Station for the elaboration of a study called “Study regarding the usage in agriculture of the processed sludge with in the Pitesti Municipal Treatment Station, and its impact on (soil) production and environment”

The study regarding the usage in agriculture of processed sludge from Piteşti Municipal Treatment Station and evaluation of the impact had on agricultural production and soil was completed in 2007, in partnership with Agricultural Research Station (ARS), with the Office for Pedology and Agricultural Chemistry Sciences (OPACS), and with the National Authority for Scientific Research (NASR).

The conclusions of the study have shown that the sludge resulted from the treatment of the used urban waters can be used in agriculture as a soil enhancer, in special for the acid ones, situation which does not generate the apparition of pollution: “The residual organic sludge obtained in the current stage experiences both an anaerobic fermentation and also a dehydration process. So, there is not only a statistical increase in the harvest but also an increase in the fertility quality of the soil.

The data found in the specialty literature (Cornell University and Cornell Waste Management Institute) shows that the organic part of the sludge from the treatment of the used city waters contains over 2000 organic compounds and isomers, their concentration being situated in a wide field, from 0 – 7600 ppm, depending on the considered compound

Regarding the usage in agriculture, the study realized also an analyze of the chemical composition of the sludge, dealing mainly with the agrochemical traits, as also a with the content of microelements (specially heavy elements).

The obtained analytical data, shown in the following table, points out the following aspects:

- The sludge is an organic fertilizer, having a field of variation of the organic carbon content 54 – 70 % Co;

- Sludge can be considered as organic fertilizer with mineral nitrogen (nitric and ammonium ions), having around 75,8 ppm N directly accessible to plants;

- Sludge is an organic fertilizer with potassium (0,35 %) and with zinc (1362 ppm);

The content of mobile heavy metals from the active sludge, shows the following values:

- led: 57,95 % of the total;

- cupper: 15 % of the total;

- zinc: 2,36 % of the total;

- cadmium: 17,32 % of the total;

- nickel: 19,44 % of the total;

- manganese: 5,68 % of the total.

Because of the chemical structure, and also because of the macro and microelements presented, very similar to stable waste, the resulted active sludge from the treatment of the city used waters can be used in agriculture, as a soil enhancer of the deficient physical - chemical traits of soils (fertility reaction and traits), or as organic enhancer, an ecologic alternative to mineral fertilizers.

In the following table there is presented the Chemical Structure of the active sludge, resulted from the treatment of the used waters from Pitesti Municipality (average values):

Table 6.5-1 The chemical structure of the active sludge

| No. | Total forms |  | Mobile forms |  |
| --- | --- | --- | --- | --- |
|  | Chemical indicator | Value | Chemical indicator | Value |
| 1 | pH | 7,16 |  |  |
| 2 | Corg., % | 53,8 – 69,5 | N – NO3 , ppm | 29,1 |
| 3 | Nt, % | 2,29 | N – NH4, ppm | 46,7 |
| 4 | Pt, % | 1,33 | P – AL, ppm | 76 |
| 5 | Kt, % | 0,35 | K – AL, ppm | 93 |
| 6 | Na, % | 0,047 | Na – AL, ppm | 78,4 |
| 7 | Ca, % | 1,38 | - | - |
| 8 | Mg, % | 0,21 | - | - |
| 9 | Pb, ppm | 88 | Pb, ppm | 51 |
| 10 | Cu, ppm | 116 | Cu, ppm | 17,4 |
| 11 | Zn, ppm | 1362 | Zn, ppm | 32,2 |
| 12 | Cd, ppm | 4,1 | Cd, ppm | 0,71 |
| 13 | Ni, ppm | 36 | Ni, ppm | 7,0 |
| 14 | Mn, ppm | 317 | Mn, ppm | 18 |
| 15 | Cr, ppm | 112,4 | - | - |
| 16 | Co, ppm | 5,1 | - | - |
| 17 | Humidity, % | 68,2 – 82,3 | - | - |

Source: Study regarding the usage in agriculture of the sludge processed in the Pitesti Municipality Treatment Station.

The average chemical elementary composition of the resulted sludge from the treatment of the used urban waters indicates an average content of 50 % carbon, 37 % oxygen, 5 % nitrate and hydrogen, approximately 2 % sulfur and 1 % other elements, predominant being the microelements specific to the plants’ mineral nutrition.

Figure 6.5-1 The average elementary composition of sludge resulted from the treatment of municipal waste waters (Piteşti)

In the following table, it is presented the heavy metal content of the sludge from Pitesti Treatment Station, as also the quantities of sludge resulted during 2005-2007.

Table 6.5-2 Content of heavy metals found in the sludge from the Pitesti Treatment Station,

| Year | Quantity of sludge (t) | Total chrome mg/kg | Led mg/kg | Cadmium mg/kg | Cupper mg/kg | Nickel mg/kg | Zinc mg/kg | Cobalt mg/kg | Manganese | Obs. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - | 500 | 300 | 10 | 500 | 100 | 2000 | 50 | - | Max. value according to Order 344/2004 |
| 2005 | 1795 | 101 | 82 | 3,84 | 132 | 30 | 1284 | 6,6 | 324 | Average |
| 2006 | 1243 | 135 | 106 | 7,2 | 154 | 40 | 1492 | 4,3 | 400 | Average |
| 2007 | 1040 | 79,6 | 140,28 | 6 | 64 | 85 | 412 | 30,3 | 137 | Average |

From the “Study regarding the usage in agriculture of the processed sludge from the Municipality of Piteşti Treatment Station, and the impact had on (soil) production and environment”, in which there were studied the after effects of using activated sludge resulted from the treatment of the city’s household waters, we can draw the following conclusions:

- The soil on which the test were conducted is of brown luvic type (acid), with a natural low fertility and a high degree of aluminium and manganese ions, ions which generate a possible acidity and maintain an actual high acidity;

- Through the increased input of nutrients, contaminants (pollutants) and energy, the active sludge determines important modifications of the chemical structure of the soil, which comprises the total and also the mobile forms.

- The after effect of using active soil on the brown luvo soil determines the continuous reduction of the two components of the potential acidity (Al + Mn), the level of the reductions being co-dependent on the administered sludge quantity;

- Keeping the fertility of the soil at such a level that it will enter a superior class of fertility for higher dosages than 10 t/ha active soil;

- Indifferent of the age of the soil recovered samples for tests, and also of the forms of the (mobile or total) heavy metals, their quality has increased in direct proportion to the applied level of active sludge and the degree of fertility of the soil, the variations being  significant from the statistic point of view;

- The specific modifications of the heavy metals found in the soil (mobile and total forms) were determined with the help of the value of the tangent of the connexion line between the amount of active soil and the content of heavy metals found in the soil. These lines are statistically ensured through, most of the time, very significant coefficients;

- In order to appreciate correctly the degree of soil pollution with heavy metals, it is necessary to establish the connections which exist between the heavy metals, in order to establish the existent type of interaction (direct, synergism, inhibiting action).

- The co-dependency relation between the organic matter of the soil, found as organic carbon and total nitrogen, and heavy metals is of grave importance, because it’s the main source of inactivation of the heavy metals, in order to reduce drastically the heavy metal contamination/pollution, in order to have a durable, non-pollutant agriculture;

- Cupper ions, nickel and manganese can react with organic matter resulting complex mixes of different chemical stability, the chain of heavy metal ions being the next: Mn  < Cu <  Zn <  Pb < Cd < Ni;

- From these ions, only the cupper and manganese ones are the only ones which can be inactivated by the organic material, no matter the material found in the soil;

- The direct connexion between the cadmium/nickel and the organic mater, proves the fact that these heavy metals react mainly with huminic acids and less with fulvic ones from the MO;

- In order to test the degree of pollution/contamination of the soil with heavy metals, there was proposed to be used a synthetic index, illustrated by the report between the the total concentration of heavy metals and the standard value specified in the Order no. 344 / 16.08.2004.

- The manganese, although it is a heavy metal, in order to establish the degree of the contamination of the soil with heavy metals was not taken into consideration, because the soil on which it was tested the active soil it is an acid one, it being naturally rich in mobile Mn in quantities of over 800 ppm. In these conditions, the soil input is much higher than the input of the active soil.

- Taking into consideration the given experimentation conditions, the values of the degree of contamination/pollution are situated  at maximum half of the standard interval, no matter the heavy metal used;

- The multiple contamination value with heavy metals has a decreasing tendency by insuring the optimum nutritional conditions for plants, as a result of the increase in plant export, thus contributing to the diminishment of the soil contamination/pollution with heavy metals;

The obtained experimental data can be utilized to realize an expert program, necessary in order to be able to specify the optimum dosage of sludge which will be applied, as also the evaluation of the effect had by applying sludge on soil – plant.

## Conclusions

According to the above information, the Master Plan for WWTP of Arges County is not considering the landfill as an option for final disposal of Sludge from Water Treatment. However, recently three cities have approached the PIU, notifying that 6.100 ton/year of sludge is planned to be disposed in Albota landfill (see Annex 6.6-1). The amounts are the following:

Mioveni - 			3.000 ton/year

Curtea de Arges -		 3.000 ton/year

Campulung 		100 ton/years

Moreover, from the area that will be Operated by the Regional water Operator (Pitesti and surroundings, Costesti, Topoloveni, etc), there will be disposed 9.900 tons/year. So in total there will have 16.000 t/year of sludge to be disposed at landfill. For this reason, this amount is considered for waste landfill as follows:

Sludge: 			14.545.000 m3/year (35% TS)

Tons: 			16.000 tons/year (considering a density of 1,1 ton/m3)

Volume for disposal: 	5.090 m3/year (considering the 35% of TS)

The amount to be registered at Albota WMC is 16.000 ton/year (through weighbridge) for invoicing purposes. However, the volume to be able for disposal is 14.545 m3/year.

In theory the total mentioned volume of sludge may be used for composting purposes; this will depend on chemical composition of sludge as high concentrations of some components can danger the composting processes. In order to be conservative, the considered amount for using in Composting plants is 40%, thus for disposal will be only 60% of the total volume of sludge that is around 3.150 m3/year.

This volume for disposal represents approximately less than 3%, of annual volume of waste being disposed in regional landfill Albota, which is a very small volume and do not overpass the limit set by the corresponding legislation.

The Romanian Legislation by Order 757/2004 allows mixing sludge with household waste in a rate of 1:10 from the deposit rate