8	Option Analyisis	127

8.1	General	127

8.2	Methodology and Evaluation Criteria	127

8.3	Components Analysis	128

8.3.1	Tempory Storage	128

8.3.2	Waste Collection	140

8.3.3	Number of Transfer Stations	158

8.3.4	Waste Transfer Configuration	168

8.3.5	Biodegradable Waste Treatment	174

8.3.6	Waste Sorting	183

8.3.7	Final Disposal	191

8.4	Integrated Waste Management Systems Option Analysis	192

8.5	Financial and Economic Analysis of options	199

8.6	Conclusions	201

# Option Analyisis

## General

In view of current ongoing projects in Arges County, see Section 2.5. Phase-I of the Integrated Waste Management in Arges County is already in implementation, and three other projects in rural areas of Zone 1. The last three projects cover all the stages of waste management, except one, the final disposal of SW. The regional landfill of Albota is planned to serve all the county’s zones. This is completely according to the NWMP, where integrated WM systems should be developed at county or rather regional level with a view to optimising costs and reducing the negative environmental impacts.

As the projects in the above mentioned area are already being implemented, it would be pointless to analyse solutions for those Zones. However, the infrastructure will be taken into account, for the present section, where options will be analysed to integrate both Phases of the IWMP of Arges County. Hence, the present Chapter presents only the option analysis for the measures to be implemented in PHASE-II of the Integrated Waste Management Project in Arges County. PHASE-II corresponds to Zones 1,2 & 8, except for areas with ongoing projects, see Section 2.5.

## Methodology and Evaluation Criteria

The option analysis has been divided into 2 main parts.

- The first consists of the analysis of the components for waste management, i.e. tempory storage, collection, transfer, biological treatment and final disposal.

The components analysis will achieve of the following:

- General targets (national and regional-/ local-specific);

- Quantified targets;

- Demands to justify the proposed measures.

The selection of the most suitable component will be carried out according to the following Methodology:

Evaluation Process Criteria

Economical

Environmental

Resource saving

Targets  accomplishment

Society

Operational

Flexibility/Adaptability

Selection

of

Component Alternative

Alternative A

Alternative B

Alternative C

Alternative D

Figure 8.2-1 Evaluation process methodology for components

The evaluation process will be carried out as follows:

  - A minimum of two options will be compared at every stage of waste management in Arges County,

  - The options will be chosen according to regular and standardized WM practices in the European Community, which fulfill the minimal requirements of corresponding legislation;

  - The options will aim at the most economic and efficient solution, without endangering human health and/or the environment

  - the features of the quantitative and qualitative options will be weighed to obtain comparable values.

  - The comparable values are given as follows:

    - Negative impact 			= Value of “0 points”

    - Negligible or minor impact		= Value of “0,5 points”

    - Positive impact			= Value of “1,0 point”

- The second part is regarding the analysis of integrated systems. The options will consist of options of Phase-II WM, integrated to the existing WM system in Phase-I.

The final assessment of the alternatives will have to be made based on the trade-off between the economic criterion, which basically means total costs of the individual alternatives expressed by the Net Present Value (NPV), and the results for the environmental, social and technological criterion.

## Components Analysis

### Tempory Storage

The objectives of the RWMP for waste collection in the Regional Waste Management Plan are as follows:

- Ensure collection capacities are adapted to the number of inhabitants and to the quantity of generated waste.

  - Extend the waste collection areas to: rural: 95% by 2010, and urban: 100% by 2013.

- Ensure the best options for the waste collection and transport, correlated with recycling and final disposal activities.

  - Separate collection of hazardous household waste.

  - Implement selective collection of recoverable materials so that statutory objectives for packaging and biodegradable waste are achieved.

To achieve the above-mentioned objectives, the options for tempory storage are as follows:

#### Domiciliary (Mixed Storage )

| The domiciliary waste storage consists of the procurement of specialized bins and/or containers for households in urban and rural areas of Arges County. The stored waste from the bins and/or containers is typically associated with door-to-door collection by compactor trucks. This procedure can be performed mechanically and/or manually. As one of the main targets of tempory storage is to avoid health hazard and because the storage period of waste is limited, a collection frequency of 2 times per week is required with the domiciliary storage alternative. A minimum of one bin per household with a capacity of 120 L is required. | Figure 8.3-1 Mixed waste storage - 1 bin for residual waste |
| --- | --- |

  - Investment Costs and O&M Costs

| Calculation of Containers |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Domiciliary |  |  |  |  |  |  |  |  |  |  |
| Storage type: | Mixed |  |  |  |  |  |  |  |  |  |  |
| Container: | 0,12 | m3 |  |  |  |  |  |  |  |  |  |
| Density | 0,35 | ton/m3 |  |  |  |  |  |  |  |  |  |
| Members /Family | 3 | Persons |  |  |  |  |  |  |  |  |  |
| House/Family | 1 | House |  |  |  |  |  |  |  |  |  |
| Container/House | 1 | Container |  |  |  |  |  |  |  |  |  |
| Amount of waste | Population 2013 |  | Ton/year | m3/year | Generated volume | Houses | m3/week-house | Containers |  |  |  |
|  | Urban | Rural |  |  | m3/Week |  |  |  |  |  |  |
| Z-1 | 32.649 | 9.219* | 18.546 | 52.989 | 1.016 | 13.956 | 0,5097 | 13.956 |  |  |  |
| Z-2 | 0 | 26.981 | 6.957 | 19.876 | 381 | 8.994 | 0,2967 | 8.994 |  |  |  |
| Z-8 | 10.604 | 57.945 | 22.053 | 63.008 | 1.208 | 22.850 | 0,3702 | 22.850 |  |  |  |
| Investment Costs |  | Qty | unit | Unit Costs (Euro) |  | Total (Euro) |  | Total (Euro) | O&M | Amortization | Running Costs |
|  |  |  |  | Civil Works | Equipment | Civil Works | Equipment |  | (Euro) |  | (Euro) |
| Z-1 | Eurobins | 13.956 | unit |  | 22 |  | 306.386 | 306.386 | 15.319 | 43.769 | 59.089 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
| Total Z1 |  |  |  |  |  | 0 | 306.386 | 306.386 | 15.319 | 43.769 | 59.089 |
| Z-2 | Eurobins | 8.994 | unit |  | 22 |  | 197.444 | 197.444 | 9.872 | 28.206 | 38.078 |
| Total Z2 |  |  |  |  |  | 0 | 197444 | 197.444 | 9.872 | 28.206 | 38.078 |
| Z-8 | Eurobins | 22.850 | unit |  | 22 |  | 501.631 | 501.631 | 25.082 | 71.662 | 96.743 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 |
| Total Z8 |  |  |  |  |  | 0 | 501.631 | 501.631 | 25.082 | 71.662 | 96.743 |
| Total investments for Option-1 (Tempory Storage)= |  |  |  |  |  | 0 | 1.005.460 | 1.005.460 | 50.273 | 143.637 | 193.910 |

*Population without PHARE Projects

  - Externalities

Externalities are known as those conditions whose benefits (called external economies) and costs (called external diseconomies) are not reflected in the market price of goods and services. The primary feature of externalities is that one entity's action directly or indirectly changes the options available to other entities. For example, widespread benefits of an option (such as resource saving) accompanied by the widespread hazards of pollution (such as destruction of ozone layer). Externalities are an important consideration in this option analysis.

The following table presents the externalities associated with the Domiciliary (Mixed Storage) Option in the Phase-II zones of Arges County.

Table 8.3-1 Externalities associated with the Domiciliary Mixed Storage Option

| Field | Implications |  | Domiciliary Storage (Mixed) | Discretional1 |
| --- | --- | --- | --- | --- |
| Environmental Impacts |  | Avoiding health hazards | + solid waste stored in specialized containers + Correct use ensures clean conditions | 1 |
|  |  | Emissions | + Hermetic bins, no emissions | 1 |
|  |  | Vectors | + Proper collection, no vectors | 1 |
| Resource Saving |  | Number of bins/containers | + Bin at household | 1 |
|  |  | Collection services | - Mixedated waste collection every week | 0 |
|  |  | Fuel Consumption | - Less speed (20 km/h) = high fuel consumption is demanded due to domiciliary collection, | 0 |
|  |  | Required space | - It is required available space in private households | 0,5 |
| Recycling Targets |  | Separated storage | - Targets difficult to reach as mixed storage | 0 |
|  |  | Biodegradable fraction | - No special bin | 0 |
|  |  | Recyclable fraction | - No special bins (space required) | 0 |
| Society |  | Acceptability | + High | 1 |
|  |  | Environmental awareness | - Very low as no separation at source done | 0 |
| Operational |  | Road access | - Poor accessibility in all households of Zones | 0 |
|  |  | Applicability | + Feasible | 1 |
| Flexibility /Adaptability |  | Methodology | + Feasible to be adapted to a separate storage system | 1 |
|  |  | Capacity | - not able to increase capacity | 0 |
| Sum |  |  |  | 7,5 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

#### Collection Platforms (bring-system)

| The ‘bring-system’ for waste tempory storage refers to a system whereby the waste generator and/or householder is required to take their waste to one communal storage point. At this point the total waste amount is separated into the main streams, in order to recover certain materials which can be introduced to recycling processes. As these communal collection points serve completely inhabited areas, their capacity has to enable the storage of the total amount of solid waste in that area. Therefore, usually containers are used more than bins, since containers have more volume and take up less space. The collection is carried out periodically mechanically and/or manually. | Figure 8.3-2 Bring-System through Collection platforms |
| --- | --- |

- Investment Costs and O&M Costs

| Calculation of Containers |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Type |  | Bring system |  |  |  |  |  |  |  |  |
|  |  | Storage type: |  | Separated |  |  |  |  |  |  |  |  |
|  |  | Container: |  | 1,1 | m3 |  |  |  |  |  |  |  |
|  |  | Density |  | 0,35 | ton/m3 |  |  |  |  |  |  |  |
|  |  | Container/plattform |  | 5 | Persons |  |  |  |  |  |  |  |
|  |  | Coverage |  | 100 | Inh/container |  |  |  |  |  |  |  |
|  |  | Container/plattform |  | 5 | Containers (Recyclables, Residual & Organic ) |  |  |  |  |  |  |  |
| Amount of waste | Population 2013 |  | Generated |  | m3/year | Generated volume | Containers of 1.1 m3 |  | Platforms | Platforms | Platforms |  |
|  | Urban | Rural | Ton/year |  |  | m3/Week | (unit) |  | (unit) | A | B |  |
| Z-1 | 32.649 | 9.219 | 18.546 |  | 52.989 | 1.016 | 597 |  | 180 | 87 | 93 | 180 |
| Z-2 | 0 | 26.981 | 6.957 |  | 19.876 | 381 | 368 |  | 184 | 0 | 184 | 184 |
| Z-8 | 10.604 | 57.945 | 22.053 |  | 63.008 | 1.208 | 937 |  | 431 | 25 | 406 | 431 |
| Investment Costs |  | Qty | unit |  | Unit Costs (Euro) |  | Total (Euro) |  | Total (Euro) | O&M | Amortization | Running Costs |
|  |  |  |  |  | Civil Works | Equipment | Civil Works | Equipment |  | (Euro) |  | (Euro) |
| Z-1 | Containers | 597 | unit |  |  | 252 |  | 150.698 | 150.698 | 7.535 | 21.528 | 29.063 |
|  | Eurobins | 0 | unit |  |  | 22 |  | 0 | 0 | 0 | 0 | 0 |
|  | Platform A | 87 | unit |  | 2.476 |  | 215.408 | 0 | 215.408 | 0 | 8.616 | 8.616 |
|  | Platform B | 93 | unit |  | 1.970 |  | 183.205 | 0 | 183.205 | 0 | 7.328 | 7.328 |
| Total Z1 |  |  |  |  |  |  | 398.613 | 150.698 | 549.311 | 7.535 | 37.473 | 45.008 |
|  | Containers | 368 | unit |  |  | 252 |  | 92.892 | 92.892 | 4.645 | 13.270 | 17.915 |
|  | Eurobins | 0 | unit |  |  | 22 |  | 0 | 0 | 0 | 0 | 0 |
|  | Platform A | 0 | unit |  | 2.476 |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Z-2 | Platform B | 184 | unit |  | 1.970 |  | 362.471 | 0 | 362.471 | 0 | 14.499 | 14.499 |
| Total Z2 |  |  |  |  |  |  | 362.471 | 92.892 | 455.363 | 4.645 | 27.769 | 32.414 |
| Z-8 | Containers | 937 | unit |  |  | 252 |  | 236.522 | 236.522 | 11.826 | 33.789 | 45.615 |
|  | Eurobins | 0 | unit |  |  | 22 |  | 0 | 0 | 0 | 0 | 0 |
|  | Platform A | 25 | unit |  | 2.476 |  | 61.899 | 0 | 61.899 | 0 | 2.476 | 2.476 |
|  | Platform B | 406 | unit |  | 1.970 |  | 799.800 | 0 | 799.800 | 0 | 31.992 | 31.992 |
| Total Z8 |  |  |  |  |  |  | 799.800 | 236.522 | 1.036.322 | 11.826 | 65.781 | 77.607 |
| Total investments for Option-2 (Tempory Storage)= |  |  |  |  |  |  | 1.560.884 | 480.112 | 2.040.996 | 24.006 | 131.023 | 155.028 |

*Population without PHARE Projects

- Externalities

The following table presents the externalities associated with the Bring System Option in the Phase-II zones of Arges County.

Table 8.3-2 Externalities associated with the Bring-System Option

| Field | Implications |  | Bring System | Discretional1 |
| --- | --- | --- | --- | --- |
| Environmental Impacts |  | Avoiding health hazards | + solid waste stored in specialized containers + Correct use ensures clean conditions | 1 |
|  |  | Emissions | + Hermetic bins, no emissions | 1 |
|  |  | Vectors | + Proper collection, no vectors | 1 |
| Resource Saving |  | Number of bins/containers | - 4 Bins and/or containers in every platform | 0 |
|  |  | Collection services | + Separated waste collection every week | 1 |
|  |  | Fuel Consumption | + More speed (40 Km/h) = less fuel consumption is demanded due to collection to platforms, | 1 |
|  |  | Required space | + More space available in public areas | 1 |
| Recycling Targets |  | Separated storage | + Targets reached in separate storage | 1 |
|  |  | Biodegradable fraction | + Special container | 1 |
|  |  | Recyclable fraction | + Containers for glass, plastic & paper | 1 |
| Society |  | Acceptability | + High | 1 |
|  |  | Environmental awareness | + High as separation at source is easier, due to space requirements | 1 |
| Operational |  | Road access | + Good to public areas | 1 |
|  |  | Applicability | + Feasible | 1 |
| Flexibility /Adaptability |  | Methodology | - Restricted as platforms are closed | 0 |
|  |  | Capacity | + High, as low used containers can storage different waste streams | 1 |
| Sum |  |  |  | 14 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

#### Domiciliary (Separate Storage )

| The domiciliary waste storage consists of the procurement of specialized bins and/or containers for households in urban and rural areas of Arges County. The stored waste from the bins and/or containers, are typically associated with door-to-door collection by compactor trucks. This procedure can be performed mechanically and/or manually. As one of the main targets of tempory storage is to avoid hazards to health, and since the storage period of waste is limited, a collection frequency of twice per week is required with domiciliary storage alternative. The number of bins for this option is thought to be 3. This number will ensure separate waste storage at source. The waste streams to be separated are: | Figure 8.3-3 Separate waste storage - 1 bin for residual waste - 1 bin for plastic & metal - 1 bin for paper & cardboard |
| --- | --- |

  - Investment Costs and O&M Costs

| Calculation of Investments |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Type | Domiciliary |  |  |  |  |  |  |  |  |  |
|  | Storage type: | Separated |  |  |  |  |  |  |  |  |  |
|  | Container: | 0,12 | m3 |  |  |  |  |  |  |  |  |
|  | Density | 0,35 | ton/m3 |  |  |  |  |  |  |  |  |
|  | Members /Family | 3 | Persons |  |  |  |  |  |  |  |  |
|  | House/Family | 1 | House |  |  |  |  |  |  |  |  |
|  | Container/House | 3 | Containers (Recyclables, Residual, Organic ) |  |  |  |  |  |  |  |  |
| Amount of waste | Population 2013 |  | Ton/year | m3/year | Generated volume | Houses | Eurobins of 0.120 m3 |  |  |  |  |
|  | Urban | Rural |  |  | m3/Week | (unit) | (unit) |  |  |  |  |
| Z-1 | 32.649 | 9.219 | 18.546 | 52.989 | 1.016 | 13.956 | 41.868 |  |  |  |  |
| Z-2 | 0 | 26.981 | 6.957 | 19.876 | 381 | 8.994 | 26.981 |  |  |  |  |
| Z-8 | 10.604 | 57.945 | 22.053 | 63.008 | 1.208 | 22.850 | 68.549 |  |  |  |  |
| Investment Costs |  | Qty | unit | Unit Costs (Euro) |  | Total (Euro) |  | Total (Euro) | O&M | Amortization | Running Costs |
|  |  |  |  | Civil Works | Equipment | Civil Works | Equipment |  | (Euro) |  | (Euro) |
| Z-1 |  |  |  |  |  |  |  |  |  |  |  |
|  | Eurobins | 41.868 | unit |  | 22 |  | 919.158 | 919.158 | 45.958 | 131.308 | 177.266 |
| Total Z1 |  |  |  |  |  | 0 | 919.158 | 919.158 | 45.958 | 131.308 | 177.266 |
|  | Eurobins | 26.981 |  |  | 22 |  | 592.332 | 592.332 | 29.617 | 84.619 | 114.235 |
| Z-2 |  |  |  |  |  |  |  |  |  |  |  |
| Total Z2 |  |  |  |  |  | 0 | 592.332 | 592.332 | 29.617 | 84.619 | 114.235 |
| Z-8 |  |  |  |  |  |  |  |  |  |  |  |
|  | Eurobins | 68.549 | unit |  | 22 |  | 1.504.892 | 1.504.892 | 75.245 | 214.985 | 290.229 |
| Total Z8 |  |  |  |  |  | 0 | 1.504.892 | 1.504.892 | 75.245 | 214.985 | 290.229 |
| Total investments for Option-3 (Tempory Storage)= |  |  |  |  |  | 0 | 3.016.381 | 3.016.381 | 150.819 | 430.912 | 581.731 |

*Population without PHARE Projects

  - Externalities

The following table presents the externalities associated with the Domiciliary Storage Option in the Phase-II zones of Arges County.

Table 8.3-3 Externalities associated with the Domiciliary Storage Option

| Field | Implications |  | Domiciliary Storage (separated) | Discretional1 |
| --- | --- | --- | --- | --- |
| Environmental Impacts |  | Avoiding health hazards | + solid waste in stored in specialized containers + Correct use ensures clean conditions | 1 |
|  |  | Emissions | + Hermetic bins, no emissions | 1 |
|  |  | Vectors | + Proper collection, no vectors | 1 |
| Resource Saving |  | Number of bins/containers | - 4 Bins and/or containers in every household | 0 |
|  |  | Collection services | - Separated waste collection every week | 0 |
|  |  | Fuel Consumption | - Less speed (20 km/h) = high fuel consumption is demanded due to domiciliary collection, | 0 |
|  |  | Required space | - It is required available space in private households | 0,5 |
| Recycling Targets |  | Separated storage | + Targets reached in separate storage | 1 |
|  |  | Biodegradable fraction | - No special bin | 0 |
|  |  | Recyclable fraction | - Special bins (space required) | 0,5 |
| Society |  | Acceptability | + High | 1 |
|  |  | Environmental awareness | + High as separation at source is easier, due to space requirements | 1 |
| Operational |  | Road access | - Poor accessibility in all households of Zones | 0 |
|  |  | Applicability | + Feasible | 1 |
| Flexibility /Adaptability |  | Methodology | + High flexibility for waste stream storage (mixed-separate) | 1 |
|  |  | Capacity | - Not able to increase the capacity | 0 |
| Sum |  |  |  | 9 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

#### Haul System (Mixed Storage)

| The tempory storage of solid waste involves positioning containers of more than 4m3 to store the solid waste that is generated in a widely inhabited zone. Subsequently, the waste generators bring their solid waste and deposit it in those containers. Normally, there is no separation of waste streams, however if additional containers are placed alongside then this would also be possible. The containers are hauled away by specialized trucks at frequent intervals. The solid waste is transported to the next stage of the waste management system, which could be a transfer station or directly to the final disposal. | Figure 8.3-4 Hauling-System for tempory storage of solid waste |
| --- | --- |

- Investment Costs and O&M Costs

| Calculation of investments |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Type | Hauling system |  |  |  |  |  |  |  |  |  |
|  | Storage type: | Mixed |  |  |  |  |  |  |  |  |  |
|  | Container: | 4 | m3 |  |  |  |  |  |  |  |  |
|  | Density | 0,35 | ton/m3 |  |  |  |  |  |  |  |  |
| Amount of waste | Population 2013 |  | Ton/year | m3/year | Generated volume | Containers of 4 m3 |  |  |  |  |  |
|  | Urban | Rural |  |  | m3/Week | (unit) |  |  |  |  |  |
| Z-1 | 32.649 | 9.219* | 18.546 | 52.989 | 1.016 | 254 |  |  |  |  |  |
| Z-2 | 0 | 26.981 | 6.957 | 19.876 | 381 | 95 |  |  |  |  |  |
| Z-8 | 10.604 | 57.945 | 22.053 | 63.008 | 1.208 | 302 |  |  |  |  |  |
| Investment Costs |  | Qty | unit | Unit Costs (Euro) |  | Total (Euro) |  | Total (Euro) | O&M | Amortization | Running Costs |
|  |  |  |  | Civil Works | Equipment | Civil Works | Equipment |  | (Euro) |  | (Euro) |
| Z-1 | Containers | 254 | unit |  | 805 |  | 204.517 | 204.517 | 10.226 | 29.217 | 39.443 |
| Total Z1 |  |  |  |  |  | 0 | 204.517 | 204.517 | 10.226 | 29.217 | 39.443 |
|  | Containers | 95 |  |  | 805 |  | 76.715 | 76.715 | 3.836 | 10.959 | 14.795 |
| Z-2 |  |  |  |  |  |  |  |  |  |  |  |
| Total Z2 |  |  |  |  |  | 0 | 76.715 | 76.715 | 3.836 | 10.959 | 14.795 |
| Z-8 | Containers | 302 | unit |  | 805 |  | 243.185 | 243.185 | 12.159 | 34.741 | 46.900 |
| Total Z8 |  |  |  |  |  | 0 | 243.185 | 243.185 | 12.159 | 34.741 | 46.900 |
| Total investments for Option-4 (Tempory Storage)= |  |  |  |  |  | 0 | 524.417 | 524.417 | 26.221 | 74.917 | 101.138 |

*Population without PHARE Projects

b) Externalities

The following table presents the externalities associated with the Haul System (Mixed Storage) Option in the Phase-II zones of Arges County.

Table 8.3-4 Externalities associated with the Haul System Domiciliary Mixed Storage Option

| Field | Implications |  | Haul System | Discretional1 |
| --- | --- | --- | --- | --- |
| Environmental Impacts |  | Avoiding health hazards | + solid waste stored in specialized containers + Correct use ensures clean conditions | 1 |
|  |  | Emissions | - No hermetic bins, no emissions | 0 |
|  |  | Vectors | + Proper collection, no vectors | 1 |
| Resource Saving |  | Number of bins/containers | + 1 Container in every collection point | 1 |
|  |  | Collection services | - Mixed waste collection every week | 0 |
|  |  | Fuel Consumption | + More speed (40 Km/h) = less fuel consumption is demanded due to collection to points | 1 |
|  |  | Required space | + More space available in public areas | 1 |
| Recycling Targets |  | Separated storage | - Difficult to reach targets as no separate storage | 0 |
|  |  | Biodegradable fraction | - No special container | 0 |
|  |  | Recyclable fraction | - No containers for glass, plastic & paper | 0 |
| Society |  | Acceptability | - Low | 0 |
|  |  | Environmental awareness | - Very low as no separation at source | 0 |
| Operational |  | Road access | + Good to public areas | 1 |
|  |  | Applicability | - Not very feasible | 0 |
| Flexibility /Adaptability |  | Methodology | - Not able to change the methodology (mixed to separate) + High mobility as can be placed differently | 1 |
|  |  | Capacity | - Not able to increase capacity | 0 |
| Sum |  |  |  | 7 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

#### Mixed System (Domiciliary + Communal System)

| Mixed System is the term used to refer to tempory storage through a combination of systems; the systems are basically domiciliary and communal systems. This responds to the needs for domiciliary storage in those areas, where communal systems cannot be implemented, as they are mountainous or difficult to access. The communal system can include the implementation of collection platforms and/or groups of containers distributed along the main streets of the urban and rural areas. The users of the system deposit their waste in the containers that will be emptied by collector trucks regularly. There are 2 possible arrangements: - Mixed waste storage. - Separate waste storage. According to the needs of the current areas and the appointed targets, the second option is going to be considered for this analysis | Figure 8.3-5 Bring system with containers on the street |
| --- | --- |

- Investment Costs and O&M Costs

| Investment Calculation |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Type | Bring system |  |  | Criteria for System Implementation |  |  |  |  |  |  |  |
|  | Storage type: | Separated |  |  | Zone | Domiciliary | Platforms | No Platforms | Sum |  |  |  |
|  | Container: | 1,1 | m3 |  | Z11 | 78% | 15% | 7% | 100% |  |  |  |
|  | Bins | 0,12 | L |  | Z22 | 26% | 50% | 24% | 100% |  |  |  |
|  | Density | 0,35 | ton/m3 |  | Z83 | 23% | 20% | 57% | 100% |  |  |  |
|  | Households | 3 | Inh/house |  |  |  |  |  |  |  |  |  |
|  | Coverage | 100 | Inh/container |  |  |  |  |  |  |  |  |  |
|  | HH Bins | 3 | Bins (Reciclables, Residual, Urban organic ) |  |  |  |  |  |  |  |  |  |
| Amount of waste |  |  | Ton/year | m3/year | Generated volume | Domiciliary | Volume for Communal |  | Volume for Domiciliary |  |  |  |
|  | Population 2013 |  |  |  |  |  | Platforms | No Platforms |  | Platforms | No Platfroms |  |
|  | Urban | Rural |  |  | m3/Week | m3/Week | m3/Week | m3/Week | Bins | Units | Units | Containers |
| Z-1 | 32.649 | 9.219 | 18.546 | 52.989 | 1.016 | 813 | 152 | 51 | 32.764 | 27 | 90 | 109 |
| Z-2 | 0 | 26.981 | 6.957 | 19.876 | 381 | 114 | 191 | 76 | 6.960 | 92 | 184 | 197 |
| Z-8 | 10.604 | 57.945 | 22.053 | 63.008 | 1.208 | 363 | 242 | 604 | 15.669 | 86 | 187 | 1004 |
| Investment Costs |  | Qty | unit | Unit Costs (Euro) |  | Total (Euro) |  | Total (Euro) | O&M | Amortization | Running Costs |  |
|  |  |  |  | Civil Works | Equipment | Civil Works | Equipment |  | (Euro) |  | (Euro) |  |
| Z-1 | Containers | 199 | unit |  | 249.5 |  | 49.646 | 32.595 | 4.521 | 4.306 | 8.827 |  |
|  | Eurobins | 32.764 | unit |  | 21.4 |  | 699.958 | 793.454 | 110.299 | 105.047 | 215.346 |  |
|  | Platforms | 27 | unit | 3.045 |  | 82.215 | 0 | 82.215 | 0 | 3.289 | 3.289 |  |
| Total Z1 |  |  |  |  |  | 82.215 | 749.905 | 908.264 | 114.820 | 112.641 | 227.461 |  |
|  | Containers | 380 |  |  | 249.5 |  | 94,802 | 104.084 | 9.754 | 9.289 | 19.043 |  |
|  | Eurobins | 6.960 |  |  | 21.4 |  | 148,691 | 163.560 | 26.655 | 25.386 | 52.041 |  |
| Z-2 | Platforms | 92 | unit | 3.045 |  | 280.140 | 0 | 280.140 | 0 | 11.206 | 11.206 |  |
| Total Z2 |  |  |  |  |  | 280.140 | 243.493 | 547.784 | 36.409 | 45.880 | 82.289 |  |
| Z-8 | Containers | 1192 | unit |  | 249.5 |  | 297,379 | 326.494 | 24.835 | 23.652 | 48.487 |  |
|  | Eurobins | 15.669 | unit |  | 21.4 |  | 334,747 | 368.222 | 67.720 | 64.495 | 132.215 |  |
|  | Platforms | 86 | unit | 3.045 |  | 262.479 | 0 | 262.479 | 0 | 10.499 | 10.499 |  |
| Total Z8 |  |  |  |  |  | 262.479 | 632.196 | 957.195 | 92.555 | 98.647 | 191.202 |  |
| Total investments for Option-5 (Tempory Storage)= |  |  |  |  |  | 624.834 | 1.625.223 | . 2.413. 243 | 243.783 | 257.168 | 500.952 |  |

*Population without PHARE Projects

1 The current system around Curtea de Arges City has already domiciliary

2 The mountain terrain of Domensti make necesary the platforms to hold containers

3 The terrain is flat, thus not big number of platforms are foressen

b) Externalities

The following table presents the externalities associated with the Mixed System (Domiciliary + Communal System) in the Phase-II zones of Arges County.

Table 8.3-5 Externalities associated with the Mixed System (Domiciliary + Communal System)

| Field | Implications |  | Bring System | Discretional1 |
| --- | --- | --- | --- | --- |
| Environmental Impacts |  | Avoiding health risks | + solid waste stored in specialized containers + Correct use ensures clean conditions | 1 |
|  |  | Emissions | + Hermetic bins, no emissions | 1 |
|  |  | Vectors | + Proper collection, no vectors | 1 |
| Resource Saving |  | Number of bins/containers | - 3 Bins and/or containers in every site | 0 |
|  |  | Collection services | + Separated waste collection every week | 1 |
|  |  | Fuel Consumption | + More speed (20-40 Km/h) = less fuel consumption is demanded due to specific sites for collection, | 1 |
|  |  | Required space | + less required space as less platforms | 1 |
| Recycling Targets |  | Separated storage | + Targets reached in separate storage | 1 |
|  |  | Biodegradable fraction | + Special container | 1 |
|  |  | Recyclable fraction | + Containers for recyclables | 1 |
| Society |  | Acceptability | + High | 1 |
|  |  | Environmental awareness | + Moderate as separation at source is only in two main streams | 1 |
| Operational |  | Road access | + Good as main sites will be streets | 1 |
|  |  | Applicability | - Feasible but the containers are more exposed to vandalism, car-crashes & theft, as they are not housed | 0,5 |
| Flexibility /Adaptability |  | Methodology | + High as sites can be changed as required | 1 |
|  |  | Capacity | + High, as containers can be exchanged as required | 1 |
| Sum |  |  |  | 14,5 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

#### Evaluation of Options for Waste Tempory Storage

| Criteria |  | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Domiciliary (Mixed Storage) | Bring System (Separate Storage) | Domiciliary (Separate Storage) | Haul System | Mixed System |
| Investments |  | 1.005.460 | 2.040.996 | 3.016.381 | 524.417 | 2.413.243 |
| Running Costs |  | 193.910 | 155.028 | 581.731 | 101.138 | 500.952 |
| Externalities |  |  |  |  |  |  |
| Environmental Impacts | Avoiding health risks | 1 | 1 | 1 | 1 | 1 |
|  | Emissions | 1 | 1 | 1 | 0 | 1 |
|  | Vectors | 1 | 1 | 1 | 1 | 1 |
| Resource Saving | Number of bins/containers | 1 | 0 | 0 | 1 | 0 |
|  | Collection services | 0 | 1 | 0 | 0 | 1 |
|  | Fuel Consumption | 0 | 1 | 0 | 1 | 1 |
|  | Required space | 0,5 | 1 | 0,5 | 1 | 1 |
| Recycling Targets | Separated storage | 0 | 1 | 1 | 0 | 1 |
|  | Biodegradable fraction | 0 | 1 | 0 | 0 | 1 |
|  | Recyclable fraction | 0 | 1 | 0,5 | 0 | 1 |
| Society | Acceptability | 1 | 1 | 1 | 0 | 1 |
|  | Environmental awareness | 0 | 1 | 1 | 0 | 1 |
| Operational | Road access | 0 | 1 | 0 | 1 | 1 |
|  | Applicability | 1 | 1 | 1 | 0 | 0,5 |
| Flexibility /Adaptability | Methodology | 1 | 0 | 1 | 1 | 1 |
|  | Capacity | 0 | 1 | 0 | 0 | 1 |
|  |  | 7,5 | 14 | 9 | 7 | 14,5 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

The table above shows the main comparison between 5 different tempory storage options for the Phase-II zones of Arges County. The Options A & D represent lower investments; however, their external benefits are the lowest. On the other hand, Option E presents the third lowest investment cost and its external benefits are at the highest level, compared with the rest of Alternatives.

It is necessary to point out that lower investments may represent higher investments in further stages of WM, in order to reach the targets. However, this issue cannot be assessed for each individual stage of WM, but it has to be assessed as a part of a WM System (see Section 8.3).

The preferred Options for tempory storage are B & E, which represent higher external benefits. The economic benefits will be assessed in Section 8.3.

### Waste Collection

The objectives of the RWMP for waste collection in the Regional Waste Management Plan are as follows:

- Ensure collection capacities are adapted to the number of inhabitants and to the quantity of generated waste.

  - Extend the waste collection areas to: rural: 95% by 2010, and urban: 100% by 2013.

- Ensure the best options for the waste collection and transport, correlated with recycling and final disposal activities.

  - Separate collection of hazardous household waste.

  - Implement selective collection of recoverable materials so that statutory objectives for packaging and biodegradable waste are achieved.

In order to reach the above-mentioned objectives, the options for tempory storage are as follows.

#### Door to Door Waste Collection (Mixed)

| This system is closely linked with the domiciliary tempory storage of waste. The Door-to-Door waste collection System is labour, fuel, and vehicle intensive, and needs to be repeated daily. Therefore this solid waste collection system normally consumes a significant portion of city revenues. Commonly, in developing countries where people do not cooperate with waste collection systems, and where traffic and road access slows the efficiency of the workers and vehicles, costs tend to be high in relation to income, compared to collection costs in developed countries. Door-to-door systems, even if by pushcart, are more costly, but may be preferred and might lead to greater willingness to pay cost recovery charges. | Figure 8.3-6 Door-to-Door Collection System |
| --- | --- |

a) Investment Costs

The main technical features of the collection trucks for urban and rural areas in Phase-II of Arges County, are presented below:

| URBAN AREAS |  |  |  | RURAL AREAS |  |  |  | HHW Mobile Unit |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Capacity: | 24 | m3 |  | Capacity: | 16 | m3 |  | Capacity: | 4 | m3 |
| Gross weight: | 22,5 | t |  | Gross weight: | 19,5 | t |  |  |  |  |
| Axle: | 3 |  |  | Axle: | 2 |  |  | Axle: | 2 |  |
| Payload: | 12 | t |  | Payload: | 8 | t |  | Payload: | 0,5 | t |
| Efficiency per load: | 90% |  |  | Efficiency per load: | 90% |  |  | Efficiency per load: | 90% |  |
| Average speed | 20 | km/h |  | Average speed | 20 | km/h |  | Average speed | 20 | km/h |
| Shift: | 1 |  |  | Shift: | 1 |  |  | Shift: | 1 |  |
| Working days/year | 300 | days |  | Working days/year | 300 | days |  | Working days/year | 300 | days |
| Availability: | 95% |  |  | Availability: | 95% |  |  | Availability: | 95% |  |
| Collected tons/year: | 3.078 | t/year |  | Collected tons/year: | 2.052 | t/year |  | Collected tons/year: | 1.283 | t/year |
| Collected tons/week: | 71,82 | t/week |  | Collected tons/week: | 47,88 | t/week |  | Collected tons/week: | 29,925 | t/week |

The calculation of the number of collection trucks needed for Door-to-Door collection services is presented as follows.

| Zone 1- Curtea de Arges |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Trip | 310 | Km |  |  |  |  |
| Coverage | 16 | Hr/trip |  |  |  |  |
| Efficiency | 0,5 | Trips/truck |  |  |  |  |
| Population 2013 |  |  | Waste generated (ton/week) | HHW (ton/week) | Residual Waste Trucks | HHW Collection trucks |
| Total | 74.925 |  |  |  |  |  |
| Urban | 32.649 |  | 306 | 2 | 10 | 1 |
| Rural | 42.276 |  | 103 | 1 | 7 | 1 |
| Sum |  |  |  |  | 17 | 2 |
| Zone 2-Domnesti |  |  |  |  |  |  |
| Trip | 202 | Km |  |  |  |  |
| Coverage | 11 | Hr/trip |  |  |  |  |
| Efficiency | 0,73 | Trips/truck |  |  |  |  |
| Population 2013 |  |  | Waste generated (ton/week) | HHW (ton/week) | Residual Waste | HHW Collection truck |
| Total | 26.981 |  |  |  |  |  |
| Urban | 0 |  | 0 | 0 | 0 | 0 |
| Rural | 26.981 |  | 62 | 1 | 5 | 0 |
| Sum |  |  |  |  | 5 | 0 |
| Zone 8-Costesti |  |  |  |  |  |  |
| Trip | 680 | Km |  |  |  |  |
| Coverage | 34 | Hr/trip |  |  |  |  |
| Efficiency | 0,24 | Trips/truck |  |  |  |  |
| Population 2013 |  |  | Waste generated (ton/week) | HHW (ton/week) | Residual Waste | HHW Collection truck |
| Total | 68.549 |  |  |  |  |  |
| Urban | 10.604 |  | 21 | 1 | 5 | 1 |
| Rural | 57.945 |  | 127 | 2 | 14 | 0 |
| Sum |  |  |  |  | 19 | 1 |
| TOTAL |  |  |  |  | 41 | 2 |

The investments in Phase-II do not take into account the purchase of Collection trucks, only those for HHW will be included as investments in the project. It is foreseen that trucks are brought by operators of the waste collection zones.

The calculation of Investment Costs for this Option is presented in the next table.

| Investment Costs |  | Qty |  | Unit Costs (Euro) |  | Total (Euro) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Civil Works | Equipment | Civil Works | Equipment |
| Z-1 | HHW Vehicle | 2 | unit |  | 35.000 |  | 70.000 |
| Total Z1 |  |  |  |  |  | 0 | 70.000 |
| Z-2 |  |  |  |  |  |  |  |
| Total Z2 |  |  |  |  |  | 0 | 0 |
| Z-8 | HHW Vehicle | 1 | unit |  | 35000 |  | 35.000 |
| Total Z8 |  |  |  |  |  | 0 | 35.000 |
| Total investments for Collection (Mixed)= |  |  |  |  |  | 0 | 105.000 |

Here, the investments are the same as in all the options for waste collection services, as there are only HHW collection trucks. No trucks need to be purchased.

The main objective to calculate the number is to know the number of trucks and obtain the O&M Costs.

b) O&M Costs

The calculation of O&M costs are presented in the next table.

| Manpower for 1 shift |  |  |  |
| --- | --- | --- | --- |
| Brut salary | Euro/Month | months/year | Euro/year |
| Driver | 350 | 12 | 4.200 |
| Crew (2) | 305 | 24 | 7.320 |
|  |  |  | 11.520 |
| cost coefficient |  |  |  |
| Gross salary |  |  |  |
| Pension insurance, paid by enterprise |  | 10% |  |
| cost coefficient |  |  | 1,10 |
| working days a year | 52 | weeks/a | 260 |
| for unemployability (illness and disabled persons) | 12% |  | -31,2 |
| working days for vacation | 18 | d/year | -18 |
| legal holidays | 10 | d/year | -10 |
| real working days per worker and year |  |  | 200,8 |
| manpower coefficient |  |  | 1,29482 |
| Total cost coefficient |  |  | 1,42430 |
| Salary cost/shift |  |  | 16.407,97 |
| Number of shift/truck |  |  | 1 |
| Number of working day/truck |  |  | 250 |
| Working days a year |  |  | 260 |
| Time coefficient |  |  | 0,96154 |
| Salary cost per truck and year |  |  | 15.776,89 |
| Fuel and others |  |  |  |
| Number of shift/truck |  |  | 1 |
| Number of working day/truck |  |  | 250 |
| working hours/truck |  |  | 2.000 |
| fuel contribution l/h, €/l | 6 | 1,1 | 13.200 |
| tyre cost 1.1m³ system (collecting truck) per year |  |  | 478 |
| maintenance / insurance (as % of investment/year) | 7,5% | 135.000 | 10.125 |
| maintenance / insurance (as % of investment/year) | 7,5% | 110.000 | 8.250 |
| vehicle care cost |  |  | 120 |
| vehicle accommodation cost |  |  | 120 |
| other working materials/lubricants etc. (as % of fuel) | 0 |  | 660 |
| Total for 24 m³ truck per year |  |  | 24.703 |
| Total for 16 m³ truck per year |  |  | 22.828 |
| Total O&M Costs per Truck and Year |  |  |  |
|  |  |  | 1 |
|  |  |  | 250 |
| 24 m³ truck (without administration) |  |  | 40.480 |
| administration (as % of total O&M costs) | 7,5% |  | 3.036 |
| Total O&M + administration for 24 m³ truck |  |  | 43.516 |
| 16 m³ truck (without administration) |  |  | 38.605 |
| administration (as % of total O&M costs) | 7,5% |  | 2.895 |
| Total O&M + administration for 16 m³ truck |  |  | 41.500 |

| Residue Collection with 1,5 shifts/day and 6 days/week (300 days/year): |  |  |  |  |  | O&M costs: |  | capital costs: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Urban Areas Residue Collection: | 15 | á | 24 m³ | collection trucks |  | 679.797 | €/year | 236.066 | €/year |
|  | 0 | á | 16m³ | collection trucks |  |  | €/year | 0 | €/year |
| Rural Areas Residue Collection: | 26 | á | 16 m³ | collection trucks |  | 1.172.578 | €/year | 402.065 | €/year |
| Total Residue Collection: | 41 |  |  |  |  | 1.852.376 | €/year | 638.132 | €/year |
| HHW |  |  |  |  |  |  |  |  |  |
| URBAN |  |  |  |  |  | O&M costs: |  | capital costs: |  |
| HHW | 2 | á | Mobil Unit | Truck |  | 90.198 | €/year | 31.476 | €/year |
| RURAL |  |  |  |  |  | O&M costs: |  | capital costs: |  |
| HHW | 1 | á | Mobil Unit | Truck |  | 45.099 | €/year | 15.738 | €/year |
| Total= | 3 |  |  | collection trucks |  | 135.297 | €/year | 47.213 | €/year |
|  |  |  |  | Sum |  | 2.122.971 | €/year | 732.558 | €/year |

c) Externalities

The following table presents the externalities associated with the Door-to-Door Waste Collection System (Mixed ) Option in the Phase-II zones of Arges County.

Table 8.3-6 Externalities associated with the Door-to-Door Waste Collection System (Mixed ) Option

| Field | Implications |  | Waste Collection (Mixed) | Discretional1 |
| --- | --- | --- | --- | --- |
| Environmental Impacts |  | Avoiding health hazards | + solid waste collected in proper way + Correct use ensures clean conditions | 1 |
|  |  | Emissions | + 709 tons CO2/year | 1 |
|  |  | Noise | + Less trucks=less noise | 1 |
| Resource Saving |  | Number of trucks | + 43 collection trucks | 1 |
|  |  | Collection services | - Mixed waste collection every week | 0 |
|  |  | Fuel Consumption | - Less speed (20 Km/h) = More fuel consumption is demanded due to door-to-door collection | 0 |
| Recycling Targets |  | Separated storage | - No | 0 |
|  |  | Household Hazardous Waste | - Special truck required | 0,5 |
|  |  | Biodegradable fraction | - No | 0 |
|  |  | Recyclable fraction | - No | 0 |
| Society |  | Acceptability | + High as door-to-door service | 1 |
|  |  | Environmental awareness | - Very low as no separation at source | 0 |
| Operational |  | Road traffic | + Good as lower number of trucks | 1 |
|  |  | Applicability | - Feasible | 1 |
| Flexibility /Adaptability |  | Methodology | + Feasible | 1 |
|  |  | Capacity | - Not feasible without more units | 0 |
| Sum |  |  |  | 8,5 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

#### Programmed Waste Collection (Bring System)

| The bring system is planned to serve an area which has collection platforms. The collection activities are programmed so that on a specific day, a specific waste stream shall be collected. In this way, no extra collection trucks need be purchased. As collection points are smaller in number and they are placed at easy accessible points, collection trucks have less stops and the routes are shorter, thus fuel consumption is lower than with the door-to-door option. Systems that require the voluntary effort of residents to walk to communal containers often offer the lowest possible costs, as the city pays only for collecting or emptying the containers at collection points. | Figure 8.3-7 Bring System Collection Service |
| --- | --- |

a) Investment Costs

The main technical features of the collection trucks for urban and rural areas in Phase-II of Arges County, are presented below:

| URBAN AREAS |  |  |  | RURAL AREAS |  |  |  | HHW Mobile Unit |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Capacity: | 24 | m3 |  | Capacity: | 16 | m3 |  | Capacity: | 4 | m3 |
| Gross weight: | 22,5 | t |  | Gross weight: | 19,5 | t |  |  |  |  |
| Axle: | 3 |  |  | Axle: | 2 |  |  | Axle: | 2 |  |
| Payload: | 12 | t |  | Payload: | 8 | t |  | Payload: | 5 | t |
| Efficiency per load: | 90% |  |  | Efficiency per load: | 90% |  |  | Efficiency per load: | 90% |  |
| Average speed | 50 | km/h |  | Average speed | 50 | km/h |  | Average speed | 50 | km/h |
| Shift: | 1 |  |  | Shift: | 1 |  |  | Shift: | 1 |  |
| Working days/year | 300 | days |  | Working days/year | 300 | days |  | Working days/year | 300 | days |
| Availability: | 95% |  |  | Availability: | 95% |  |  | Availability: | 95% |  |
| Collected tons/year: | 3.078 | t/year |  | Collected tons/year: | 2.052 | t/year |  | Collected tons/year: | 1.283 | t/year |
| Collected tons/week: | 71,82 | t/week |  | Collected tons/week: | 47,88 | t/week |  | Collected tons/week: | 29,925 | t/week |

The calculation of the number of collection trucks needed for Bring System collection services is presented as follows.

| Zone 1-Curtea de Arges |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Coverage Time/trip | 7,00 | hrs |  |  |  |  |  |
| Trips/truck-shift | 1,14 | Trips |  |  |  |  |  |
| Population 2013 |  | Waste generation Rates (tons/inh.-year) |  |  |  |  |  |
| Trip (km) | 310 | Organic | Glass | Plastic | Paper | Residues | CW |
| Total | 74.925 |  |  |  |  |  |  |
| Urban | 32.649 | 0,269 | 0,0346 | 0,0460 | 0,0374 | 0,1022 | 0,0700 |
| Rural | 42.276 | 0,042 | 0,0141 | 0,0188 | 0,0153 | 0,0371 | 0,0281 |
|  |  | Weekly amount of waste (tons/week) |  |  |  |  |  |
| Urban |  | 169 | 22 | 29 | 23 | 64 | 44 |
| Rural |  | 34 | 11 | 15 | 12 | 30 | 23 |
|  |  | Collection trucks (unit) |  |  |  |  |  |
| Type | Organic | Glass | Plastic | Paper | Residues | Total | HHW |
| 24 m3 | 3 | 1 | 1 | 1 | 1 | 7 | 1 |
| 16 m3 | 1 | 1 | 1 | 1 | 1 | 5 | 0 |
| Sum |  |  |  |  |  | 12 | 1 |
| Zone 2-Domnesti |  |  |  |  |  |  |  |
| Coverage Time/trip | 5,00 | hrs |  |  |  |  |  |
| Trips/truck-shift | 1,60 | Trips |  |  |  |  |  |
| Population 2013 |  | Waste generation Rates (tons/inh.-year) |  |  |  |  |  |
| Trip (km) | 202 | Organic | Glass | Plastic | Paper | Residues | CW |
| Total | 26.981 |  |  |  |  |  |  |
| Urban | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Rural | 26.981 | 0,0399 | 0,0133 | 0,0177 | 0,0144 | 0,0349 | 0,0264 |
|  |  | Weekly amount of waste (tons/week) |  |  |  |  |  |
| Urban |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Rural |  | 21 | 7 | 9 | 7 | 18 | 14 |
|  |  | Collection trucks (unit) |  |  |  |  |  |
| Type | Organic | Glass | Plastic | Paper | Residues | Total | HHW |
| 24 m3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 16 m3 | 1 | 1 | 1 | 1 | 1 | 5 | 0 |
| Sum |  |  |  |  |  | 5 | 0 |
| Zone 8-Costesti |  |  |  |  |  |  |  |
| Coverage Time/trip | 14,00 | hrs |  |  |  |  |  |
| Trips/truck-shift | 0,57 | Trips |  |  |  |  |  |
| Population 2013 |  | Waste generation Rates (tons/inh.-year) |  |  |  |  |  |
| Trip (km) | 680 | Organic | Glass | Plastic | Paper | Residues | CW |
| Total | 68.549 |  |  |  |  |  |  |
| Urban | 10.604 | 0,0571 | 0,0073 | 0,0098 | 0,0079 | 0,0217 | 0,0149 |
| Rural | 57.945 | 0,0379 | 0,0126 | 0,0168 | 0,0137 | 0,0332 | 0,0251 |
|  |  | Weekly amount of waste (tons/week) |  |  |  |  |  |
| Urban |  | 12 | 1 | 2 | 2 | 4 | 3 |
| Rural |  | 42 | 14 | 19 | 15 | 37 | 28 |
|  |  | Collection trucks (unit) |  |  |  |  |  |
| Type | Organic | Glass | Plastic | Paper | Residues | Total | HHW |
| 24 m3 | 1 | 1 | 1 | 1 | 1 | 5 | 1 |
| 16 m3 | 2 | 1 | 1 | 1 | 2 | 7 | 0 |
| Sum |  |  |  |  |  | 12 | 1 |
| Total in Phase-II |  |  |  |  |  | 29 | 2 |

The investments in Phase-II do not take into account the purchase of Collection trucks, only those for HHW will be included as investments in the project. The trucks are to be brought by the operators in the waste collection zones.

The calculation of investment costs for this option is presented in the next table.

| Investment Costs |  | Qty |  | Unit Costs (Euro) |  | Total (Euro) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Civil Works | Equipment | Civil Works | Equipment |
| Z-1 | HHW Vehicle | 1 | unit |  | 35.000 |  | 35.000 |
| Total Z1 |  |  |  |  |  | 0 | 35.000 |
| Z-2 |  |  |  |  |  |  |  |
| Total Z2 |  |  |  |  |  | 0 | 0 |
| Z-8 | HHW Vehicle | 1 | unit |  | 35.000 |  | 35.000 |
| Total Z8 |  |  |  |  |  | 0 | 35.000 |
| Total investments for Collection (Separated)= |  |  |  |  |  | 0 | 70.000 |

Here, the investments are the same as for all the other options for waste collection services, as there are only HHW collection trucks. No trucks need to be purchased.  The main objective in calculating the required number of trucks is to obtain the O&M Costs.

b) O&M Costs

The calculation of O&M costs are presented in the following table.

| Manpower for 1 shift |  |  |  |
| --- | --- | --- | --- |
| Gross salary | Euro/Month | months/year | Euro/year |
| Driver | 350 | 12 | 4.200 |
| Crew (2) | 305 | 24 | 7.320 |
|  |  |  | 11.520 |
| cost coefficient |  |  |  |
| Gross salary |  |  |  |
| Pension insurance, paid by enterprise |  | 10% |  |
| cost coefficient |  |  | 1,10 |
| working days a year | 52 | weeks/a | 260 |
| for unemployability (illness and disabled persons) | 12% |  | -31,2 |
| working days for vacation | 18 | d/year | -18 |
| legal holidays | 10 | d/year | -10 |
| real working days per worker and year |  |  | 200,8 |
| manpower coefficient |  |  | 1,29482 |
| Total cost coefficient |  |  | 1,42430 |
| Salary cost/shift |  |  | 16.407,97 |
| Number of shift/truck |  |  | 1 |
| Number of working day/truck |  |  | 250 |
| Working days a year |  |  | 260 |
| Time coefficient |  |  | 0,96154 |
| Salary cost per truck and year |  |  | 15.776,89 |
| Fuel and others |  |  |  |
| Number of shift/truck |  |  | 1 |
| Number of working day/truck |  |  | 250 |
| working hours/truck |  |  | 2.000 |
| fuel contribution l/h, €/l | 6 | 1,1 | 13.200 |
| tyre cost 1.1m³ system (collecting truck) per year |  |  | 478 |
| maintenance / insurance (as % of investment/year) | 7,5% | 157.378 | 10.125 |
| maintenance / insurance (as % of investment/year) | 7,5% | 154.641 | 8.250 |
| vehicle care cost |  |  | 120 |
| vehicle accomodation cost |  |  | 120 |
| other working materials/lubricants etc. (as % of fuel) | 0 |  | 660 |
| Total for 24 m³ truck per year |  |  | 24.703 |
| Total for 16 m³ truck per year |  |  | 22.828 |
| Total O&M Costs per Truck and Year |  |  |  |
|  |  |  | 1 |
|  |  |  | 250 |
| 24 m³ truck (without administration) |  |  | 40.480 |
| administration (as % of total O&M costs) | 7,5% |  | 3.036 |
| Total O&M + administration for 24 m³ truck |  |  | 43.516 |
| 16 m³ truck (without administration) |  |  | 38.605 |
| administration (as % of total O&M costs) | 7,5% |  | 2.895 |
| Total O&M + administration for 16 m³ truck |  |  | 41.500 |

| Residue Collection with 1,5 shifts/day and 6 days/week (300 days/year): |  |  |  |  |  | O&M costs: |  | capital costs: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Urban Areas Residue Collection: | 2 | á | 24 m³ | collection trucks |  | 90.640 | €/year | 31.476 | €/year |  |
|  | 0 | á | 16m³ | collection trucks |  | 0 | €/year | 0 | €/year |  |
| Rural Areas Residue Collection: | 4 | á | 16 m³ | collection trucks |  | 180.397 | €/year | 61.856 | €/year |  |
| Total Residue Collection: | 6 |  |  |  |  | 271.036 | €/year | 93.332 | €/year |  |
| Recyclcables Collection with 1,5 shifts/day and 6 days/week (300 days/year): |  |  |  |  |  | O&M costs: |  | capital costs: |  |  |
| Urban Areas Recyclables Collection: | 10 | á | 24 m³ | collection trucks |  | 453.198 | €/year | 157.378 | €/year |  |
|  | 0 | á | 16m³ | collection trucks |  | 0 | €/year | 0 | €/year |  |
| Rural Areas RecyclablesCollection: | 13 | á | 16 m³ | collection trucks |  | 586.289 | €/year | 201.033 | €/year |  |
| Total Residue Collection: | 23 |  |  |  |  | 1.039.487 | €/year | 358.410 | €/year |  |
|  |  |  |  | Sum |  | 1.310.524 | €/year | 451.742 | €/year |  |
| HHW |  |  |  |  |  |  |  |  |  |  |
| URBAN |  |  |  |  |  | O&M costs: |  | capital costs: |  |  |
| HHW | 2 | á | Mobil Unit | Truck |  | 90.198 | €/year | 31.476 | €/year |  |
| RURAL |  |  |  |  |  | O&M costs: |  | capital costs: |  |  |
| HHW | 0 | á | Mobil Unit | Truck |  | 0 | €/year | 0 | €/year |  |
| Total= | 2 |  |  | collection trucks |  | 90.198 | €/year | 31.476 | €/year |  |
|  |  |  |  | Sum |  | 1.490.920 | €/year | 514.693 | 2.005.613 | €/year |

c) Externalities

The following table presents the externalities associated with the programmed waste collection (Bring System) Option in the Phase-II zones of Arges County.

Table 8.3-7 Externalities associated with the programmed waste collection (Bring System) option

| Field | Implications |  | Bring System | Discretional1 |
| --- | --- | --- | --- | --- |
| Environmental Impacts |  | Avoiding health risks | + solid waste collected in proper way + Correct use ensures clean conditions | 1 |
|  |  | Emissions | - 905 tons CO2/year | 0,5 |
|  |  | Noise | - More trucks=more noise | 0,5 |
| Resource Saving |  | Number of trucks | + 31 collection trucks | 1 |
|  |  | Collection services | + Separate waste collection every week | 1 |
|  |  | Fuel Consumption | + More speed (40 Km/h) = Less fuel consumption is demanded due to collection platforms | 1 |
| Recycling Targets |  | Separated storage | + Yes | 1 |
|  |  | Household Hazardous Waste | - Special truck required | 0,5 |
|  |  | Biodegradable fraction | + Yes | 1 |
|  |  | Recyclable fraction | + Yes | 1 |
| Society |  | Acceptability | - Average as service is to platforms | 0,5 |
|  |  | Environmental awareness | + Very high as separation at source | 1 |
| Operational |  | Road traffic | - Average as more collection trucks | 0,5 |
|  |  | Applicability | + Feasible | 1 |
| Flexibility /Adaptability |  | Methodology | - Not Feasible | 0 |
|  |  | Capacity | - Not Feasible | 0 |
| Sum |  |  |  | 11,5 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

#### Door to Door Waste Collection (Separated)

| This system is closely linked with the domiciliary tempory storage of waste. The Door-to-Door waste collection system is labour, fuel, and vehicle intensive, and needs to be repeated every week. This option is similar to the first option, however with the distinction that collection services will be for waste storage separated at source i.e. every household has three bins with different waste streams. Here, the main variables will be the number of trucks needed for separate collection services, thus the O&M costs will rise. | Figure 8.3-8 Separate Collection System |
| --- | --- |

a) Investment Costs

The main technical features of the collection trucks for urban and rural areas in Phase-II of Arges County, are presented below:

| URBAN AREAS |  |  |  | RURAL AREAS |  |  |  | HHW Mobile Unit |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Capacity: | 24 | m3 |  | Capacity: | 16 | m3 |  | Capacity: | 4 | m3 |
| Gross weight: | 22,5 | t |  | Gross weight: | 19,5 | t |  |  |  |  |
| Axle: | 3 |  |  | Axle: | 2 |  |  | Axle: | 2 |  |
| Payload: | 12 | t |  | Payload: | 8 | t |  | Payload: | 5 | t |
| Efficiency per load: | 90% |  |  | Efficiency per load: | 90% |  |  | Efficiency per load: | 90% |  |
| Average speed | 20 | km/h |  | Average speed | 20 | km/h |  | Average speed | 20 | km/h |
| Shift: | 1 |  |  | Shift: | 1 |  |  | Shift: | 1 |  |
| Working days/year | 300 | days |  | Working days/year | 300 | days |  | Working days/year | 300 | days |
| Availability: | 95% |  |  | Availability: | 95% |  |  | Availability: | 95% |  |
| Collected tons/year: | 3.078 | t/year |  | Collected tons/year: | 2.052 | t/year |  | Collected tons/year: | 1.283 | t/year |
| Collected tons/week: | 71,82 | t/week |  | Collected tons/week: | 47,88 | t/week |  | Collected tons/week: | 29,925 | t/week |

The calculation of the number of collection trucks needed for Door-to-Door collection services is presented as follows:

| Zone 1-Curtea de Arges |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Coverage Time/trip | 28,00 | hrs |  |  |  |  |  |
| Trips/truck-shift | 0,29 | Trips |  |  |  |  |  |
| Population 2013 |  | Waste generation Rates (tons/inh.-year) |  |  |  |  |  |
| Trip (km) | 543 | Organic | Glass | Plastic | Paper | Residues | CW |
| Total | 74.925 |  |  |  |  |  |  |
| Urban | 32.649 | 0,269 | 0,0346 | 0,0460 | 0,0374 | 0,1022 | 0,0700 |
| Rural | 42.276 | 0,042 | 0,0141 | 0,0188 | 0,0153 | 0,0371 | 0,0281 |
|  |  | Weekly amount of waste (tons/week) |  |  |  |  |  |
| Urban |  | 169 | 22 | 29 | 23 | 64 | 44 |
| Rural |  | 34 | 11 | 15 | 12 | 30 | 23 |
|  |  | Collection trucks (unit) |  |  |  |  |  |
| Type | Organic | Glass | Plastic | Paper | Residues | Total | HHW |
| 24 m3 | 9 | 2 | 2 | 2 | 4 | 19 | 1 |
| 16 m3 | 3 | 1 | 2 | 1 | 3 | 10 | 1 |
| Sum |  |  |  |  |  | 29 | 2 |
| Zone 2-Domnesti |  |  |  |  |  |  |  |
| Coverage Time/trip | 11,00 | hrs |  |  |  |  |  |
| Trips/truck-shift | 0,73 | Trips |  |  |  |  |  |
| Population 2013 |  | Waste generation Rates (tons/inh.-year) |  |  |  |  |  |
| Trip (km) | 202 | Organic | Glass | Plastic | Paper | Residues | CW |
| Total | 26.981 |  |  |  |  |  |  |
| Urban | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Rural | 26.981 | 0,0399 | 0,0133 | 0,0177 | 0,0144 | 0,0349 | 0,0264 |
|  |  | Weekly amount of waste (tons/week) |  |  |  |  |  |
| Urban |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Rural |  | 21 | 7 | 9 | 7 | 18 | 14 |
|  |  | Collection trucks (unit) |  |  |  |  |  |
| Type | Organic | Glass | Plastic | Paper | Residues | Total | HHW |
| 24 m3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 16 m3 | 1 | 1 | 1 | 1 | 1 | 5 | 0 |
| Sum |  |  |  |  |  | 5 | 0 |
| Zone 8-Costesti |  |  |  |  |  |  |  |
| Coverage Time/trip | 34,00 | hrs |  |  |  |  |  |
| Trips/truck-shift | 0,24 | Trips |  |  |  |  |  |
| Population 2013 |  | Waste generation Rates (tons/inh.-year) |  |  |  |  |  |
| Trip (km) | 680 | Organic | Glass | Plastic | Paper | Residues | CW |
| Total | 68.549 |  |  |  |  |  |  |
| Urban | 10.604 | 0,0571 | 0,0073 | 0,0098 | 0,0079 | 0,0217 | 0,0149 |
| Rural | 57.945 | 0,0379 | 0,0126 | 0,0168 | 0,0137 | 0,0332 | 0,0251 |
|  |  | Weekly amount of waste (tons/week) |  |  |  |  |  |
| Urban |  | 12 | 1 | 2 | 2 | 4 | 3 |
| Rural |  | 42 | 14 | 19 | 15 | 37 | 28 |
|  |  | Collection trucks (unit) |  |  |  |  |  |
| Type | Organic | Glass | Plastic | Paper | Residues | Total | HHW |
| 24 m3 | 1 | 1 | 1 | 1 | 1 | 5 | 1 |
| 16 m3 | 4 | 2 | 2 | 2 | 4 | 14 | 0 |
| Sum |  |  |  |  |  | 19 | 1 |
| Total in Phase-II |  |  |  |  |  | 53 | 3 |

The investments in Phase-II do not take into account the purchase of Collection trucks, only those for HHW will be included as investments for the project. It is foreseen that the trucks be brought by operators in the waste collection zones.

The calculation of the Investment Costs for this option is presented in the next table.

| Investment Costs |  | Qty |  | Unit Costs (Euro) |  | Total (Euro) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Civil Works | Equipment | Civil Works | Equipment |
| Z-1 | HHW Vehicle | 2 | unit |  | 35.000 |  | 70.000 |
| Total Z1 |  |  |  |  |  | 0 | 70.000 |
| Z-2 |  |  |  |  |  |  |  |
| Total Z2 |  |  |  |  |  | 0 | 0 |
| Z-8 | HHW Vehicle | 1 | unit |  | 35.000 |  | 35.000 |
| Total Z8 |  |  |  |  |  | 0 | 35.000 |
| Total investments for Collection (Mixed)= |  |  |  |  |  | 0 | 105.000 |

Here, the investments are the same as for all the other options for waste collection services, as there are only HHW collection trucks. No trucks need to be purchased.

The main objective in calculating the number is to know the number of trucks and to obtain the O&M Costs.

b) O&M Costs

The calculation of O&M costs are presented in the next table.

| Manpower for 1 shift |  |  |  |
| --- | --- | --- | --- |
| Gross salary | Euro/Month | months/year | Euro/year |
| Driver | 350 | 12 | 4.200 |
| Crew (2) | 305 | 24 | 7.320 |
|  |  |  | 11.520 |
| cost coefficient |  |  |  |
| Gross salary |  |  |  |
| Pension insurance, paid by enterprise |  | 10% |  |
| cost coefficient |  |  | 1,10 |
| working days a year | 52 | weeks/a | 260 |
| for unemployability (illness and disabled persons) | 12% |  | -31,2 |
| working days for vacancie | 18 | d/year | -18 |
| legal holidays | 10 | d/year | -10 |
| real working days per worker and year |  |  | 200,8 |
| manpower coefficient |  |  | 1,29482 |
| Total cost coefficient |  |  | 1,42430 |
| Salary cost/shift |  |  | 16.407,97 |
| Number of shift/truck |  |  | 1 |
| Number of working day/truck |  |  | 250 |
| Working days a year |  |  | 260 |
| Time coefficient |  |  | 0,96154 |
| Salary cost per truck and year |  |  | 15.776,89 |
| Fuel and others |  |  |  |
| Number of shift/truck |  |  | 1 |
| Number of working day/truck |  |  | 250 |
| working hours/truck |  |  | 2.000 |
| fuel contribution l/h, €/l | 6 | 1,1 | 13.200 |
| tyre cost 1.1m³ system (collecting truck) per year |  |  | 478 |
| maintenance / insurance (as % of investment/year) | 7,5% | 135.000 | 10.125 |
| maintenance / insurance (as % of investment/year) | 7,5% | 110.000 | 8.250 |
| vehicle care cost |  |  | 120 |
| vehicle accomodation cost |  |  | 120 |
| other working materials/lubricants etc. (as % of fuel) | 0 |  | 660 |
| Total for 24 m³ truck per year |  |  | 24.703 |
| Total for 16 m³ truck per year |  |  | 22.828 |
| Total O&M Costs per Truck and Year |  |  |  |
|  |  |  | 1 |
|  |  |  | 250 |
| 24 m³ truck (without administration) |  |  | 40.480 |
| administration (as % of total O&M costs) | 7,5% |  | 3.036 |
| Total O&M + administration for 24 m³ truck |  |  | 43.516 |
| 16 m³ truck (without administration) |  |  | 38.605 |
| administration (as % of total O&M costs) | 7,5% |  | 2.895 |
| Total O&M + administration for 16 m³ truck |  |  | 41.500 |

| Residue Collection with 1,5 shifts/day and 6 days/week (300 days/year): |  |  |  |  |  | O&M costs: |  | capital costs: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Urban Areas Residue Collection: | 5 | á | 24 m³ | collection trucks |  | 226.599 | €/year | 78.689 | €/year |  |
|  | 0 | á | 16m³ | collection trucks |  | 0 | €/year | 0 | €/year |  |
| Rural Areas Residue Collection: | 8 | á | 16 m³ | collection trucks |  | 360.793 | €/year | 123.712 | €/year |  |
| Total Residue Collection: | 13 |  |  |  |  | 587.392 | €/year | 202.401 | €/year |  |
| Recyclcables Collection with 1,5 shifts/day and 6 days/week (300 days/year): |  |  |  |  |  | O&M costs: |  | capital costs: |  |  |
| Urban Areas Recyclables Collection: | 19 | á | 24 m³ | collection trucks |  | 861.077 | €/year | 299.017 | €/year |  |
|  | 0 | á | 16m³ | collection trucks |  | 0 | €/year | 0 | €/year |  |
| Rural Areas RecyclablesCollection: | 21 | á | 16 m³ | collection trucks |  | 947.082 | €/year | 324.745 | €/year |  |
| Total Residue Collection: | 40 |  |  |  |  | 1.808.159 | €/year | 623.762 | €/year |  |
| HHW |  |  |  |  |  |  |  |  |  |  |
| URBAN |  |  |  |  |  | O&M costs: |  | capital costs: |  |  |
| HHW | 2 | á | Mobil Unit | Truck |  | 90.198 | €/year | 31.476 | €/year |  |
| RURAL |  |  |  |  |  | O&M costs: |  | capital costs: |  |  |
| HHW | 1 | á | Mobil Unit | Truck |  | 45.099 | €/year | 15.738 | €/year |  |
| Total= | 3 |  |  | collection trucks |  | 135.297 | €/year | 47.213 | €/year |  |
|  |  |  |  |  | Sum | 2.621.047 | €/year | 904.852 | 3.525.900 | €/year |

c) Externalities

The following table presents the externalities associated with the Door-to-Door Waste Collection System (Separate) Option in the Phase-II zones of Arges County.

Table 8.3-8 Externalities associated to Door-to-Door Waste Collection System (Separate) Option

| Field | Implications |  | Domiciliary Collection (Separate) | Discretional1 |
| --- | --- | --- | --- | --- |
| Environmental Impacts |  | Avoiding health risks | + solid waste collected in proper way + Correct use ensures clean conditions | 1 |
|  |  | Emissions | + 922 tons CO2/year | 0,5 |
|  |  | Noise | - More trucks=more noise | 0,5 |
| Resource Saving |  | Number of trucks | - 56 collection trucks | 0 |
|  |  | Collection services | + Separate waste collection every week | 1 |
|  |  | Fuel Consumption | - Less speed (20 Km/h) = More fuel consumption is demanded due to door-to-door collection | 0 |
| Recycling Targets |  | Separated storage | + Yes | 1 |
|  |  | Household Hazardous Waste | - Special truck required | 0,5 |
|  |  | Biodegradable fraction | + Yes | 1 |
|  |  | Recyclable fraction | + Yes | 1 |
| Society |  | Acceptability | + High as service is domiciliary | 1 |
|  |  | Environmental awareness | + Very high as separation at source | 1 |
| Operational |  | Road traffic | - More trucks= more traffic in roads | 0 |
|  |  | Applicability | - Feasible | 1 |
| Flexibility /Adaptability |  | Methodology | + Feasible | 1 |
|  |  | Capacity | - Not feasible without more units | 0 |
| Sum |  |  |  | 10,5 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

#### Mixed Waste Collection System (Domiciliary + Communal)

| This system is closely linked with a mixed waste storage system, where there are areas served with domiciliary waste collection and communal waste collection. The criteria to define the areas to be served by a particular collection system follows the same principle as tempory storage (see previous section), where natural topography and available space rule the implementation of domiciliary or communal collection service. The Door-to-Door waste collection system is labour, fuel, and vehicle intensive, and needs to be repeated every week. The communal waste collection services, however, have a higher average speed with the corresponding reduction in operational costs. Here, the main variables will be the number of trucks needed for separate collection services, thus the O&M costs will rise. | Figure 8.3-9 Mixed Collection System |
| --- | --- |

a) Investment Costs

The main technical features of the collection trucks for urban and rural areas in Phase-II of Arges County, are presented below:

| URBAN AREAS |  |  |  | RURAL AREAS |  |  |  | HHW Mobile Unit |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Capacity: | 24 | m3 |  | Capacity: | 16 | m3 |  | Capacity: | 4 | m3 |
| Gross weight: | 22,5 | t |  | Gross weight: | 19,5 | t |  |  |  |  |
| Axle: | 3 |  |  | Axle: | 2 |  |  | Axle: | 2 |  |
| Payload: | 12 | t |  | Payload: | 8 | t |  | Payload: | 5 | t |
| Efficiency per load: | 90% |  |  | Efficiency per load: | 90% |  |  | Efficiency per load: | 90% |  |
| Average speed | 40 | km/h |  | Average speed | 40 | km/h |  | Average speed | 40 | km/h |
| Shift: | 1 |  |  | Shift: | 1 |  |  | Shift: | 1 |  |
| Working days/year | 300 | days |  | Working days/year | 300 | days |  | Working days/year | 300 | days |
| Availability: | 95% |  |  | Availability: | 95% |  |  | Availability: | 95% |  |
| Collected tons/year: | 3.078 | t/year |  | Collected tons/year: | 2.052 | t/year |  | Collected tons/year: | 1.283 | t/year |
| Collected tons/week: | 71,82 | t/week |  | Collected tons/week: | 47,88 | t/week |  | Collected tons/week: | 29,925 | t/week |

The calculation of the number of collection trucks needed for mixed collection services is presented as follows:

| Zone 1-Curtea de Arges |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Coverage Time/trip | 12,00 | hrs |  |  |  |  |  |
| Trips/truck-shift | 0,67 | Trips |  |  |  |  |  |
| Population 2013 |  | Waste generation Rates (tons/inh.-year) |  |  |  |  |  |
| Trip (km) | 465 | Organic | Glass | Plastic | Paper | Residues | CW |
| Total | 74.925 |  |  |  |  |  |  |
| Urban | 32.649 | 0,269 | 0,0346 | 0,0460 | 0,0374 | 0,1022 | 0,0700 |
| Rural | 42.276 | 0,042 | 0,0141 | 0,0188 | 0,0153 | 0,0371 | 0,0281 |
|  |  | Weekly amount of waste (tons/week) |  |  |  |  |  |
| Urban |  | 169 | 22 | 29 | 23 | 64 | 44 |
| Rural |  | 34 | 11 | 15 | 12 | 30 | 23 |
|  |  | Collection trucks (unit) |  |  |  |  |  |
| Type | Organic | Recyclables |  |  | Residues | Total | HHW |
| 24 m3 | 4 | 2 |  |  | 2 | 8 | 1 |
| 16 m3 | 2 | 2 |  |  | 1 | 5 | 0 |
| Sum |  |  |  |  |  | 13 | 1 |
| Zone 2-Domnesti |  |  |  |  |  |  |  |
| Coverage Time/trip | 7,00 | hrs |  |  |  |  |  |
| Trips/truck-shift | 0,89 | Trips |  |  |  |  |  |
| Population 2013 |  | Waste generation Rates (tons/inh.-year) |  |  |  |  |  |
| Trip (km) | 263 | Organic | Glass | Plastic | Paper | Residues | CW |
| Total | 26.981 |  |  |  |  |  |  |
| Urban | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Rural | 26.981 | 0,0399 | 0,0133 | 0,0177 | 0,0144 | 0,0349 | 0,0264 |
|  |  | Weekly amount of waste (tons/week) |  |  |  |  |  |
| Urban |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Rural |  | 21 | 7 | 9 | 7 | 18 | 14 |
|  |  | Collection trucks (unit) |  |  |  |  |  |
| Type | Organic | Recyclables |  |  | Residues | Total | HHW |
| 24 m3 | 0 | 0 |  |  | 0 | 0 | 0 |
| 16 m3 | 1 | 1 |  |  | 1 | 3 | 0 |
| Sum |  |  |  |  |  | 3 | 0 |
| Zone 8-Costesti |  |  |  |  |  |  |  |
| Coverage Time/trip | 23,00 | hrs |  |  |  |  |  |
| Trips/truck-shift | 0,35 | Trips |  |  |  |  |  |
| Population 2013 |  | Waste generation Rates (tons/inh.-year) |  |  |  |  |  |
| Trip (km) | 884 | Organic | Glass | Plastic | Paper | Residues | CW |
| Total | 68.549 |  |  |  |  |  |  |
| Urban | 10.604 | 0,0571 | 0,0073 | 0,0098 | 0,0079 | 0,0217 | 0,0149 |
| Rural | 57.945 | 0,0379 | 0,0126 | 0,0168 | 0,0137 | 0,0332 | 0,0251 |
|  |  | Weekly amount of waste (tons/week) |  |  |  |  |  |
| Urban |  | 12 | 1 | 2 | 2 | 4 | 3 |
| Rural |  | 42 | 14 | 19 | 15 | 37 | 28 |
|  |  | Collection trucks (unit) |  |  |  |  |  |
| Type | Organic | Recylcables |  |  | Residues | Total | HHW |
| 24 m3 | 1 | 1 |  |  | 1 | 3 | 1 |
| 16 m3 | 3 |  |  |  | 3 | 9 | 0 |
| Sum |  |  |  |  |  | 18 | 1 |
| Total in Phase-II |  |  |  |  |  | 28 | 2 |

The investments in Phase-II do not take into account the purchase of Collection trucks, only those for HHW will be included as investments for the project. It is foreseen that the trucks be brought by operators of waste collection zones.

The calculation of the Investment Costs for this Option is presented in the next table.

| Investment Costs |  | Qty |  | Unit Costs (Euro) |  | Total (Euro) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Civil Works | Equipment | Civil Works | Equipment |
| Z-1 | HHW Vehicle | 1 | unit |  | 35.000 |  | 35.000 |
| Total Z1 |  |  |  |  |  | 0 | 35.000 |
| Z-2 |  |  |  |  |  |  |  |
| Total Z2 |  |  |  |  |  | 0 | 0 |
| Z-8 | HHW Vehicle | 1 | unit |  | 35.000 |  | 35.000 |
| Total Z8 |  |  |  |  |  | 0 | 35.000 |
| Total investments for Collection (Mixed)= |  |  |  |  |  | 0 | 70.000 |

Here, the investments are the same as for all the other options for waste collection services, as there are only HHW collection trucks. No trucks need to be purchased.

The main objective in calculating the number is to know the number of trucks and to obtain the O&M Costs.

b) O&M Costs

The calculation of O&M costs are presented in the next table.

| Manpower for 1 shift |  |  |  |
| --- | --- | --- | --- |
| Brut salary | Euro/Month | months/year | Euro/year |
| Driver | 350 | 12 | 4.200 |
| Crew (2) | 305 | 24 | 7.320 |
|  |  |  | 11.520 |
| cost coefficient |  |  |  |
| Brut salary |  |  |  |
| Pension insurance, payed by enterprise |  | 10% |  |
| cost coefficient |  |  | 1,10 |
| working days a year | 52 | weeks/a | 260 |
| for unemployability (illnes and disabled persons) | 12% |  | -31,2 |
| working days for vacancie | 18 | d/year | -18 |
| legal holidays | 10 | d/year | -10 |
| real working days per worker and year |  |  | 200,8 |
| manpower coefficient |  |  | 1,29482 |
| Total cost coefficient |  |  | 1,42430 |
| Salary cost/shift |  |  | 16.407,97 |
| Number of shift/truck |  |  | 1 |
| Number of working day/truck |  |  | 250 |
| Working days a year |  |  | 260 |
| Time coefficient |  |  | 0,96154 |
| Salary cost per truck and year |  |  | 15.776,89 |
| Fuel and others |  |  |  |
| Number of shift/truck |  |  | 1 |
| Number of working day/truck |  |  | 250 |
| working hours/truck |  |  | 2.000 |
| fuel contribution l/h, €/l | 6 | 1,1 | 13.200 |
| tyre cost 1.1m³ system (collecting truck) per year |  |  | 478 |
| maintenance / insurance (as % of investment/year) | 7,5% | 135.000 | 10.125 |
| maintenance / insurance (as % of investment/year) | 7,5% | 110.000 | 8.250 |
| vehicle care cost |  |  | 120 |
| vehicle accomodation cost |  |  | 120 |
| other working materials/lubricants etc. (as % of fuel) | 0 |  | 660 |
| Total for 24 m³ truck per year |  |  | 24.703 |
| Total for 16 m³ truck per year |  |  | 22.828 |
| Total O&M Costs per Truck and Year |  |  |  |
|  |  |  | 1 |
|  |  |  | 250 |
| 24 m³ truck (without administration) |  |  | 40.480 |
| administration (as % of total O&M costs) | 7,5% |  | 3.036 |
| Total O&M + administration for 24 m³ truck |  |  | 43.516 |
| 16 m³ truck (without administration) |  |  | 38.605 |
| administration (as % of total O&M costs) | 7,5% |  | 2.895 |
| Total O&M + administration for 16 m³ truck |  |  | 41.500 |

| Residue Collection with 1 shifts/day and 6 days/week (300 days/year): |  |  |  |  |  | O&M costs: |  | capital costs: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Urban Areas Residue Collection: | 3 | á | 24 m³ | collection trucks |  | 135.959 | €/year | 47.213 | €/year |  |
|  | 0 | á | 16m³ | collection trucks |  | 0 | €/year | 0 | €/year |  |
| Rural Areas Residue Collection: | 5 | á | 16 m³ | collection trucks |  | 225.496 | €/year | 77.320 | €/year |  |
| Total Residue Collection: | 8 |  |  |  |  | 361.455 | €/year | 124.534 | €/year |  |
| Residue Collection with 1 shifts/day and 6 days/week (300 days/year): |  |  |  |  |  | O&M costs: |  | capital costs: |  |  |
| Urban Areas Recyclables Collection: | 8 | á | 24 m³ | collection trucks |  | 362.559 | €/year | 125.902 | €/year |  |
|  | 0 | á | 16m³ | collection trucks |  | 0 | €/year | 0 | €/year |  |
| Rural Areas RecyclablesCollection: | 12 | á | 16 m³ | collection trucks |  | 541.190 | €/year | 185.569 | €/year |  |
| Total Residue Collection: | 20 |  |  |  |  | 903.749 | €/year | 311.471 | €/year |  |
| HHW |  |  |  |  |  |  |  |  |  |  |
| URBAN |  |  |  |  |  | O&M costs: |  | capital costs: |  |  |
| HHW | 2 | á | Mobil Unit | Truck |  | 90.198 | €/year | 31.476 | €/year |  |
| RURAL |  |  |  |  |  | O&M costs: |  | capital costs: |  |  |
| HHW | 0 | á | Mobil Unit | Truck |  | 0 | €/year | 0 | €/year |  |
| Total= | 2 |  |  | collection trucks |  | 90.198 | €/year | 31.476 | €/year |  |
|  |  |  |  |  | Sum | 1.246.869 | €/year | 308.389 | 1.555.258 | €/year |

c) Externalities

The following table presents the externalities associated with the Mixed Waste Collection System Option in the Phase-II zones of Arges County.

Table 8.3.9 Externalities associated with the Mixed Waste Collection System (Separate) Option

| Field | Implications |  | Domiciliary Collection (Separate) | Discretional1 |
| --- | --- | --- | --- | --- |
| Environmental Impacts |  | Avoiding health risks | + solid waste collected in proper way + Correct use ensures clean conditions | 1 |
|  |  | Emissions | + 987 tons CO2/year | 0,5 |
|  |  | Noise | - More trucks=more noise | 0,5 |
| Resource Saving |  | Number of trucks | - 42 collection trucks | 0 |
|  |  | Collection services | + Separate waste collection every week | 1 |
|  |  | Fuel Consumption | - More speed (30 Km/h) = Average fuel consumption is demanded due to door-to-door collection | 0,5 |
| Recycling Targets |  | Separated storage | + Yes | 1 |
|  |  | Household Hazardous Waste | - Special truck required | 0,5 |
|  |  | Biodegradable fraction | + Yes | 1 |
|  |  | Recyclable fraction | + Yes | 1 |
| Society |  | Acceptability | + High as service is domiciliary | 1 |
|  |  | Environmental awareness | + Very high as separation at source | 1 |
| Operational |  | Road traffic | - More trucks= more traffic in roads | 0 |
|  |  | Applicability | - Feasible | 1 |
| Flexibility /Adaptability |  | Methodology | + Feasible | 1 |
|  |  | Capacity | - Not feasible without more units | 0 |
| Sum |  |  |  | 11 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

#### Evaluation of Options for Waste Collection Services

| Criteria |  | A | B | C | D |
| --- | --- | --- | --- | --- | --- |
|  |  | Door-to Door (Mixed Storage) | Collection to Platfroms -Bring System (Separate Storage) | Door-toDoor (Separate Storage) | Mixed (Domiciliary + Communal) |
| Investments |  | 981.215 | 823.837 | 981.215 | 823.837 |
| Running Costs |  | 2.855.529 | 2.005.613 | 3.525.900 | 1.944.556 |
| Externalities |  |  |  |  |  |
| Environmental Impacts | Preservation of Human health | 1 | 1 | 1 | 1 |
|  | Emissions | 1 | 0,5 | 0,5 | 0,5 |
|  | Noise | 1 | 0,5 | 0,5 | 0,5 |
| Resource Saving | Number of trucks | 1 | 1 | 0 | 0 |
|  | Collection services | 0 | 1 | 1 | 1 |
|  | Fuel Consumption | 0 | 1 | 0 | 0,5 |
| Recycling Targets | Separated storage | 0 | 1 | 1 | 1 |
|  | Household Hazardous Waste | 0,5 | 0,5 | 0,5 | 0,5 |
|  | Biodegradable fraction | 0 | 1 | 1 | 1 |
|  | Recyclable fraction | 0 | 1 | 1 | 1 |
| Society | Acceptability | 1 | 0,5 | 1 | 1 |
|  | Environmental awareness | 0 | 1 | 1 | 1 |
| Operational | Road traffic | 1 | 0,5 | 0 | 0 |
|  | Applicability | 1 | 1 | 1 | 1 |
| Flexibility /Adaptability | Methodology | 1 | 0 | 1 | 1 |
|  | Capacity | 0 | 0 | 0 | 0 |
|  |  | 8,5 | 11,5 | 10,5 | 11 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

The table above shows the main comparison between the 4 different waste collection options for the Phase-II zones of Arges County. Options B & D represent lower investments and their external benefits are the highest. It is necessary to point out that lower investments may represent higher investments in the further stages of WM, in order to reach the targets. However, this issue can not be assessed for individual stages of WM, but has to be assessed as a part of a WM System (see Section 8.3).

The preferred option for waste collection is option D, since it represents the second best external benefits and the flexibility in its implementation seems to be the highest compared to the rest of the options. The economic benefits will be assessed in Section 8.4.

The main reasons for these results are associated with the length of collection routes and average collection speeds. Door-to-Door collection services require longer distances to travel at lower speeds, thus the number of trucks to cover the same route has to be increased. Moreover, these conditions lead to a higher consumption of resources which are reflected in the O&M costs.

### Number of Transfer Stations

Targets and Principles

There are no quantified targets for transfer and transport, but the system component principally aims to optimise the logistic processes within the overall shipment of residual waste from the waste producers to the landfill.

It is worth mentioning the following framework conditions for the transfer and transport system:

- location of the new county landfill.

- transfer stations already designed and/or under implementation

- definition of locations of the transfer stations as set out during the Master Plan stage of the project.

- good road accessibility (connection to the main road system) to ensure safe handling of the transport processes even under adverse weather conditions.

- location in the vicinity of the main centre of waste generation to optimise the collection processes.

- good road connection to the allocated new landfill, using the Romanian primary road system (that is not the same as mentioned in the first paragraph).

According to the Guideline provisions for Transfer Stations for Solid Municipal Waste, the transfer stations location criteria are:

- surface between 0.3 and 2 ha

- availability of facilities (electricity, water supply, wastewater network)

- services: entrance control, weighbridge with data recorder, discharge from waste collection trucks to transfer trucks, short-term storage and waste transfer to disposal, collection and treatment of rain water and leachate, civic amenity area.

The alternatives to be analyzed were chosen according to previous WM stages, like waste collection services. This study does not claim to include alternatives that simply cannot be implemented or which are unaffordable for the Phase-II case.

The most convenient variants are as follows:

- 2 new Transfer stations (Curtea de Arges & Costesti) to be fed by a mixed waste collection in the areas of Phase-II

- 2 new Transfer stations (Curtea de Arges & Costesti) to be feed by a separate waste collection (bring system) in the areas of Phase-II

- 1 new Transfer station (Costesti) and an extension of the Transfer Station in Campulung (Phase-I), both to be fed by a separate waste collection (Domiciliary) in the areas of Phase-II

a) 2 New Transfer Stations, Curtea de Arges & Costesti for Mixed Collection

This option basically consists of implementing two transfer stations.

- Transfer Station of Curtea de Arges. Located on the land where the disposal site of Curtea de Arges is currently placed. The transfer station will serve Zones 1 & 2.

- Transfer Station of Costesti. This Transfer Station will be located on the fringes of Costesti City. This facility will serve the entire Zone 8 of Arges County.

The transfer stations will be fed by a mixed collection system. The TS should have a large enough capacity to manage the wastes that are expected to be handled at the facility throughout its operating life. Factors that should be considered when determining the appropriate size of a transfer facility include:

- capacity of collection vehicles using the facility

- desired number of days of storage space on tipping floor

- time required to unload collection vehicles

- number of vehicles that will use the station and their expected days and hours of arrival (design to accommodate peak requirements)

- waste sorting or processing to be accomplished at the facility

- transfer trailer capacity

- hours of station operation

- availability of transfer trailers waiting for loading

| The distances travelled, from the proposed transfer stations to the regional landfill in Albota, are presented in the next atble. Distances (Km); Curtea de Arges city; Domnesti Commune; Costesti city; Albota landfill. Curtea de Arges city; 0; 19,3; 64,7; 40,0. Domnesti Commune; 19,3; 0; 69,6; 48,5. Costesti city; 64,7; 69,6; 0; 23,1. Albota lnadfill; 40,0; 48,5; 23,1; 0. The transfer trucks will have at least the capacity of five collection trucks; thereby reducing the number of trips of each vehicle from the Zones to the Landfill. If the diversion, of biodegradable, recyclables and HHW at these facilities, is taken into account, the waste for disposal will be reduced, and thus also the number of trips from the transfer stations to the landfill in Albota. | Figure 8.3-10 Location of new transfer station for zones 1,2 & 8 |
| --- | --- |

a.1) Investment Costs and O&M Costs

The investments considered for the new transfer stations are:

- Concrete platform for facilities of tipping floor and green point, incl. earth works

- Pavement for roads, incl. boundaries, accesses and road marking

- Garage, building for waste recycling and composting storage, inc. Earth works

- Building for personnel, incl. Earth works

- Container 30 mc

- Front end loader

- Car washing

- Waste water collection system, incl. earth works

- Basin for rainwater, incl. Earthworks

- Water supply, electricity, sanitary and heating services

- Weighbridge

- Well for water supply, including filter and installation

- Pump for the well

- Fence for facilities of Transfer station, composting plant and green point, incl. earth works

- Landscaping

- Container 30 mc

- Container 10 mc for HHW

- Compactors for waste

The civil works and equipment investments are as presented in the next table.

| New TS | Component | Civil Works | Equipment | Investment | O&M Costs | Amortization | Running Costs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Curtea de Arges | 649.103 € | 354.568 € | 1.003.671 € | 155.786 € | 71.565 € | 227.351 € |
| 2 | Costesti | 628.835 € | 346.863 € | 975.697 € | 110.605 € | 56.529 € | 167.134 € |

The calculation of O&M costs are as follows:

| I. | capital costs / amortization |  | Curtea de Arges | Costesti |
| --- | --- | --- | --- | --- |
| I.1 | transfer station civil works |  | 25.244 | 23.953 |
| I.2 | transfer station stationary equipment |  | 15.069 | 15.069 |
| I.3 | transfer station mobile equipment |  | 0 | 0 |
| I.4 | access road civil works |  | 3.339 | 2.825 |
| I.5 | container equipment (residues) |  | 1.449 | 1.449 |
| I.6 | container equipment (other waste streams) |  | 26.464 | 13.232 |
|  | Total amortization |  | 71.565 | 56.529 |
| II. | O&M costs |  |  |  |
| II.1 | staff |  | 21.720 | 17.520 |
| II.2 | TS maintenance & insurance |  | 41.130 | 40.117 |
| II.3 | energy / water / heating (200 €/Month] |  | 2.400 | 2.400 |
| II.4 | access road maintenance |  | 0 | 0 |
| II.5 | trucks maintenance and insurance |  | 13.232 | 6.616 |
| II.6 | annual for fuel/lubricants (acc. to MP-assumption) |  | 62.400 | 31.200 |
| II.7 | drivers |  | 7.860 | 7.860 |
| II.8 | administrative costs |  | 7.044 | 4.893 |
|  | Total O&M costs |  | 155.786 | 110.605 |
| TOTAL RUNNING (ANNUAL) COSTS |  |  | 227.351 | 167.134 |

a.2) Externalities

The following table presents the externalities associated with the 2 new TS (Mixed collection) Option in the Phase-II zones of Arges County.

Table 8.3-10 Externalities associated with the 2 new TS (Mixed Collection)

| Field | Implications | Transfer (Mixed collection) | Discretional1 |
| --- | --- | --- | --- |
| Environmental Impacts | Preservation of Human health | + Proper waste transfer to trailers + Correct operation ensures clean conditions | 1 |
|  | Emissions | - Average | 0,5 |
|  | Noise | - Average | 0,5 |
| Resource Saving | Number of transfer trucks | - 3 | 0 |
|  | Collection services | + Separate waste collection every week | 1 |
|  | Fuel Consumption | + 2 TS = less travelled distance by collection trucks | 1 |
|  | Land availability | + Designated areas ensured | 1 |
| Recycling Targets | Separated streams | - NO | 0 |
|  | Household Hazardous Waste | + Yes | 1 |
|  | Biodegradable fraction | - Difficult as mixed waste streams | 0 |
|  | Recyclable fraction | - Difficult as mixed waste streams | 0 |
| Society | Acceptability | + High as service is ensured | 1 |
|  | Environmental awareness | + Very high as separation at source and at TS | 1 |
| Operational | Road traffic | + 2 TS= less traffic in roads | 1 |
|  | Applicability | + Feasible | 1 |
| Flexibility /Adaptability | Methodology | - Average | 0,5 |
|  | Capacity | + High as service is ensured | 1 |
| Sum |  |  | 11,5 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

b) 2 New Transfer Stations, Curtea de Arges & Costesti for Bring System Collection

This option is basically the same as in the previous Option, i.e. the implementation of two transfer stations.

- Transfer Station of Curtea de Arges. Located on land where the disposal site of Curtea de Arges is currently placed. The transfer station will serve the Zones 1 & 2.

- Transfer Station of Costesti. This transfer Station will be located on the fringes of Costesti City. This facility will serve the entire Zone 8 of Arges County.

The main difference between this Option and the previous one lies in the kind of waste that will be received. The waste arriving at these TS are pre-sorted as it is planned to have a bring system for collection services.

Nevertheless, the transfer stations should have a large enough capacity to manage the wastes that are expected to be handled at the facility throughout its operating life. Factors that should be considered when determining the appropriate size of a transfer facility include:

- capacity of collection vehicles using the facility

- desired number of days of storage space on tipping floor

- time required to unload collection vehicles

- number of vehicles that will use the station and their expected days and hours of arrival (design to accommodate peak requirements)

- waste sorting or processing to be accomplished at the facility

- transfer trailer capacity

- hours of station operation

- availability of transfer trailers waiting for loading

| The distances travelled, from the proposed transfer stations to the regional landfill in Albota, are presented in the next atble. Distances (Km); Curtea de Arges city; Domnesti Commune; Costesti city; Albota landfill. Curtea de Arges city; 0; 19,3; 64,7; 40,0. Domnesti Commune; 19,3; 0; 69,6; 48,5. Costesti city; 64,7; 69,6; 0; 23,1. Albota lnadfill; 40,0; 48,5; 23,1; 0. The transfer trucks will have at least the capacity of five collection trucks; thereby reducing the number of trips of each vehicle from the Zones to the Landfill. If the diversion, of biodegradable, recyclables and HHW at these facilities is taken into account, the waste for disposal will be reduced, and thus also the number of trips from the transfer stations to the landfill in Albota. | Figure 8.3-11 Location of new transfer station for zones 1,2 & 8 |
| --- | --- |

b.1) Investment Costs and O&M Costs

The considered investments for the new transfer stations are:

- Concrete platform for facilities of tipping floor and green point, incl. earth works

- Pavement for roads, incl. boundaries, accesses and road marking

- Garage, building for waste recycling and composting storage, inc. Earth works

- Building for personnel, incl. Earth works

- Container 30 mc

- Front end loader

- Car washing

- Waste water collection system, incl. earth works

- Basin for rainwater, incl. Earthworks

- Water supply, electricity, sanitary and heating services

- Weighbridge

- Well for water supply, including filter and installation

- Pump for the well

- Fence for facilities of Transfer station, composting plant and green point, incl. earth works

- Landscaping

- Container 30 mc

- Container 10 mc for HHW

- Compactors for waste

The civil works and equipment investments are as presented in the next table.

| New TS | Component | Civil Works | Equipment | Investment | O&M Costs | Amortization | Running Costs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Curtea de Arges | 649.103 € | 354.568 € | 1.003.671 € | 155.786 € | 71.565 € | 227.351 € |
| 2 | Costesti | 628.835 € | 346.863 € | 975.697 € | 110.605 € | 56.529 € | 167.134 € |

The calculation of O&M costs are as follows:

| I. | capital costs / amortization |  | Curtea de Arges | Costesti |
| --- | --- | --- | --- | --- |
| I.1 | transfer station civil works |  | 25.244 | 23.953 |
| I.2 | transfer station stationary equipment |  | 15.069 | 15.069 |
| I.3 | transfer station mobile equipment |  | 0 | 0 |
| I.4 | access road civil works |  | 3.339 | 2.825 |
| I.5 | container equipment (residues) |  | 1.449 | 1.449 |
| I.6 | container equipment (other waste streams) |  | 26.464 | 13.232 |
|  | Total amortization |  | 71.565 | 56.529 |
| II. | O&M costs |  |  |  |
| II.1 | staff |  | 21.720 | 17.520 |
| II.2 | TS maintenance & insurance |  | 41.130 | 40.117 |
| II.3 | energy / water / heating (200 €/Month] |  | 2.400 | 2.400 |
| II.4 | access road maintenance |  | 0 | 0 |
| II.5 | trucks maintenance and insurance |  | 13.232 | 6.616 |
| II.6 | annual for fuel/lubricants (acc. to MP-assumption) |  | 62.400 | 31.200 |
| II.7 | drivers |  | 7.860 | 7.860 |
| II.8 | administrative costs |  | 7.044 | 4.893 |
|  | Total O&M costs |  | 155.786 | 110.605 |
| TOTAL RUNNING (ANNUAL) COSTS |  |  | 227.351 | 167.134 |

b.2) Externalities

The following table presents the externalities associated with the 2 new TS (Bring System) Option in the Phase-II zones of Arges County.

Table 8.3-11 Externalities associated with the 2 new TS (Bring System) option

| Field | Implications | Transfer (Mixed collection) | Discretional1 |
| --- | --- | --- | --- |
| Environmental Impacts | Preservation of Human health | + Proper waste transfer to trailers + Correct operation ensures clean conditions | 1 |
|  | Emissions | - Average | 0,5 |
|  | Noise | - Average | 0,5 |
| Resource Saving | Number of transfer trucks | - 3 | 0 |
|  | Collection services | + Separate waste collection every week | 1 |
|  | Fuel Consumption | + 2 TS = less travelled distance by collection trucks | 1 |
|  | Land availability | + Designated areas ensured | 1 |
| Recycling Targets | Separated streams | - NO | 0 |
|  | Household Hazardous Waste | + Yes | 1 |
|  | Biodegradable fraction | - Difficult as mixed waste streams | 0 |
|  | Recyclable fraction | - Difficult as mixed waste streams | 0 |
| Society | Acceptability | + High as service is ensured | 1 |
|  | Environmental awareness | + Very high as separation at source and at TS | 1 |
| Operational | Road traffic | + 2 TS= less traffic in roads | 1 |
|  | Applicability | + Feasible | 1 |
| Flexibility /Adaptability | Methodology | - Average | 0,5 |
|  | Capacity | + High as service is ensured | 1 |
| Sum |  |  | 11,5 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

c) 1 New Transfer Station in Curtea de Arges & Extension of TS in Campulung (Existing)

| This option consists of utilizing the existing infrastructure of Arges County for waste transfer, in order to fill the needs of Zones 1,2 & 8. Phase-I of the IWMS for Arges County foresees a Transfer station in Campulung for waste transfer activities of the waste generated in Zones 1&2. The distance between the Transfer station Campulung and the Domnesti commune (Zone 2) is 16.4 Km. The distance from Campulung to Curtea de Arges (Zone-1) is 34 Km. Here, most experts agree that more than 16 to 25 km to a transfer station is reasonable, as any further than 25 km for collection trucks has a high economic impact on the O&M costs. | Figure 8.3-12 Location of existing station for zones 1,2 & 8 |
| --- | --- |

c.1) Investment, O&M Costs

The investments considered for the new transfer stations are:

- Concrete platform for facilities of tipping floor and green point, incl. earth works

- Pavement for roads, incl. boundaries, accesses and road marking

- Garage, building for waste recycling and composting storage, incl. Earth works

- Building for personnel, incl. Earth works

- Container 30 mc

- Front end loader

- Car washing

- Waste water collection system, incl. earth works

- Basin for rainwater, incl. Earthworks

- Water supply, electricity, sanitary and heating services

- Weighbridge

- Well for water supply, including filter and installation

- Pump for the well

- Fence for facilities of Transfer station, composting plant and green point, incl. earth works

- Landscaping

- Container 30 mc

- Container 10 mc for HHW

- Compactors for waste

The investment costs for an extension to the existing transfer Station are considered to be 25% of civil works and 75% for the equipment of the new one in Curtea de Arges. The civil works and equipment investments for this Option are as presented in the next table:

| No. | Component | Civil Works | Equipment | Investment | O&M Costs | Amortization | Running Costs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Extension Campulung | 162.276 € | 265.926 € | 428.201 € | 116.840 € | 53.674 € | 170.514 € |
| 2 | Costesti | 628.835 € | 346.863 € | 975.697 € | 110.605 € | 56.529 € | 167.134 € |

c.2) Externalities

The following table presents the externalities associated with 1 new TS (Separated collection) + Extension of TS in Campulung Option in the Phase-II zones of Arges County.

Table 8.3-12 Externalities associated with 1 new TS + Extension of TS in Campulung

| Field | Implications | Transfer (Separate collection) | Discretional1 |
| --- | --- | --- | --- |
| Environmental Impacts | Preservation of Human health | + Proper waste transfer to trailers + Correct operation ensures clean conditions | 1 |
|  | Emissions | - Average | 0,5 |
|  | Noise | - Average | 0,5 |
| Resource Saving | Number of transfer trucks | - 3 | 0 |
|  | Collection services | + Separate waste collection every week | 1 |
|  | Fuel Consumption | - 1 TS = More travelled distances from Z1 & Z2 by collection trucks | 0 |
|  | Land availability | + Designated area for TS in Costesti - Difficulties for extension in existing TS | 0 |
| Recycling Targets | Separated streams | + Yes | 1 |
|  | Household Hazardous Waste | + Yes | 1 |
|  | Biodegradable fraction | + Yes | 1 |
|  | Recyclable fraction | + Yes | 1 |
| Society | Acceptability | + High as service is ensured | 1 |
|  | Environmental awareness | + Very high as separation at source and at TS | 1 |
| Operational | Road traffic | - 1 TS= More traffic in roads close to Campulung TS - Inadequate roads in Z2 | 0 |
|  | Applicability | + Feasible | 1 |
| Flexibility /Adaptability | Methodology | - Average | 0,5 |
|  | Capacity | + Low as original design did not foreseen extensions | 0 |
| Sum |  |  | 10,5 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

#### Evaluation of Options for Waste Transfer

| Criteria |  | A | B | C |
| --- | --- | --- | --- | --- |
|  |  | 2 New TS TS-Curtea de Arges TS- Costesti (Mixed Storage) | 2 New TS TS-Curtea de Arges TS- Costesti (Separate Storage) | 1 New TS TS- Costesti TS Extension in Campulung (Separate Storage) |
| Investments |  | 1.979.368 € | 1.979.368 € | 1.403.899 € |
| Running Costs |  | 394.486 € | 394.486 € | 337.648 € |
| Externalities |  |  |  |  |
| Environmental Impacts | Preservation of Human health | 1 | 1 | 1 |
|  | Emissions | 0,5 | 0,5 | 0,5 |
|  | Noise | 0,5 | 0,5 | 0,5 |
| Resource Saving | Number of transfer trucks | 0 | 0 | 0 |
|  | Collection services | 1 | 1 | 1 |
|  | Fuel Consumption | 1 | 1 | 0 |
|  | Land availability | 1 | 1 | 0 |
| Recycling Targets | Separated streams | 0 | 0 | 1 |
|  | Household Hazardous Waste | 1 | 1 | 1 |
|  | Biodegradable fraction | 0 | 0 | 1 |
|  | Recyclable fraction | 0 | 0 | 1 |
| Society | Acceptability | 1 | 1 | 1 |
|  | Environmental awareness | 1 | 1 | 1 |
| Operational | Road traffic | 1 | 1 | 0 |
|  | Applicability | 1 | 1 | 1 |
| Flexibility /Adaptability | Methodology | 0,5 | 0,5 | 0,5 |
|  | Capacity | 1 | 1 | 0 |
|  |  | 11,5 | 11,5 | 10,5 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

The table above shows the main comparison between the 3 different waste transfer options for the Phase-II zones of Arges County. Option C presents lower investments and O&M Costs. The main reasons for these results are due to investments required being lower than for new TS.

In Option C, the main common road connecting Gap Zones 1&2 with the existing transfer station is the Dn73C which is a small road in a mountainous zone. This road would face serious congestion problems if collection trucks (30 trips a day) are circulating towards Transfer station in Campulung on Dn73C.  There is no alternate road that connects these points.

Options A & B present the preferred option with the highest external benefits to this WM stage. The economic benefits will be assessed in Section 8.3.

### Waste Transfer Configuration

The waste transfer activities include the waste loading directly from collection vehicles onto larger vehicles to be transported to final disposal.

The primary reason for using a transfer station is to reduce the cost of transporting waste to disposal facilities. Consolidating smaller loads from collection vehicles into larger transfer vehicles reduces hauling costs by enabling collection crews to spend less time traveling to and from distant disposal sites and more time collecting waste. This also reduces fuel consumption and collection vehicle maintenance costs, plus produces less overall traffic, air emissions, and road wear.

In addition, a transfer station also provides:

- An opportunity to screen waste prior to disposal.

- Flexibility in selecting waste disposal options.

- An opportunity to serve as a convenience center for public use.

The type of station that will be feasible for the current project depends on the following design variables:

- required capacity and amount of waste storage desired

- types of wastes received

- processes required to recover material from wastes or prepare it (e.g., shred or bale) for shipment

- types of collection vehicles using the facility

- types of transfer vehicles that can be accommodated at the disposal facilities

- site topography and access.

The amounts of waste to be managed by the Transfer stations in Phase-II areas of Arges County are under 100 tons per day, thus these are considered as small capacity facilities. Where typically, small to medium transfer stations are direct-discharge stations that provide no intermediate waste storage area. These stations usually have civic amenity areas for use by the general public to accompany the principal operating areas dedicated to municipal and private refuse collection trucks.

Depending on weather, site aesthetics, and environmental concerns, transfer operations of this size may be located either indoors or outdoors. More complex small transfer stations are usually attended during hours of operation and may include some simple waste and material processing facilities. For example, the station might include a screening area for recyclable materials.

Usually, direct-discharge stations have two operating floors. On the lower level, a compactor or open-top container is located. Station users dump wastes into hoppers connected to these containers from the top level. Smaller transfer stations used in rural areas often have a simple design and are often left unattended.

The stations, used with the drop-off collection method, consist of a series of open-top containers that are filled by station users. These containers are then emptied into a larger vehicle at the station or hauled to the disposal site and emptied. For ease of loading, a simple retaining wall will allow containers to be at a lower level so that the tops of the containers are at or slightly above ground level in the loading area.

The configuration of the transfer stations is drafted according to the following analysis.

a) Transport mode:

The principle options for the development of the solid waste transport system are firstly:

- road transport

- railway transport

- combination road/railway transport

A pre-assessment of these options allowing for the infrastructural situation of Arges County makes it fairly obvious that pure railway transport of solid waste can be excluded, since the railway network of Arges County is modest. Urban areas, like Curtea de Arges and Costesti are connected by railway network. But the central landfill site of Albota has no railway connection. A combination of railway and road transport makes no sense in this case, because either an additional transfer station (re-loading of waste from railway to road vehicles) would be needed in Pitesti or the existing railway system would have to be extended to the landfill. The subsequent analysis therefore focuses on road transport.

b) Transport of Technology

The definition of different options for the road transport therefore concentrates on:

The type of equipment to be used, such as

- roll-off (roll-on) equipment (trucks and containers)

- skip equipment (trucks and containers)

- dump trucks, and

The size of the transport equipment, such as

- articulated vehicle

- truck-trailer pair

- single truck

Pre-assessing the aforementioned options of the type of equipment, it has to be concluded that:

- Skips – available up to a maximum transport capacity of 10m³ (~ 4.0 tons) – do not improve the transport efficiency, since the transport volume per trip is not increased in comparison with the standard collection vehicles. Therefore, the use of skips will not be pursued.

- Dump trucks would require special measures at the transfer stations for loading. The basic idea that containers – placed at the transfer stations – will be autonomously filled by the collection vehicle driver and then taken by the transport trucks would be made impossible. Therefore, in the Consultant’s opinion, the use of dump trucks for waste transport is not recommended.

- Roll-off/roll-on container equipment is widely used for waste transport in Europe. The experience with this equipment is good. The capacity can vary from less than 10m³ to more than 30m³.

Therefore, the following analyses are based on the assumption of exclusive use of roll-off equipment for waste transport. The pre-assessment of the required size of transport equipment (single truck, articulated truck, truck-trailer couple) is as follows:

- Single trucks (preferably 3 axle trucks due to higher load bearing capacity and higher allowed total weight) provide very good flexibility, even in areas with poor infrastructure or at the landfills. On the other hand the transport efficiency is much lower, since only 50% … 60% of waste can be transported per trip (in comparison to truck-trailer pairs or articulated vehicles), since single trucks only transport one container and must not exceed a maximum load of 18 tons whereas an articulated truck transports much larger containers and truck trailer pairs transport two containers with a maximum load of 34.5 tons. In order to optimise the transport costs per ton, single trucks will only be used as an exception, if the daily transport capacity requirement cannot be distributed amongst larger transport vehicles.

- Articulated vehicles, despite providing sufficient transport capacity, have no flexibility at all in covering different load requirements. Furthermore, landfill sites and smaller roads are much less accessible by articulated trucks, since it is not possible to handle truck and trailer separately.

In selecting the right option, both legal requirements (traffic load, axle load, further constraints) and current road conditions (including bridges with low load bearing capacity) have to be carefully considered and respected.

Legally, the transfer/ transport system is subject to the regulations for roads and traffic currently in force (Government Ordinance No. 43/1997 as amended by Government Ordinances Nos. 79/2991, 26/2003 and 21/2005, as well as Legal Acts Nos. 413/2002 and 227/2003 implementing these ordinances):

Table 3.8-13 Maximal dimensions and weight limits for transport vehicles.

| All loads stated for paved roads (not upgraded) | max. dimensions [length/width/height] | max. total tonnage [paved roads] |
| --- | --- | --- |
| Single truck (three axles) | 12,00 m ¦ 2,55 m ¦ 4,00 m | 18 t (2-axle) ¦ 24 t (3-axle) ¦ 30 t (4-axle) |
| Trailer | 12,00 m ¦ 2,55 m ¦ 4,00 m | 16 t (2-axle) ¦ 20 t (3-axle) |
| Articulated truck | 16,50 m ¦ 2,55 m ¦ 4,00 m | 38 t |
| Road train | 18,75 m ¦ 2,55 m ¦ 4,00 m | 34 t (4-axle) |

b.1 Investment, O&M Costs for Transport Technology

| Options | Curtea de Arges TS |  |  |  |  | Costesti TS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | units | Price/Unit | Investment | O&M | Amortization | units | Price/Unit | Investment | O&M | Amortization |
| Single truck (three axles) | 2 | 132.319 | 264.638 | 83.492 | 26.464 | 1 | 132.319 | 132.319 | 45.676 | 13.232 |
| Articulated truck | 1 | 200.000 | 200.000 | 49.060 | 20.000 | 1 | 200.000 | 200.000 | 49.060 | 20.000 |

b.2 Evaluation of Transfer Technology Options

Table 8.3-14 Externalities associated to 1 new TS + Extension of TS in Campulung

| Field | Implications | Discretional1 |  |
| --- | --- | --- | --- |
|  |  | Single trucks | Articulated Trucks |
| Environmental Impacts | Preservation of Human health | 1 | 1 |
|  | Emissions | 1 | 0,5 |
|  | Noise | 0 | 0 |
| Resource Saving | Number of transfer trucks | 0,5 | 1 |
|  | Fuel Consumption | 0,5 | 0,5 |
| Recycling Targets | Separated streams | 1 | 1 |
|  | Household Hazardous Waste | 1 | 1 |
|  | Biodegradable fraction | 1 | 1 |
|  | Recyclable fraction | 1 | 1 |
| Society | Acceptability | 1 | 0 |
|  | Environmental awareness | 1 | 1 |
| Operational | Road traffic | 0,5 | 0 |
|  | Applicability | 1 | 0 |
| Flexibility /Adaptability | Methodology | 0,5 | 1 |
|  | Capacity | 0,5 | 1 |
| Sum |  | 11,5 | 10 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

Bearing in mind the specific road infrastructure in the roads from Curtea de Arges and Costesti Arges (selected Transfer Station sites) to the central Landfill of Albota (see Figure 8.2-8), the decision was made to use single trucks. That decision was based on maximum gross truck load (legal and technical load bearing capacity of Arges roads), the efficiency of the transport processes taking the resulting maximum payload into account, and the traffic conditions around Albota site.

To conclude, the most advantageous vehicle for transporting waste from transfer stations to the landfill is the truck (3 axles). This provides the necessary flexibility in covering the load requirements (20 tons per trip), and also the transport and the transfer system (by types of roll-off containers suitable for both truck and trailer) are simplified and the transport is efficient.

c) Transfer Operations

The main objective in designing a transfer station should be to facilitate efficient operations. The operating scheme should be as simple as possible; it should require a minimum of waste handling, while offering the flexibility to modify the facility when needed. Equipment and building durability are essential to ensure reliability and minimize maintenance costs. With modification, the facility should be capable of handling all types of wastes.

For the transfer operations, it is obvious that the simpler the transfer station, the lower its capital and operating costs will be on one hand, and the greater the reliability and robustness on the other. Here, the following options have to be assessed:

c.1 Direct Dump Stations. Waste is dumped directly from collection vehicles into waiting transfer trailers.

- Advantages:

  - Because little hydraulic equipment is used, a shutdown is unlikely.

  - Minimizes handling of wastes.

  - Relatively inexpensive construction costs.

  - Drive-through arrangement of transfer vehicles can be easily provided.

  - Higher payloads than compactor trailers.

- Disadvantages:

  - Requires larger trailers than compaction station.

  - Dropping bulky items directly into trailers can damage trailers.

  - Minimizes opportunity to recover materials.

  - Number and availability of stalls may not be adequate to allow direct dumping during peak periods.

  - Requires bi-level construction.

c.2 Pit or Platform Noncompaction Stations. Waste is dumped into a pit or onto a platform and then loaded into trailers using waste handling equipment.

- Advantages:

  - Convenient and efficient waste storage area is provided.

  - Uncompacted waste can be crushed by bulldozer in pit or on platform.

  - Top-loading trailers are less expensive than compaction trailers.

  - Peak loads can be handled easily.

  - Drive-through arrangement of transfer vehicles can be easily provided.

  - Simplicity of operation and equipment minimizes potential for station shutdown.

  - Can allow recovery of materials.

- Disadvantages:

  - Higher capital cost, compared to other alternatives, for structure and equipment.

  - Increased floor area to maintain.

  - Requires larger trailers than compaction station.

c.3 Hopper Compaction Station. Waste is unloaded from the collection truck, through a hopper, and loaded into an enclosed trailer through a compactor.

- Advantages:

  - Uses smaller trailers than non-compaction stations uncompacted.

  - Extrusion/”log” compactors can maximize payloads in lighter trailers.

  - Some compactors can be installed in a manner that eliminates the need for a separate, lower level for trailers.

- Disadvantages:

  - If compactor fails, there is no other way to load trailers.

  - Weight of ejection system and reinforced trailer reduces legal payload.

  - Capital costs are higher for compaction trailers.

  - Compactor capacity may not be adequate for peak inflow.

  - Cost to operate and maintain compactors may be high.

c.4 Push Pit Compaction Station. Waste is unloaded from the collection truck into a push pit, and then loaded into an enclosed trailer through a compactor.

- Advantages:

  - Pit provides waste storage during peak periods.

  - Increased opportunity for recovery of materials.

  - All advantages of hopper compaction stations.

- Disadvantages:

  - Capital costs for pit equipment are significant.

  - All other disadvantages of hopper compaction stations.

c.5 Investment, O&M Costs for Transport Operations

| Options | Curtea de Arges TS & Costesti TS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Civil Works | Equipment | Investment | O&M | Amortization | Total |
| C.1 Direct Dump Station | 779.239 | 27.011 | 806.249 | 11.275 | 35.028 | 46.304 |
| C.2 Pit or Platform Noncompaction Stations | 769.239 | 27.011 | 796.249 | 11.275 | 34.628 | 45.904 |
| C.3 Hopper Compaction Station | 784.239 | 27.011 | 811.249 | 11.275 | 35.228 | 46.504 |
| C.4 Push Pit Compaction Station | 779.239 | 84.511 | 863.749 | 12.967 | 43.242 | 56.209 |

c.6 Evaluation of Transfer Operation Options

Table 8.3-15 Externalities associated with Transfer Station Operations

| Field | Implications | Discretional1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | C.1 | C.2 | C.3 | C.4 |
| Environmental Impacts | Preservation of Human health | 1 | 1 | 1 | 1 |
|  | Emissions | 1 | 1 | 1 | 1 |
|  | Noise | 0,5 | 0,5 | 0,5 | 0,5 |
| Resource Saving | Number of transfer trucks | 0,5 | 0,5 | 0,5 | 1 |
|  | Fuel Consumption | 0,5 | 0,5 | 0,5 | 1 |
| Recycling Targets | Separated streams | 1 | 1 | 1 | 1 |
|  | Screening operations | 0 | 1 | 0 | 0 |
|  | Recyclable fraction | 1 | 1 | 1 | 1 |
| Society | Acceptability | 1 | 1 | 1 | 1 |
|  | Environmental awareness | 1 | 1 | 1 | 1 |
| Operational | Road traffic | 1 | 1 | 1 | 1 |
|  | Applicability | 1 | 1 | 1 | 0,5 |
| Flexibility /Adaptability | Methodology | 1 | 0,5 | 1 | 0 |
|  | Capacity | 0,5 | 1 | 0,5 | 0,5 |
| Sum |  | 11 | 12 | 11 | 10,5 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

Considering the general objective of a Transfer Station, Option 1 & Option 2 are the two best options, as the simpler the transfer station the lower will be its capital and operating costs on one hand, and the greater the reliability and robustness on the other. However, looking at the Investment and O&M costs, Option 2 is preferable. The implementation of pit platforms in transfer stations to carry out the transfer operation, from collection trucks to bigger units to be transport to Albota site, is the best obtained solution.

The simplicity of the pit platforms allows the pre-screening operations to identify easy recoverable sub-products along with waste that could represent a risk and/or could damage the equipment in further WM stages of the system.

### Biodegradable Waste Treatment

Targets and Principles

According to the Landfill Directive and its transposition into Romanian Law, Biodegradable waste to landfill has to be reduced to the percentage shown relative to the 1995 production of biodegradable waste (estimated 1995 production for the County – 143.635 tons): the absolute values and deadlines are:

| % | Amount (ton) | Deadline |
| --- | --- | --- |
| 75% | 107.726 | 2010 |
| 50% | 71.818 | 2013 |
| 35% | 50.272 | 2016 |

Organic waste is a major component of municipal solid waste. Most of this fraction originates from household waste but commercial, institutional and industrial waste can also contain significant proportions of organic waste e.g. market waste. Organic waste is biodegradable and can be processed in the presence of oxygen by composting or in the absence of oxygen using anaerobic digestion. Both methods produce a soil conditioner, which when prepared correctly can also be used as a valuable source of nutrients in urban agriculture. Anaerobic digestion also produces methane gas an important source of bio-energy.

The following components are part of the biodegradable waste fraction:

| - Animal manure - Cardboard rolls - Clean paper - Coffee grounds and filters - Cotton rags - Dryer and vacuum | - cleaner lint - Eggshells - Fireplace ashes - Fruits and vegetables - Grass clippings - Hair and fur | - Shredded newspaper - Tea bags - Wood chips - Wool rags - Yard trimmings | - Hay and straw - Houseplants - Leaves - Nut shells - Sawdust |
| --- | --- | --- | --- |

In this section three options for Biodegradable waste management will be compared, within the frame of the Integrated Waste Management Project of Arges County, Phase-II. The selected options to be analyzed are according to the previous WM stages in the area and the main objective is the achievement of the above mentioned targets. Hence, no over expensive and sophisticated methods will be proposed, since at this stage of Romanian WM, aerobic processes are suitable.

The 3 main options to be analyzed are:

A. 2 CP in Curtea de Arges & Costesti. The option addresses individual composting in rural areas & wind row composting of green waste from parks from urban and rural areas

B. 2 CP in Curtea de Arges & Costesti for central composting. The option addresses individual composting in rural areas & wind row composting of biodegradable waste from urban and rural areas

C. 1 CP in Costesti and Extension of CP-Campulung for central composting. The option addresses individual composting in rural areas & wind row composting of biodegradable waste from urban and rural areas

a) 2 New  Composting Plants ( Mixed Waste Collection)

This option consists of the implementation of aerobic treatment processes for the biodegradable waste fraction. There will be three main components:

  - Composting plants in Curtea de Arges and Costesti. The process will be windrow composting for biodegradable fractions from urban areas and green waste from rural areas,

| Composting is a biological process in which micro-organisms convert biodegradable organic matter into a stabilised residue know as compost. The process uses oxygen drawn from the air and produces carbon dioxide and water vapour as by-products. As the mixed waste collection, the majority of biodegradable municipal solid waste compost consists of garden type waste collected at civic amenity sites, the remainder being source-segregated kerbside-collected garden waste or garden waste. The biodegradable waste feedstock is delivered to a reception area, where it is shredded into finer particle sizes to speed up the composting process. The shredded waste is then commonly formed into windrows of 1.5 to 3 metres in height for composting. | Figure 8.3-13 Windrow composting |
| --- | --- |

It is foreseen that the windrow composting plants be erected at the transfer stations in Curtea de Arges and Costesti. The first will process the biodegradable waste from Zones 1 & 2, and the second the biodegradable waste from Zone 8. The location of these composting plants at those facilities conform with economic and operational reason, as the use of resources are maximized with the implementation of multi-functional facilities such as transfer stations, equipped with composting plants.

  - Individual composting in rural areas. The process will be an aerobic composting process at each individual household for biodegradable fractions from rural areas

| Backyard composting is the implementation of the composting process on a small scale by householders. The main inputs to this process are degradable waste streams from domestic solid waste, and waste from gardens and green areas. The composting process of organic materials is performed in piles or containers, located on each property. We propose implementing this measure in the rural areas of Zones 1,2 & 8 of Arges County. The main targets are the individual households in this area, which represent 25% of the total population of the Zones . | Figure 8.3-14 Individual Composting devices |
| --- | --- |

According to site visits to these areas in Zones 1,2 & 8 of Arges County, a certain part of the population has been carrying out this practice for some time. However, the most urgent issue is to minimize the burn and dump practices of this fraction of waste. The Figure 8.3-14 presents the most common methods used for backyard composting. These methods are economic and they have proven their efficiency for individual composting.

  - Improving measures for Composting Plants in Phase-I areas (Campulung + Albota)

This measure results from the insufficient capacity of treatmentat the current composting plants in Campulung and Albota. These facilities were originally designed to treat only organic waste from commerces, markets and green waste from gardens and parks, the original design does not include the organic fraction from households in urban areas. Therefore, the amounts being sent for disposal are higher than those stipulated by the national targets.

The proposed measure of improvement is the conversion of the original static windrow composting process, into an aereated windrow composting process. The improvement of the process will treat the required amount of organic waste and significantly reduce the disposed material, thus the County targets in this matter can be fulfilled.

a.1) Investment Costs

The considered investments for the new composting plants are:

- Concrete platform for the facilities of composting plant

- Individual composting devices

- Containers 30 mc for compost

- Shredder for green waste

- Front end loader

- Water pump & balance

- Compost sieve

- Drainage system for leachates

- Lump sum for improvement of composting processes in Albota and Campulung

The civil works and equipment investments for this Option are as presented in the next table:

| New CP | Component | Civil Works | Equipment | Investment |
| --- | --- | --- | --- | --- |
| 1 | Curtea de Arges | 351.364 € | 520.563 € | 871.927 € |
| 2 | Costesti | 328.218 € | 572.374 € | 900.592 € |

a.2) O&M Costs

The calculation of O&M costs are as follows:

| I. | capital costs / amortization |  | Curtea de Arges | Costesti |
| --- | --- | --- | --- | --- |
| I.1 | composting plant civil works |  | 14.055 | 13.129 |
| I.2 | composting plant stationary equipment |  | 22.851 | 25.729 |
| I.3 | composting plant mobile equipment |  | 10.925 | 10.925 |
|  | Total amortization |  | 47.830 | 49.783 |
| II. | O&M costs |  |  |  |
| II.1 | staff |  | 13.320 | 13.320 |
| II.2 | CP maintenance & insurance |  | 37.315 | 38.774 |
| II.3 | energy / water / heating (200 €/Month] |  | 2.400 | 2.400 |
| II.4 | access road maintenance |  | 0 | 0 |
| II.5 | trucks maintenance and insurance |  | 10.465 | 10.465 |
| II.6 | annual for fuel/lubricants (acc. to MP-assumption) |  | 31.200 | 31.200 |
| II.7 | drivers |  | 8.400 | 8.400 |
| II.8 | administrative costs |  | 5.155 | 5.228 |
|  | Total O&M costs |  | 108.255 | 109.786 |
| TOTAL RUNNING (ANNUAL) COSTS |  |  | 156.085 | 159.569 |

a.3) Externalities

The following table presents the externalities for 2 new CPs (Mixed waste collection) with individual composting and windrow composting for green waste.

Table 8.3-16 Externalities associated associated with 2 New CPs (Mixed waste collection)

| Field | Implications | Composting (Mixed collection) | Discretional1 |
| --- | --- | --- | --- |
| Environmental Impacts | Preservation of Human health | + Correct operation ensures clean conditions | 1 |
|  | Emissions | - Average | 0,5 |
|  | Odours | - Average | 0,5 |
| Resource Saving | Number of transfer trucks | - 2 | 0,5 |
|  | Required equipment | - Shredder, end loader | 0,5 |
|  | Human resources | + 2 persons | 1 |
|  | Land availability | + Designated area for TS in Curtea de Arges and Costesti | 1 |
|  | Revenues | + Possible as quality of output | 1 |
| Recycling Targets | Landfill diversion | - Difficult, high efficiency required to reach targets | 0 |
|  | Landfill of biodegradable | - Household organic waste to landfill | 0 |
|  | Output | + well quality as per purer input | 1 |
| Society | Acceptability | - Average due to odours | 0,5 |
|  | Environmental awareness | - Very low as NO separation at source | 0 |
| Operational | Road traffic | - Average due to low number of trucks needed | 0,5 |
|  | Applicability | + Feasible | 1 |
| Flexibility /Adaptability | Methodology | - Average as process can be improved | 0,5 |
|  | Capacity | + Feasible | 1 |
| Sum |  |  | 10,5 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

b) Two New  Composting Plants (bring System)

The second option, for the biodegradable waste fraction, involves the  implementation of windrows composting for the entire biodegradable waste fraction, from Zones 1,2 & 8 of Arges County. The main components are:

- Composting plants in Curtea de Arges and Costesti. The process will be windrow composting for all biodegradable fractions from urban areas and rural areas (except household from rural areas). The windrow method is suited to large volumes of compost. These piles are generally turned to improve porosity and oxygen content, mix in or remove moisture, and redistribute cooler and hotter portions of the pile. Windrow composting is the most commonly used of farm scale composting methods. Process control parameters include the initial ratios of carbon and nitrogen rich materials, the amount of bulking agents added to assure air porosity, the pile size, moisture content, and turning frequency.

- Individual composting in rural areas. The process will be an aerobic composting process at every individual household for biodegradable fractions from rural areas (see above)

- Improvement measures for composting plants in phase-I areas, see above.

b.1) Investment Costs

The investments considered for the new transfer stations are:

- Concrete platform for facilities of  composting plant

- Individual composting devices

- Containers 30 mc for compost

- Shredder for green waste

- Front end loader

- Water pump & balance

- Compost sieve

- Drainage system for leachates

- Lump sum for improvement of composting processes in Albota and Campulung

The civil works and equipment investments for this Option are as presented in the next table.

| New CP | Component | Civil Works | Equipment | Investment |
| --- | --- | --- | --- | --- |
| 1 | Curtea de Arges | 429.514 € | 521.643 € | 951.157 € |
| 2 | Costesti | 371.651 € | 572.682 € | 944.333 € |

b.2) O&M Costs

The calculation of O&M costs are as follows:

| I. | capital costs / amortization |  | Curtea de Arges | Costesti |
| --- | --- | --- | --- | --- |
| I.1 | composting plant civil works |  | 17.181 | 14.866 |
| I.2 | composting plant stationary equipment |  | 22.911 | 25.746 |
| I.3 | composting plant mobile equipment |  | 10.925 | 10.925 |
|  | Total amortization |  | 51.016 | 51.537 |
| II. | O&M costs |  |  |  |
| II.1 | staff |  | 13.320 | 13.320 |
| II.2 | CP maintenance & insurance |  | 40.425 | 40.148 |
| II.3 | energy / water / heating (200 €/Month] |  | 2.400 | 2.400 |
| II.4 | access road maintenance |  | 0 | 0 |
| II.5 | trucks maintenance and insurance |  | 10.465 | 10.465 |
| II.6 | annual for fuel/lubricants (acc. to MP-assumption) |  | 31.200 | 31.200 |
| II.7 | drivers |  | 8.400 | 8.400 |
| II.8 | administrative costs |  | 5.310 | 5.297 |
|  | Total O&M costs |  | 111.520 | 111.229 |
| TOTAL RUNNING (ANNUAL) COSTS |  |  | 162.536 | 162.767 |

b.3) Externalities

The following table presents the externalities associated with 2 new CPs (Bring System waste collection) with individual composting and windrow composting for green waste.

Table 8.3-17 Externalities associated with 2 New CP (Bring System waste collection)

| Field | Implications | Composting (bring System collection) | Discretional1 |
| --- | --- | --- | --- |
| Environmental Impacts | Preservation of Human health | + Correct operation ensures clean conditions | 1 |
|  | Emissions | - Average | 0,5 |
|  | Odours | - Average | 0,5 |
| Resource Saving | Number of transfer trucks | - 3 | 0,5 |
|  | Required equipment | - Shredder, end loader | 0,5 |
|  | Human resources | - 5 persons | 0 |
|  | Land availability | + Designated area for TS in Curtea de Arges and Costesti | 1 |
|  | Revenues | + Possible as quality of output | 1 |
| Recycling Targets | Landfill diversion | + Feasible, increasing efficiency required to reach targets | 1 |
|  | Landfill of biodegradable | + Reduced as pre-sorted at source | 1 |
|  | Output | + well quality as per purer input | 1 |
| Society | Acceptability | - Average due to odours | 0,5 |
|  | Environmental awareness | + Very high as separation at source | 1 |
| Operational | Road traffic | - Average due to low number of trucks needed | 0,5 |
|  | Applicability | + Feasible | 1 |
| Flexibility /Adaptability | Methodology | - Average as process can be improved | 0,5 |
|  | Capacity | + Feasible | 1 |
| Sum |  |  | 12,5 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

c) One New  Composting Plant & Extension of Campulung CP (Separate Collection)

The third option, for biodegradable waste, consists of the implementation of windrows composting for the entire biodegradable waste fraction, from Zones 1,2 & 8 of Arges County. The main components are:

- One new CP in Costesti and the extension of the Campulung CP. The process will be windrow composting for all biodegradable fractions from urban areas and rural areas (except household from rural areas). The windrow method is suited to producing large volumes of compost. These piles are generally turned to improve porosity and oxygen content, mix in or remove moisture, and redistribute cooler and hotter portions of the pile. Windrow composting is the most commonly used of farm scale composting methods. Process control parameters include the initial ratios of carbon and nitrogen rich materials, the amount of bulking agents added to assure air porosity, the pile size, moisture content, and turning frequency.

- Individual composting in rural areas. The process will be an aerobic composting process at each individual household for biodegradable fractions from rural areas (see above)

- Improvement measures for composting plants in phase-I areas, see above.

c.1) Investment Costs

The investments considered for the new transfer stations are:

- Concrete platform for facilities of  the composting plant

- Individual composting devices

- Containers 30 mc for compost

- Shredder for green waste

- Front end loader

- Water pump & balance

- Compost sieve

- Drainage system for leachates

- Lump sum for improvement of composting processes in Albota and Campulung

The civil works and equipment investments for this Option are as presented in the next table.

| CP | Component | Civil Works | Equipment | Investment |
| --- | --- | --- | --- | --- |
| 1 | Extension of Campulung CP | 107.379 € | 391.232 € | 498.611 € |
| 2 | Costesti | 371.651 € | 572.682 € | 944.333 € |

c.2) O&M Costs

The calculation of O&M costs are as follows:

| CP | Component | O&M Costs | Amortization | Running Costs |
| --- | --- | --- | --- | --- |
| 1 | Extension of Campulung CP | 83.640 € | 38.262 € | 121.902 € |
| 2 | Costesti | 111.229 € | 51.537 € | 162.767 € |

c.3) Externalities

The following table presents the externalities associated with 1 new CP + 1 Extension (separate waste collection) with individual composting and windrow composting for green waste.

Table 8.3-18 Externalities associated with 1 New CP + 1 Campulung Extension (Separate waste collection)

| Field | Implications | Composting (Separate collection) | Discretional1 |
| --- | --- | --- | --- |
| Environmental Impacts | Preservation of Human health | + Correct operation ensures clean conditions | 1 |
|  | Emissions | - Average | 0,5 |
|  | Odours | - Average | 0,5 |
| Resource Saving | Number of transfer trucks | - 2 | 0,5 |
|  | Required equipment | - Shredder, end loader | 0,5 |
|  | Human resources | - 2 persons | 0,5 |
|  | Land availability | + Designated area for TS in Costesti - No foreseen extension in Campulung | 0 |
|  | Revenues | + Possible as quality of output | 1 |
| Recycling Targets | Landfill diversion | + Feasible, increasing efficiency required to reach targets | 1 |
|  | Landfill of biodegradable | + Reduced as pre-sorted at source | 1 |
|  | Output | + well quality as per purer input | 1 |
| Society | Acceptability | - Average due to odours | 0,5 |
|  | Environmental awareness | + Very high as separation at source | 1 |
| Operational | Road traffic | - Average due to low number of trucks needed | 0,5 |
|  | Applicability | + Feasible | 1 |
| Flexibility /Adaptability | Methodology | - Average | 0,5 |
|  | Capacity | - Low, as only one can be improved | 0 |
| Sum |  |  | 11 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

d) Two New  Composting Plants (Mixed Domiciliary&Communal collection)

This option is basically the same as Option B which essentially involves the implementation of 2 new composting plants in Zone 1-Curtea de Arges and Zone 8-Costesti. (See Option B above)

d.1) Investment Costs

See Option B (Section B.1)

d.2) O&M Costs

See Option B (Section B.2)

d.3) Externalities

See Option B (Section B.3)

#### Evaluation of Options for Biodegradable Treatment

| Criteria |  | A | B | C | D |
| --- | --- | --- | --- | --- | --- |
|  |  | 2 New CP TS-Curtea de Arges TS- Costesti (Mixed Storage) | 2 New CP TS-Curtea de Arges TS- Costesti (Separate Storage) | 1 New CP CP- Costesti CP Extension in Campulung (Separate Storage) | 2 New CP TS-Curtea de Arges TS- Costesti (Separate Storage) |
| Investments |  | 1.772.519 € | 1.895.490 € | 1.442.944 € | 1.895.490 € |
| Running Costs |  | 315.654 € | 325.303 € | 284.669 € | 325.303 € |
| Externalities |  |  |  |  |  |
| Environmental Impacts | Preservation of Human health | 1 | 1 | 1 | 1 |
|  | Emissions | 0,5 | 0,5 | 0,5 | 0,5 |
|  | Odours | 0,5 | 0,5 | 0,5 | 0,5 |
| Resource Saving | Number of transfer trucks | 0,5 | 0,5 | 0,5 | 0,5 |
|  | Required equipment | 0,5 | 0,5 | 0,5 | 0,5 |
|  | Human resources | 1 | 0 | 0,5 | 0 |
|  | Land availability | 1 | 1 | 0 | 1 |
|  | Revenues | 1 | 1 | 1 | 1 |
| Recycling Targets | Landfill diversion | 0 | 1 | 1 | 1 |
|  | Landfill of biodegradable | 0 | 1 | 1 | 1 |
|  | Output | 1 | 1 | 1 | 1 |
| Society | Acceptability | 0,5 | 0,5 | 0,5 | 0,5 |
|  | Environmental awareness | 0 | 1 | 1 | 1 |
| Operational | Road traffic | 0,5 | 0,5 | 0,5 | 0,5 |
|  | Applicability | 1 | 1 | 1 | 1 |
| Flexibility /Adaptability | Methodology | 0,5 | 0,5 | 0,5 | 0,5 |
|  | Capacity | 1 | 1 | 0 | 1 |
|  |  | 10,5 | 12,5 | 11 | 12,5 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

The table above shows the main comparison between the 4 different biodegradable waste treatment options for the Phase-II zones of Arges County. The 4 options have similar investments and O&M Costs per year. The differences are due to the amount of input for the processes and due to type of investment, i.e. new plant and/or extension of existing infrastructure.

With regards to the external benefits; the  Options B&D have the highest external benefits.

### Waste Sorting

Targets and Principles

Waste sorting or selective sorting is a sorting method used to separate waste on the basis of its composition. It is carried out by individuals and industry and then more precisely in specialised sorting centres. Waste reclamation is dependent on sorting. Among other things, it is used to separate recyclable waste from other waste.

Packaging waste is generated both by the household and the industrial/commercial/institutional sectors. The Table 8.2-14 presents the minimal amounts of recovery material required in Arges County to achieve the above mentioned targets.

Table 8.3-19 Minimal amounts of PW to be recovered or recycled in Arges County

| Year | Households (ton) |  |  |  |  | Commerce & Institutions (ton) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Paper | Glass | Metal | Plastic | Wood | Paper | Glass | Metal | Plastic | Wood |
| 2010 | 8.032 | 7.791 | 3.063 | 9.870 | 4.084 | 5.355 | 4.946 | 2.042 | 6.580 | 2.723 |
| 2011 | 8.434 | 8.180 | 3.216 | 10.364 | 4.288 | 5.623 | 5.194 | 2.144 | 6.909 | 2.859 |
| 2012 | 8.856 | 8.589 | 3.377 | 10.882 | 4.503 | 5.904 | 5.453 | 2.251 | 7.255 | 3.002 |
| 2013 | 9.298 | 9.019 | 3.546 | 11.426 | 4.728 | 6.199 | 5.726 | 2.364 | 7.617 | 3.152 |
| 2014 | 9.763 | 9.470 | 3.723 | 11.997 | 4.964 | 6.509 | 6.012 | 2.482 | 7.998 | 3.310 |
| 2015 | 10.252 | 9.754 | 3.909 | 12.597 | 5.213 | 6.834 | 6.313 | 2.606 | 8.398 | 3.475 |
| 2016 | 10.559 | 10.046 | 4.027 | 12.975 | 5.369 | 7.039 | 6.502 | 2.685 | 8.650 | 3.579 |

It is worth mentioning that the Commerce and Institutions are under legal obligation to ensure recycling/recovery of the entire quantity of packaging waste which they generate.

In Phase-I of the project, a sorting plant was implemented with a capacity of 20,000 tons/year. The current plant has space reserved for a second unit, which is meant to be implemented by Phase-II of IWMP Arges. The unit should have the same capacity and features.

The need for a sorting plant was stated in in Section 5 of the Master Plan. In order to analyze integrated waste systems with different arrangements, the following options for recovery of recyclables are considered:

A. 2 New Sorting Plants at TS in Curtea de Arges and Costesti (Mixed collection)

B. 1 Extension of the existing sorting plant at Albota landfill (Sorted Collection)

C. 1 Extension of the existing sorting plant at Albota landfill (Mixed Collection)

D. 1 Extension of the existing sorting plant at Albota landfill (Domiciliary&Communal Collection)

a) Two New Sorting Plants at Transfer Stations (Mixed Collection)

This option essentially consists of the implementation of 2 new sorting plants. These facilities are foreseen to be located at the Transfer Stations of Curtea de Arges and Costesti. The stream for input will be the residual waste (mixed waste collection services), coming from rural and urban areas of Phase-II areas.

Every sorting plant has been dimensioned for an amount of approximately 5,500 tons/year paper & cardboard and 10,400 tons/year plastic & metal. Considering seasonal variations and a reserve capacity according to the economic situation with overall 10%, the planning capacity of the plant is calculated at approximately 6,100 tons/year paper & cardboard and 11,400 tons/year plastic & metal.

Once the recyclables are separated, they must be processed into materials for sale. Processing typically includes baling for paper, steel cans, and plastic bottles, flattening or densifying for aluminum cans; granulating or perforating for plastic bottles; and crushing for glass bottles. Once processed, the materials are sold directly to specialty recyclers for reuse.

The residual fraction, which remains from these sorting operations, is disposed of in the landfill Albota.

The facility consists of the following operational units:

Figure 8.3-15 Main components of the manual Sorting Plant for Abidjan Proposal

a.1) Investment Costs

The investments considered for the new transfer stations are:

| - underfloor belt conveyer - regulator for layer level, deduction of oversized layer - bag opener (plastic/metals) - separation belt - conveyers to sorting hall and to baling press - sorting cabin with ventilation - control equipment - vibration filter (plastic/metals) - sorting belt - over belt magnetic separator | - bottle perforator - intermediate storage containers (6) - baling press 800mm - maintenance equipment - handling equipment (wheel loader, forklift) - sorting building - other costs - sum of investments - financing costs |
| --- | --- |

The civil works and equipment investments are as presented in the next table:

| New SP | Component | Civil Works | Equipment | Investment |
| --- | --- | --- | --- | --- |
| 1 | Curtea de Arges | 1.050.000 € | 746.500 € | 1.796.500 € |
| 2 | Costesti | 1.050.000 € | 746.500 € | 1.796.500 € |

a.2) O&M Costs

The calculation of O&M costs are as follows:

| I. | capital costs / amortization |  | Curtea de Arges | Costesti |
| --- | --- | --- | --- | --- |
| I.1 | underfloor belt conveyer |  | 2.500 | 2.500 |
| I.2 | regulator for layer level, deduction of oversized layer |  | 375 | 375 |
| I.3 | separation belt |  | 1.000 | 1.000 |
| I.4 | bag opener (plastic/metals) |  | 10.000 | 10.000 |
| I.5 | conveyers to sorting hall and to baling press |  | 2.438 | 2.438 |
| I.6 | sorting cabin with ventilation |  | 12.500 | 12.500 |
| I.7 | control equipment |  | 8.250 | 8.250 |
| I.8 | vibration filter (plastic/metals) |  | 3.750 | 3.750 |
| I.9 | sorting belt |  | 3.750 | 3.750 |
| I.10 | over belt magnetic separator |  | 3.750 | 3.750 |
| I.11 | underfloor belt conveyer |  | 2.500 | 2.500 |
| I.12 | bottle perforator |  | 2.000 | 2.000 |
| I.13 | intermediate storage |  | 2.400 | 2.400 |
| I.14 | baling press 800mm |  | 21.250 | 21.250 |
| I.15 | maintenance equipment |  | 1.000 | 1.000 |
| I.16 | handling equipment (wheel loader, forklift) |  | 12.000 | 12.000 |
| I.18 | sorting building |  | 24.000 | 24.000 |
| I.20 | other costs |  | 8.333 | 8.333 |
|  | financing |  | 0 | 0 |
|  | sum of amortization |  | 121.796 | 121.796 |
|  | plant insurance |  | 44.913 | 44.913 |
|  | technical inspection |  | 2000 | 2000 |
|  | Total amortization |  | 168.708 | 168.708 |
| II. | O&M costs |  |  |  |
| II.1 | staff |  | 29.280 | 29.280 |
| II.2 | maintenance |  | 143.720 | 143.720 |
| II.3 | fuel and other working materials |  | 3.000 | 3.000 |
| II.4 | energy, water, heating |  | 86.400 | 86.400 |
| II.5 | ground rents (2250m², 20 €/m²*a) |  |  |  |
| II.6 | waste disposal |  |  |  |
| II.7 | administrative costs |  | 20.000 | 20.000 |
|  |  |  | 282.400 | 282.400 |
| TOTAL RUNNING (ANNUAL) COSTS |  |  | 451.108 | 451.108 |

a.3) Externalities

The following table presents the externalities associated with 2 new sorting plants at TS in Curtea de Arges and Costesti (Mixed waste collection).

Table 8.3-20 Externalities associated with 2 New SP at TS (Mixed waste collection)

| Field | Implications | Sorting (Mixed collection) | Discretional1 |
| --- | --- | --- | --- |
| Environmental Impacts | Preservation of Human health | + Correct operation ensures clean conditions | 1 |
|  | Emissions | - Average | 0,5 |
|  | Odours | - Average | 0,5 |
| Resource Saving | Human resources | - Average | 0,5 |
|  | Land availability | + Available area in TS facilities | 1 |
|  | Revenues | + Possible as quality of output | 1 |
| Recycling Targets | Paper & Cardboard | + Feasible, increasing efficiency required to reach targets | 1 |
|  | Plastics & Metal | + Feasible, increasing efficiency required to reach targets | 1 |
|  | Efficiency | - Difficult as mixed collection | 0 |
| Society | Acceptability | - Average due to noise | 0,5 |
|  | Environmental awareness | - Low as mixed collection | 0 |
| Operational | Technology | + Feasible as It is manual separation | 1 |
|  | Applicability | + Feasible | 1 |
| Flexibility /Adaptability | Methodology | - Low | 0 |
|  | Capacity | + high as second line can be added | 1 |
| Sum |  |  | 10 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

b) Extension of Sorting Plant at Albota Landfill (Separate Collection-Bring System)

his option basically consists of the implementation of 1 extension of the sorting plant at the Albota facilities with a capacity of 20,000 tons/year. The stream for input will be the residual waste (Separate waste collection services through Bring System), coming from rural and urban areas of Phase-II areas.

The sorting plant has been dimensioned for an amount of approximately 5,500 tons/year Paper & cardboard and 10,400 tons/year plastic & metal. Considering seasonal variations and a reserve capacity according to the economic situation with overall 10%, the planning capacity of the plant is calculated for approximately 6,100 tons/year paper & cardboard and 11,400 tons/year plastic & metal.

Once the recyclables are separated, they must be processed into materials for sale. Processing typically includes baling for paper, steel cans, and plastic bottles, flattening or densifying for aluminum cans; granulating or perforating for plastic bottles; and crushing for glass bottles. Once processed, the materials are sold directly to specialty recyclers for reuse.

The residual fraction, which remains from these sorting operations, is disposed of in the landfill Albota.

The facility consists of the following operational units:

a)	Operational unit 1: delivery, storage and feeding

b)	Operational unit 2: manual pre-sorting

c)	Operational unit 3: sieving

d)	Operational unit 4: manual sorting

e)	Operational unit 5: magnetic separation

f)	Operational unit 6: baling press

Figure 8.3-16 Sorting plant in Albota landfill facilities

b.1) Investment Costs

The civil works and equipment investments are as presented in the next table.

| New SP | Component | Civil Works | Equipment | Investment |
| --- | --- | --- | --- | --- |
| 1 | Albota Extension | 0 € | 120.000 € | 120.000 € |

b.2) O&M Costs

The O&M costs are as follows:

| Item | Description of works | TOTAL COST (Euro) |  |  |
| --- | --- | --- | --- | --- |
|  |  | O&M | Amortization | TOTAL |
| 1 | Extension of Albota Sorting Plant (20.000 tons/y) | 141.200 € | 10.909 € | 152.109 € |

b.3) Externalities

The following table presents the externalities associated with the Extension of the Sorting Plant at Albota facilities (Separate waste collection).

Table 8.3-21 Externalities Associated with the Extension of Albota Sorting Plant Option (Separate waste collection)

| Field | Implications | Sorting (Mixed collection) | Discretional1 |
| --- | --- | --- | --- |
| Environmental Impacts | Preservation of Human health | + Correct operation ensures clean conditions | 1 |
|  | Emissions | - Average | 0,5 |
|  | Odours | - Average | 0,5 |
| Resource Saving | Human resources | - Average | 0,5 |
|  | Land availability | + Available area in Albota facilities | 1 |
|  | Revenues | + Possible as quality of output | 1 |
| Recycling Targets | Paper & Cardboard | + Feasible, increasing efficiency required to reach targets | 1 |
|  | Plastics & Metal | + Feasible, increasing efficiency required to reach targets | 1 |
|  | Efficiency | + Feasible as separate collection | 1 |
| Society | Acceptability | - Average due to noise | 0,5 |
|  | Environmental awareness | + High as separate collection | 1 |
| Operational | Technology | + Feasible as It is manual separation | 1 |
|  | Applicability | + Feasible | 1 |
| Flexibility /Adaptability | Methodology | - Low | 0 |
|  | Capacity | - Low | 0 |
| Sum |  |  | 11 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

c) Extension of Sorting Plant at Albota Landfill (Separate Collection)

This option is basically the same as Option B, which  essentially involves implementing  the extension of the Sorting plant at the Albota facilities with a capacity of 20.000 tons/year. The stream for input will be the residual waste (Separate waste collection services), coming from rural and urban areas of Phase-II areas.

The sorting plant has been dimensioned for an amount of approximately 5,500 tons/year paper & cardboard and 10,400 tons/year plastic & metal. Considering seasonal variations and a reserve capacity according to the economic situation with overall 10%, the planning capacity of the plant is calculated at approximately 6,100 tons/year paper & cardboard and 11,400 tons/year plastic & metal (See Option B above)

c.1) Investment Costs

See Option B (Section B.1)

c.2) O&M Costs

See Option B (Section B.2)

c.3) Externalities

See Option B (Section B.3)

d) Extension of the existing sorting plant at Albota landfill (Domiciliary&Communal Collection)

Like Option C, this option is basically the same as Option B, which consists  of the implementation of the extension of the sorting plant at the Albota facilities with a capacity of 20,000 tons/year. The stream for input will be the residual waste (Separate waste collection services), coming from the rural and urban areas of Phase-II areas.

The sorting plant has been dimensioned for an amount of approximately 5,500 tons/year paper & cardboard and 10,400 tons/year plastic & metal. Considering seasonal variations and a reserve capacity according to the economic situation with overall 10%, the planning capacity of the plant is calculated at approximately 6,100 tons/year paper & cardboard and 11,400 tons/year plastic & metal (See Option B above)

d.1) Investment Costs

See Option B (Section B.1)

d.2) O&M Costs

See Option B (Section B.2)

d.3) Externalities

See Option B (Section B.3)

#### Evaluation of Options for Waste Sorting

| Criteria |  | A | B | C | D |
| --- | --- | --- | --- | --- | --- |
|  |  | 2 New SP TS-Curtea de Arges TS- Costesti (Mixed Storage) | 1 Extension of SP in Albota (Separate Storage-Bring System) | 1 Extension of SP in Albota (Separate Storage) | 1 Extension of SP in Albota (Dom&Comm collection) |
| Investments |  | 3.593.000 € | 120.000 € | 120.000 € | 120.000 € |
| Running Costs |  | 902.217 € | 152.109 € | 152.109 € | 152.109 € |
| Externalities |  |  |  |  |  |
| Environmental Impacts | Preservation of Human health | 1 | 1 | 1 | 1 |
|  | Emissions | 0,5 | 0,5 | 0,5 | 0,5 |
|  | Odours | 0,5 | 0,5 | 0,5 | 0,5 |
| Resource Saving | Human resources | 0,5 | 0,5 | 0,5 | 0,5 |
|  | Land availability | 1 | 1 | 1 | 1 |
|  | Revenues | 1 | 1 | 1 | 1 |
| Recycling Targets | Paper & Cardboard | 1 | 1 | 1 | 1 |
|  | Plastics & Metal | 1 | 1 | 1 | 1 |
|  | Efficiency | 0 | 1 | 1 | 1 |
| Society | Acceptability | 0,5 | 0,5 | 0,5 | 0,5 |
|  | Environmental awareness | 0 | 1 | 1 | 1 |
| Operational | Technology | 1 | 1 | 1 | 1 |
|  | Applicability | 1 | 1 | 1 | 1 |
| Flexibility /Adaptability | Methodology | 0 | 0 | 0 | 0 |
|  | Capacity | 1 | 0 | 0 | 0 |
|  |  | 10 | 11 | 11 | 11 |

1 Note: The discretional values are awarded as: Positive value=1; Negligible/neutral= 0,5; Negative value= 0

The table above shows the main comparison between the 4 different waste sorting options for the Phase-II zones of Arges County. Option A has higher investment costs as they are 2 new complete sorting plants, and the rest are only extensions of the existing infrastructure. This difference follows the same trend for O&M costs.

With regards to the external benefits, options B, C & D have almost the same benefits.

### Final Disposal

| As presented and concluded in the Section 5.2.6 of the Master Plan, the capacity of Cell-1 in Albota Landfill will have expired by 2014. Therefore, the extension of the Albota landfill capacity, through Cell-2, has been addressed as the most suitable option for final disposal in Arges County. The capacity of this Cell will be around 1.1 Mio cubic meters. The location of this Cell-2 will be overlapping the north-west part of Cell-1. The Figure 8.2-14 presents the general layout of Cell-2. This final disposal option will be considered for further Option analysis (Integrated analysis). The main features of this component are: - Capacity: 1,1 Mio m3 (10 years) - Location: N-W side of Cell-1 - Zones to be covered: Entire Arges County (Zones 1,2,3,4,5,6,7&8) - Type of waste: segregated municipal waste - Maximal height: 30 m - Compaction ratio: 1,2 to 1,4 ton/m3 - Daily cover ratio: 1/6 | Figure 8.2-17 Cell-2 in Albota Landfill |
| --- | --- |

a) Investment Costs and O&M Costs

The Investment and O&M costs are as follows:

| Item | Description of works | TOTAL COST (Euro) |  |  | O&M Costs |
| --- | --- | --- | --- | --- | --- |
|  |  | Main works | Plant and Machinery | TOTAL |  |
| 1 | Extension of Albota landfill | 3.148.100 | 0 | 3.148.100 | 178.337 |

## Integrated Waste Management Systems Option Analysis

The Option Analysis of Integrated WMS takes the existing system, in the Phase-I areas of the county, as a reference. The system for the rest of the county will be the variable option (4 different options).

The main condition for the analysis is:

- the achievement of the same objectives as per RWMP3 (see Annex 4.3-1)

- same amounts for waste generation, collection and composition

The Option Analysis will be based on the comparison of the NPV’s resulting from the following components combination, i.e.

    - Option 1: Current IWMS-Phase-I areas + Option 1 (IWMS-Phase-II areas)

    - Option 2: Current IWMS-Phase-I areas + Option 2 (IWMS-Phase-II areas)

    - Option 3: Current IWMS-Phase-I areas + Option 3 (IWMS-Phase-II areas)

    - Option 4: Current IWMS-Phase-I areas + Option 4 (IWMS-Phase-II areas)

The costs related to the current system are fixed, thus the interesting variables are the costs related to the Options 1, 2, 3 & 4 for the IWMS in phase-II areas. The costs represent the investment in civil works, equipment, O&M costs for every option

The Figure below describes the 4 options to be analyzed and integrated to the current IWMS of Arges-Phase-I.

| Existing System in Phase-I |  | Stage |  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bring system |  | Tempory storage |  | Domiciliary | Bring system (Platforms) | Domiciliary separated | Mixed (Domiciliary-Communal) |
| - separated waste streams: | INTEGRATION TO PHASE-I |  |  | - Mixed waste storage | - separated waste streams: | - separated waste streams: | - separated waste streams: |
| - Residual |  |  |  | - Bins of 120 L | - Residual | - Residual | - Residual |
| - Recyclables |  |  |  |  | - Mixed recyclables | - Mixed recyclables | - Mixed recyclables |
| - Organic |  |  |  |  | - Organic (only urban areas) | - Organic (only urban areas) | - Organic (only urban areas) |
| -HHW |  |  |  |  | -HHW | -HHW | -HHW |
| - Containers 1.1 m3 in platforms |  |  |  |  | - Containers 1.1 m3 in platforms | - Containers 120 L | - Containers 1.1 m3 on streets - Bins 120 L |
| Separated Collection |  | Collection |  | Mixed collection | Separated Collection | Separated Collection | Separated Collection |
|  |  |  |  | - Door-toDoor collection | - Collection to platforms | - Door-toDoor collection | - Mixture of Door-to Door & communal waste collection |
| - Frequency 1 per week |  |  |  | - Frequency 1 per week | - Frequency 1 per week/stream | - Frequency 1 per week/stream | - Frequency 1 per week/stream |
| -16 m3 & 24m3 compactor trucks |  |  |  | -16 m3 & 24m3 compactor trucks | -16 m3 & 24m3 compactor trucks | -16 m3 & 24m3 compactor trucks | -16 m3 & 24m3 compactor trucks |
|  |  |  |  | - Density 0.35 ton/m3 | - Density 0.35 ton/m3 | - Density 0.35 ton/m3 | - Density 0.35 ton/m3 |
| Transfer Station |  | Transfer |  | 2 Transfer Stations | 2 Transfer Stations | 1 Transfer Station | 2 Transfer Stations |
| - TS in Campulung |  |  |  | - TS in Curtea de Arges (18.719 ton/y) | - TS in Curtea de Arges (18.719 ton/y) | - TS in Costesti (9.870 ton/y) | - TS in Curtea de Arges (18.719 ton/y) |
|  |  |  |  | - TS in Costesti (9.870 ton/y) | - TS in Costesti (9.870 ton/y) | - Use of existing TS Campulung (extension 18.719 ton/y) | - TS in Costesti (9.870 ton/y) |
| Central Composting |  | Waste treatment | Biodegradable | Composting | Central Composting | Central Composting | Central Composting |
| - 2 Plants (Campulung + Albota) |  |  |  | - Individual composting at rural areas | - 2 Plants at TS Curtea de A =(13.621 t/y) Costesti =(5.878 t/y) | - 1 Plants at TS in Costesti (5.878 t/y) - Use of existing CP Campulung (extension 13.621 t/y)) | - 2 Plants at TS Curtea de A =(13.621 t/y) Costesti =(5.878 t/y) |
| - Individual composting at rural areas |  |  |  | - composting of garden & park waste in urban&rural areas | - Individual composting at rural areas | - Individual composting at rural areas | - Individual composting at rural areas |
| - composting of organic waste from parks and gardens |  |  |  | Curtea de A =(7.163 t/y) Costesti =(3.780 t/y) | - composting of organic waste from urban areas | - composting of organic waste from urban areas | - composting of organic waste from urban areas |
| Sorting plant at Albota |  |  | Recyclables | Sorting plant | Sorting plant | Sorting plant | Sorting plant |
| - Dry recyclables separation |  |  |  | - Dry recyclables separation | - Dry recyclables separation | - Dry recyclables separation | - Dry recyclables separation |
|  |  |  |  | - 2 New plants at TS Curtea de A =(10.000 t/y) Costesti =(10.000 t/y) | - Extension of Albota sorting plant Albota =(20.000 t/y) | - Extension of Albota sorting plant Albota =(20.000 t/y) | - Extension of Albota sorting plant Albota =(20.000 t/y) |
| Regional landfill Albota |  | Final Disposal |  | Regional landfill Albota | Regional landfill Albota | Regional landfill Albota | Regional landfill Albota |
| Cell-1 in Albota landfill |  |  |  | - Cell-2 of Albota (1,1 Mio m3) | - Cell-2 of Albota(1,1 Mio m3) | - Cell-2 of Albota(1,1 Mio m3) | - Cell-2 of Albota(1,1 Mio m3) |

The table below is a summary table comparing the main options integrated at project level, including the achievement of targets, the investment and operational costs for the main scenarios:

| Phases | Description/ main investments |  | Estimated amounts | O&M/year | Targets/Expected Results |
| --- | --- | --- | --- | --- | --- |
| Phase I (ISPA) | - Separated waste collection system (Bring system) - Closure of 4 urban landfills in Albota, Campulung, Mioveni and Topoloveni - Closure of rural dumpsites - Construction of Cell-1 at Albota landfill (750.000 m3 capacity) - One transfer station at Campulung (including composting plant + green point) - Composting plant at Albota landfill - Sorting plant at Albota landfill |  | 34,792 Mio Euro | 8,770 Mio Euro | - To provide 100% collection coverage in 5 Areas of the county - To comply with the EU Landfill Directive - To improve the separate waste collection and disposal; - To promote the recycling of packaging and bio-waste in accordance with the national and EU legislation, thus reducing the proportion of waste to be landfilled - To ensure closure/remediation of 4 non-compliant urban landfills and rural dumpsites - To optimize the waste management at county level and to improve service standards; to improve the institutional capacity of the local authorities in order to have a sound implementation of the project and subsequent operation. - To raise the awareness of the population about sound waste management and disposal practices by the introduction of publicity measures. |
| Phase II (Application Phase) | Option 1 | - Domiciliary mixed tempory astorage with bins of 120 L - Domiciliary waste collection - Closure of 1 urban landfill in Curtea de Arges - Relocation of Costesti disposal site - Construction of Cell-2 at Albota landfill (1.100.000 m3 capacity) - Two transfer stations at Curtea de Arges and Costesti (including composting plants + civic amenity areas) - Two new sorting plants at TS in Curtea de Arges and Costesti | 20,040 Mio Euro | 3,266 Mio Euro | - To comply with the EU Landfill Directive - To improve the separate waste collection and disposal; - To promote the recycling of packaging and bio-waste in accordance with the national and EU legislation, thus reducing the proportion of waste to be landfilled - To optimize the waste management at county level and to improve service standards; - to improve the institutional capacity of the local authorities in order to have a sound implementation of the project and subsequent operation. - To raise the awareness of the population about sound waste management and disposal practices by the introduction of publicity measures. - To develop an integrated waste management system at county level by taking into account economies of scale and environmental benefits; - to ensure the coverage of waste collection in the county from 75% to 100%; - develop an efficient collection and transportation scheme at county level, fully compliant with the EU Directives and Romanian legislation; - to implement a selective pre-collection system for paper, glass, organic matters and packaging waste so as to comply with the EU legislation and with the principles of maximum recycling and composting and minimum residual waste disposal; - to develop solid waste treatment facilities in order to minimise the resources usage and to decrease the quantity of waste to be disposed; this includes transfer stations, sorting plant and composting plants; - to ensure the closure of 1 urban disposal site at Curtea de Arges - to ensure the relocation and remediation of urban disposal site at Costesti - to provide safe waste disposal facilities for the whole population residing in Arges County, in compliance with the requirements of the Landfill Directive 1999/31/EC; - to promote job creation in the tertiary sector related with the collection, transport, sorting, recycling and elimination of MSW; to stimulate the market of by-products. |
|  | Option 2 | - Bring system with collection platforms - Waste collection to collection platforms - Closure of 1 urban landfill in Curtea de Arges - Relocation of Costesti disposal site - Construction of Cell-2 at Albota landfill (1.100.000 m3 capacity) - Two transfer stations at Curtea de Arges and Costesti (including composting plants + civic amenity areas) - Extension of sorting plant in Albota facilities | 18,320 Mio Euro | 2,218 Mio Euro |  |
|  | Option 3 | - Domiciliary separated tempory astorage with bins of 120 L - Domiciliary waste collection - Closure of 1 urban landfill in Curtea de Arges and - Relocation of Costesti disposal site - Construction of Cell-2 at Albota landfill (1.100.000 m3 capacity) - One transfer stations at Curtea de Arges (including composting plants + civic amenity areas) - Extension of TS in Campulung (including composting plants + civic amenity areas) - Extension of sorting plant in Albota facilities | 18,300 Mio Euro | 3,290Mio Euro |  |
|  | Option 4 | - Combination system of domiciliary and bring system (collection platforms+ communal points) - Combined waste collection (Domiciliary + bring system) - Closure of 1 urban landfill in Curtea de Arges - Relocation of Costesti disposal site - Construction of Cell-2 at Albota landfill (1.100.000 m3 capacity) - Two transfer stations at Curtea de Arges and Costesti (including composting plants + civic amenity areas) - Extension of sorting plant in Albota facilities | 18,565 Mio Euro | 2,312 Mio Euro |  |

#### Current IWMS-Phase-I areas + Option 1 (IWMS-Phase-II areas)

Option A is a combination of the existing IWMS-Phase-I and Option 1 from the table above. The main features of this Option are:

| Storage: | Mixed storage in Eurobins (See Section 8.2.1, Option A) |
| --- | --- |
| Collection: | Mixed door-to-door collection(See Section 8.2.2, Option A) |
| Transfer: | 2 New TS in C. de Arges & Costesti (See Section 8.2.3, option A) |
| Composting: | 2 New CP in C. de Arges & Costesti (See Section 8.2.4, option A) |
| Sorting: | 2 New SP in C. de Arges & Costesti (See Section 8.2.5, option A) |
| Final Disposal: | Albota Landfill (See Section 8.2.6) |

Table 8.4-1 The Investment and O&M costs of Option 1 are described below.

| No. | Component | Civil Works | Equipment | Investment | O&M Costs | Amortization | Running Costs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Mixed Waste Collection | 0 € | 1.110.460 € | 1.110.460 € | 2.025.243 € | 619.203 € | 2.644.446 € |
| 1.1 | Curtea de Arges | 0 € | 306.386 € | 306.386 € | 15.319 € | 43.769 € | 59.089 € |
| 1.2 | Domnesti | 0 € | 197.444 € | 197.444 € | 9.872 € | 28.206 € | 38.078 € |
| 1.3 | Costesti | 0 € | 501.631 € | 501.631 € | 25.082 € | 71.662 € | 96.743 € |
| 1.4 | Residue Collection Trucks | 0 € | 0 € | 0 € | 1.704.375 € | 454.565 € | 2.158.941 € |
| 1.5 | Recyclables Coll.-Trucks | 0 € | 0 € | 0 € | 0 € | 0 € | 0 € |
| 1.5 | HHW Coll. Trucks | 0 € | 105.000 € | 105.000 € | 270.595 € | 21.000 € | 291.595 € |
| 2. | Transfer Stations | 1.277.938 € | 701.430 € | 1.979.368 € | 266.391 € | 128.094 € | 394.486 € |
| 2.1.2 | Curtea de Arges | 649.103 € | 354.568 € | 1.003.671 € | 155.786 € | 71.565 € | 227.351 € |
| 2.1.3 | Costesti | 628.835 € | 346.863 € | 975.697 € | 110.605 € | 56.529 € | 167.134 € |
| 3. | Landfill | 3.071.100 € | 77.000 € | 3.148.100 € | 177.355 € | 0 € | 177.355 € |
| 3.2 | Albota Cell-2 | 3.071.100 € | 77.000 € | 3.148.100 € | 177.355 € | 0 € | 177.355 € |
| 4. | Sorting Plants | 1.050.000 € | 1.493.000 € | 3.593.000 € | 564.800 € | 337.417 € | 902.217 € |
| 4.1 | Curtea de Arges Plant | 1.050.000 € | 746.500 € | 1.796.500 € | 282.400 € | 168.708 € | 451.108 € |
| 4.2 | Costesti | 1.050.000 € | 746.500 € | 1.796.500 € | 282.400 € | 168.708 € | 451.108 € |
| 5. | Landfill Closure | 3.418.991 € | 713.850 € | 4.132.841 € | 14.396 € | 27.525 € | 41.921 € |
| 5.1 | Curtea de Arges | 2.115.563 € | 713.850 € | 2.829.413 € | 14.396 € | 27.525 € | 41.921 € |
| 5.2 | Costesti | 1.303.428 € | 0 € | 1.303.428 € | 0 € | 0 € | 0 € |
| 6. | Composting Plant | 679.582 € | 1.092.936 € | 1.772.519 € | 218.041 € | 97.613 € | 315.654 € |
| 6.1 | Curtea de Arges Plant | 351.364 € | 520.563 € | 871.927 € | 108.255 € | 47.830 € | 156.085 € |
| 6.2 | Costesti Plant | 328.218 € | 572.374 € | 900.592 € | 109.786 € | 49.783 € | 159.569 € |
|  | TOTAL: | 9.497.611 € | 5.188.677 € | 15.736.288 € | 3.266.226 € | 1.209.851 € | 4.476.077 € |
|  | TA |  | 16,52% | 2.600.000 € |  |  |  |
|  | Site Preparation |  |  | 500.000 € |  |  |  |
|  | Contingencies |  | 10,0% | 2.103.712 € |  |  |  |
|  | Total incl. Contingencies, FD, TA |  |  | 20.940.000 € |  |  |  |

#### Current IWMS-Phase-I areas + Option 2 (IWMS-Phase-II areas)

Option B is a combination of the existing IWMS-Phase-I and Option 2 from the table above. The main features of this Option are:

| Storage: | Separate storage in collection platforms (See Section 8.2.1, Option B) |
| --- | --- |
| Collection: | Sseparate door-to-door collection(See Section 8.2.2, Option B) |
| Transfer: | 2 New TS in C. de Arges & Costesti (See Section 8.2.3, Option B) |
| Composting: | 2 New CP in C. de Arges & Costesti (See Section 8.2.4, Option B) |
| Sorting: | Extension of Albota sorting plant (See Section 8.2.5, Option B) |
| Final Disposal: | Albota Landfill (See Section 8.2.6) |

Table 8.4-2 The Investment and O&M costs of Option 2 are described below.

| No. | Component | Civil Works | Equipment | Investment | O&M Costs | Amortization | Running Costs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Mixed Waste Collection | 1.560.884 € | 550.112 € | 2.110.996 € | 1.396.526 € | 449.912 € | 1.846.437 € |
| 1.1 | Curtea de Arges | 398.613 € | 150.698 € | 549.311 € | 7.535 € | 37.473 € | 45.008 € |
| 1.2 | Domnesti | 362.471 € | 92.892 € | 455.363 € | 4.645 € | 27.769 € | 32.414 € |
| 1.3 | Costesti | 799.800 € | 236.522 € | 1.036.322 € | 11.826 € | 65.781 € | 77.607 € |
| 1.4 | Residue Collection Trucks | 0 € | 0 € | 0 € | 251.303 € | 68.856 € | 320.159 € |
| 1.5 | Recyclables Coll.-Trucks | 0 € | 0 € | 0 € | 940.821 € | 236.033 € | 1.176.853 € |
| 1.5 | HHW Coll. Trucks | 0 € | 70.000 € | 70.000 € | 180.397 € | 14.000 € | 194.397 € |
| 2. | Transfer Stations | 1.277.938 € | 701.430 € | 1.979.368 € | 266.391 € | 128.094 € | 394.486 € |
| 2.1.2 | Curtea de Arges | 649.103 € | 354.568 € | 1.003.671 € | 155.786 € | 71.565 € | 227.351 € |
| 2.1.3 | Costesti | 628.835 € | 346.863 € | 975.697 € | 110.605 € | 56.529 € | 167.134 € |
| 3. | Landfill | 3.071.100 € | 77.000 € | 3.148.100 € | 177.355 € | 0 € | 177.355 € |
| 3.2 | Albota Cell-2 | 3.071.100 € | 77.000 € | 3.148.100 € | 177.355 € | 0 € | 177.355 € |
| 4. | Sorting Plants | 0 € | 120.000 € | 120.000 € | 141.200 € | 10.909 € | 152.109 € |
| 4.1 | Curtea de Arges Plant | 0 € | 120.000 € | 120.000 € | 141.200 € | 10.909 € | 152.109 € |
| 4.2 | Costesti | 0 € | 0 € | 0 € | 0 € | 0 € | 0 € |
| 5. | Landfill Closure | 3.418.991 € | 713.850 € | 4.132.841 € | 14.396 € | 27.525 € | 41.921 € |
| 5.1 | Curtea de Arges | 2.115.563 € | 713.850 € | 2.829.413 € | 14.396 € | 27.525 € | 41.921 € |
| 5.2 | Costesti | 1.303.428 € | 0 € | 1.303.428 € | 0 € | 0 € | 0 € |
| 6. | Composting Plant | 801.165 € | 1.094.325 € | 1.895.490 € | 222.749 € | 102.554 € | 325.303 € |
| 6.1 | Curtea de Arges Plant | 429.514 € | 521.643 € | 951.157 € | 111.520 € | 51.016 € | 162.536 € |
| 6.2 | Costesti Plant | 371.651 € | 572.682 € | 944.333 € | 111.229 € | 51.537 € | 162.767 € |
|  | TOTAL: | 10.130.077 € | 3.256.717 € | 13.386.794 € | 2.218.617 € | 718.993 € | 2.937.610 € |
|  | TA |  | 19,42% | 2.600.000 € |  |  |  |
|  | Site Preparation |  |  | 500.000 € |  |  |  |
|  | Contingencies |  | 10,0% | 1.833.206 € |  |  |  |
|  | Total incl. Contingencies, FD, TA |  |  | 18.320.000 € |  |  |  |

#### Current IWMS-Phase-I areas + Option 3 (IWMS-Phase-II areas)

Option C is a combination of the existing IWMS-Phase-I and the Option 3 in the table above. The main features of this Option are:

| Storage: | Separate storage in Eurobins (See Section 8.2.1, Option C) |
| --- | --- |
| Collection: | Separate door-to-door collection(See Section 8.2.2, Option C) |
| Transfer: | 1 New TS in Costesti + Extension in Campulung TS (See Section 8.2.3, Option C) |
| Composting: | 1 New CP in Costesti + Extension in Campulung CP (See Section 8.2.4, Option C) |
| Sorting: | Extension of Albota sorting plant (See Section 8.2.5, Option C) |
| Final Disposal: | Albota Landfill (See Section 8.2.6) |

Table 8.4-3 The Investment and O&M costs of Option 3 are described below.

| No. | Component | Civil Works | Equipment | Investment | O&M Costs | Amortization | Running Costs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Mixed Waste Collection | 0 € | 3.121.381 € | 3.121.381 € | 2.535.066 € | 980.869 € | 3.515.935 € |
| 1.1 | Curtea de Arges | 0 € | 919.158 € | 919.158 € | 45.958 € | 131.308 € | 177.266 € |
| 1.2 | Domnesti | 0 € | 592.332 € | 592.332 € | 29.617 € | 84.619 € | 114.235 € |
| 1.3 | Costesti | 0 € | 1.504.892 € | 1.504.892 € | 75.245 € | 214.985 € | 290.229 € |
| 1.4 | Residue Collection Trucks | 0 € | 0 € | 0 € | 538.059 € | 141.212 € | 679.271 € |
| 1.5 | Recyclables Coll.-Trucks | 0 € | 0 € | 0 € | 1.620.692 € | 391.245 € | 2.011.937 € |
| 1.5 | HHW Coll. Trucks | 0 € | 105.000 € | 105.000 € | 225.496 € | 17.500 € | 242.996 € |
| 2. | Transfer Stations | 791.110 € | 612.788 € | 1.403.899 € | 227.445 € | 110.203 € | 337.648 € |
| 2.1.2 | Curtea de Arges | 162.276 € | 265.926 € | 428.201 € | 116.840 € | 53.674 € | 170.514 € |
| 2.1.3 | Costesti | 628.835 € | 346.863 € | 975.697 € | 110.605 € | 56.529 € | 167.134 € |
| 3. | Landfill | 3.071.100 € | 77.000 € | 3.148.100 € | 177.355 € | 0 € | 177.355 € |
| 3.2 | Albota Cell-2 | 3.071.100 € | 77.000 € | 3.148.100 € | 177.355 € | 0 € | 177.355 € |
| 4. | Sorting Plants | 0 € | 120.000 € | 120.000 € | 141.200 € | 10.909 € | 152.109 € |
| 4.1 | Curtea de Arges Plant | 0 € | 120.000 € | 120.000 € | 141.200 € | 10.909 € | 152.109 € |
| 4.2 | Costesti | 0 € | 0 € | 0 € | 0 € | 0 € | 0 € |
| 5. | Landfill Closure | 3.418.991 € | 713.850 € | 4.132.841 € | 14.396 € | 27.525 € | 41.921 € |
| 5.1 | Curtea de Arges | 2.115.563 € | 713.850 € | 2.829.413 € | 14.396 € | 27.525 € | 41.921 € |
| 5.2 | Costesti | 1.303.428 € | 0 € | 1.303.428 € | 0 € | 0 € | 0 € |
| 6. | Composting Plant | 479.029 € | 963.914 € | 1.442.944 € | 194.869 € | 89.799 € | 284.669 € |
| 6.1 | Extension Campulung | 107.379 € | 391.232 € | 498.611 € | 83.640 € | 38.262 € | 121.902 € |
| 6.2 | Costesti Plant | 371.651 € | 572.682 € | 944.333 € | 111.229 € | 51.537 € | 162.767 € |
|  | TOTAL: | 7.760.230 € | 5.608.934 € | 13.369.164 € | 3.290.331 € | 1.219.305 € | 4.509.636 € |
|  | TA |  | 19,45% | 2.600.000 € |  |  |  |
|  | Site Preparation |  |  | 500.000 € |  |  |  |
|  | Contingencies |  | 10,0% | 1.830.836 € |  |  |  |
|  | Total incl. Contingencies, FD, TA |  |  | 18.300.000 € |  |  |  |

#### Current IWMS-Phase-I areas + Option 4 (IWMS-Phase-II areas)

The Option C is assembled taking the existing IWMS-Phase-I and the Option 4 in the table above. The main features of this Option are:

| Storage: | Separate storage in Eurobins (See Section 8.2.1, Option E) |
| --- | --- |
| Collection: | Separate door-to-door collection(See Section 8.2.2, Option D) |
| Transfer: | 1 New TS in Costesti + Extension in Campulung TS (See Section 8.2.3, Option D) |
| Composting: | 1 New CP in Costesti + Extension in Campulung CP (See Section 8.2.4, Option D) |
| Sorting: | Extension of Albota sorting plant (See Section 8.2.5, Option D) |
| Final Disposal: | Albota Landfill (See Section 8.2.6) |

Table 8.4-4 The Investment and O&M costs of Option 4 are described below.

| No. | Component | Civil Works | Equipment | Investment | O&M Costs | Amortization | Running Costs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Mixed Waste Collection | 624.834 € | 1.695.223 € | 2.320.057 € | 1.490.652 € | 565.557 € | 2.056.209 € |
| 1.1 | Curtea de Arges | 82.215 € | 765.466 € | 847.681 € | 114.820 € | 112.641 € | 227.461 € |
| 1.2 | Domnesti | 280.140 € | 242.724 € | 522.864 € | 36.409 € | 45.880 € | 82.289 € |
| 1.3 | Costesti | 262.479 € | 617.033 € | 879.512 € | 92.555 € | 98.647 € | 191.202 € |
| 1.4 | Residue Collection Trucks | 0 € | 0 € | 0 € | 331.855 € | 87.820 € | 419.675 € |
| 1.5 | Recyclables Coll.-Trucks | 0 € | 0 € | 0 € | 824.815 € | 213.569 € | 1.038.384 € |
| 1.5 | HHW Coll. Trucks | 0 € | 70.000 € | 70.000 € | 90.198 € | 7.000 € | 97.198 € |
| 2. | Transfer Stations | 1.277.938 € | 701.430 € | 1.979.368 € | 266.391 € | 128.094 € | 394.486 € |
| 2.1.2 | Curtea de Arges | 649.103 € | 354.568 € | 1.003.671 € | 155.786 € | 71.565 € | 227.351 € |
| 2.1.3 | Costesti | 628.835 € | 346.863 € | 975.697 € | 110.605 € | 56.529 € | 167.134 € |
| 3. | Landfill | 3.071.100 € | 77.000 € | 3.148.100 € | 177.355 € | 0 € | 177.355 € |
| 3.2 | Albota Cell-2 | 3.071.100 € | 77.000 € | 3.148.100 € | 177.355 € | 0 € | 177.355 € |
| 4. | Sorting Plants | 0 € | 120.000 € | 120.000 € | 141.200 € | 10.909 € | 152.109 € |
| 4.1 | Extension at Albota | 0 € | 120.000 € | 120.000 € | 141.200 € | 10.909 € | 152.109 € |
| 4.2 |  | 0 € | 0 € | 0 € | 0 € | 0 € | 0 € |
| 5. | Landfill Closure | 3.418.991 € | 713.850 € | 4.132.841 € | 14.396 € | 27.525 € | 41.921 € |
| 5.1 | Curtea de Arges | 2.115.563 € | 713.850 € | 2.829.413 € | 14.396 € | 27.525 € | 41.921 € |
| 5.2 | Costesti | 1.303.428 € | 0 € | 1.303.428 € | 0 € | 0 € | 0 € |
| 6. | Composting Plant | 801.165 € | 1.094.325 € | 1.895.490 € | 222.749 € | 102.554 € | 325.303 € |
| 6.1 | Curtea de Arges Plant | 429.514 € | 521.643 € | 951.157 € | 111.520 € | 51.016 € | 162.536 € |
| 6.2 | Costesti Plant | 371.651 € | 572.682 € | 944.333 € | 111.229 € | 51.537 € | 162.767 € |
|  | Sub-total: | 9.194.028 € | 4.401.828 € | 13.595.856 € | 2.312.743 € | 834.639 € | 3.147.382 € |
|  | TA |  | 19,12% | 2.600.000 € |  |  |  |
|  | Site Preparation |  |  | 500.000 € |  |  |  |
|  | Contingencies |  | 8,91% | 1.633.323 € |  |  |  |
|  | Total incl. Contingencies, FD, TA |  |  | 18.329.179 € |  |  |  |
|  | Fees, taxes, allowances |  |  | 125.880 € |  |  |  |
|  | Salaries PIU |  |  | 70.098 € |  |  |  |
|  | PIU Furniture |  |  | 10.000 € |  |  |  |
|  | Audit Costs |  |  | 30.000 € |  |  |  |
|  | TOTAL |  |  | 18.565.157 |  |  |  |

#### Evaluation of Integrated Waste Management Systems for Arges County

The comparison between the 4 different Options for IWMS in Arges County (Phase-I & Phase-II) are resumed in the following table, along with the main results of NPV analysis. (see Annex 8.4-1)

| Stage | Phase-I (Zones 3,4,5,6&7) | Phase-II (Zones 1,2&8) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Option1 | Option 2 | Option 3 | Option 4 |
| Storage | -Bring system | -Domiciliary | -Bring system | -Domiciliary | -Domiciliary -Communal |
| Collection | -Separated | -Mixed | -Separated | -Separated | -Separated |
| Transport | - 1 TS for Z3&Z4 | -1 TS for Z1&Z2 -1 TS for Z8 | -1 TS for Z1&Z2 -1 TS for Z8 | -1 Extension in Z-3 for Z1&Z2 -1 TS for Z8 | -1 TS for Z1&Z2 -1 TS for Z8 |
| Treatment | -1 CP-Z3&Z4 -Home composting | -1 CP for Z1&Z2 -1 CP for Z8 -Home composting | -1 CP for Z1&Z2 -1 CP for Z8 -Home composting | -1 Extension in Z-3 for Z1&Z2 -1 CP for Z8 -Home composting | -1 CP for Z1&Z2 -1 CP for Z8 -Home composting |
| Sorting | -1 Unit 20.000 t/y | -1 SP for Z1&Z2 -1 SP for Z8 | -1 Unit 20.000 t/y (Extension) | -1 Unit 20.000 t/y (Extension) | -1 Unit 20.000 t/y (Extension) |
| Landfill | -Cell-1 | -Cell-2 | -Cell-2 | -Cell-2 | -Cell-2 |
| Investment |  | 20.940.000 | 18.320.000 | 18.300.000 | 18.329.179 |
| O&M Costs |  | 3.266.226 | 2.218.617 | 3.290.331 | 2.312.743 |

## Financial and Economic Analysis of options

According to standard practice, the calculation of ”dynamic unit cost” is based on a present value approach, according to which the present value of the cost cash flow related to a particular option is to be divided by the present value of the corresponding financial and economic flow over a determined period of evaluation.

The calculation of the “financial dynamic unit cost” is based on and takes into account:

- The actual value of the existing assets related to the particular option in 2010; (which are in this case zero for all options);

- The investment cost related to the particular option; allocated over the period of implementation in line with the respective implementation schedule and split by main works;

- Appropriate O&M costs for each option presented in detail in the feasibility study;

- The anticipated quantities of solid waste collected in each particular option.

For Option 2, which considers that the collection of waste will be performed by building platforms and not performing door-to-door collection into the rural area, we have a different trend for collected waste in rural areas. In our opinion this approach will lead to non-collection of a part of the generated waste from rural areas because in some villages the door-to-door collection already exists and there is difficult to convince the inhabitants to bring the waste to the platform which will be located quite far from the houses (in order to respect all legal and environmental requirements).  Considering the reasoning presented above, we estimated that approximately 30% of the waste generated in rural areas will not be collected and disposed properly.

The results obtained are the following:

Table 8.5-1 FNPV and Dynamic Prime Costs for each Option

|  | FNPV (Euro) | DPC (Euro/t) |
| --- | --- | --- |
| Option 1 | 48,398,023 | 78.7 |
| Option 2 | 36,416,184 | 66.7 |
| Option 3 | 46,339,255 | 75.4 |
| Option 4 | 37,297,161 | 60.7 |

The analysis show that Option 2 and Option 4 are the most feasible one from financial point of view. The NPV for Option 2 is the lowest while the DPC for Option 4 is the lowest. Considering this close results for the 2 options we decided to perform also an economic analysis of these 2 options.

In general, the economic benefits of each option are equal for each option. The Consultant has considered the following additional elements for Option 2 and 4 for the economic analysis:

- For option 2, considering the reasoning presented above, we estimated that approximately 30% of the waste generated in rural areas will not be collected and disposed properly. For this quantity of waste we have calculated the externalities regarding the CO2 emission according to the provisions of the CBA Guide.

- We have included in the analysis also the value of land acquisition both for Option 2 and Option 4:

  - Land needs for Option 2:

  - Land needs for Option 4:

  - We have considered an average price of land of 8 Euro/m2 for both options;

- The construction of platforms and other solid waste facilities will affect the value of the land in the immediate surroundings of these facilities. We have considered also this economic additional costs of the land value decrease based on the following assumptions:

  - The value of the surrounding land will decrease by 25% (2 Euro/m2);

  - The size of the affected land represents six times the value of the land needed for the facilities.

- All the other economic benefits and costs are identical for both options so they were included in the analysis considering all benefits calculated in the economic benefits sheet of the financial model.

The results obtained are the following:

Table 8.5-2 Economic Net Present Value (ENPV) for Option 2 and 4

|  | ENPV (Euro) |
| --- | --- |
| Option 2 | 1,474,190 |
| Option 4 | 3,046,197 |

The analysis shows that the Economic NPV for Option 4 is higher than the Economic NPV for Option 2. Considering this, Option 4 is the selected option.

The detailed calculations of the Dynamic Prime Cost and Economic Net Present Value for each option are presented in Annex 2 to the CBA Report.

## Conclusions

According to the above, the option analysis was carried out in two ways, the first analysing components options at different stages of WM, i.e. storage, collection, transfer, treatment and final disposal. The investment costs for every component were set out, as along with O&M Costs.

The external benefits were also evaluated in a qualitative manner for every component at every stage. The results favoured the following components:

Storage: 	Separate storage at source through Domiciliary + Communal through collection platforms (Mixed system)

Collection: 	Separate collection through collection platforms (Bring System)

Transfer: 	2 new Transfer Station at Curtea de Arges and Costesti with separated collection

Treatment of Biodegradable: 	2 new Transfer Station at Curtea de Arges and Costesti with separated collection

Sorting: 	Extension of Albota sorting plant

In the second part of this option analysis, an option analysis for Integrated WMS was carried out in order to identify the most suitable components to be implemented in the Phase-II (Zones 1, Zone-3 & Zone-8) in Arges County.

Four different variants were assembled, considering the components analyzed in the first part of this analysis and considering the ENPV comparison as a basis for the Option differentiation.

According to the qualitative analysis performed in Section 8.3 and the Quantitative Analysis in Section 8.4, the main conclusions can be summarized in the following table.

Table 8.6-1 Results of Option Analysis

| RANKING | CRITERIA |  |  |
| --- | --- | --- | --- |
|  | External Benefits | ENPV | DPC |
| 1 | Option 4 | Option 4 | Option 4 |
| 2 | Option 2 | Option 2 | Option 2 |
| 3 | Option 3 | Option 1 | Option 3 |
| 4 | Option 1 | Option 3 | Option 1 |

According to above table results, Option 4 is chosen as the best alternative to be implemented for Arges County. This can be described as follows:

| - Mixed (Domiciliary-Communal) - separated waste streams - Residual - Mixed recyclables - Organic (only urban areas) - Containers 1.1 m3 on streets - Bins 120 L - Separated Collection - Mixture of Door-to Door & communal waste collection - Frequency 1 per week/stream - 16 m3 & 24m3 compactor trucks | - 2 Transfer Stations - TS in Curtea de Arges - TS in Costesti - Central Composting - 2 Plants at TS - Individual composting at rural areas - composting of organic waste from urban areas - Sorting plant - Dry recyclables separation - Extension of Albota sorting plant - Regional landfill Albota - Cell-2 of Albota |
| --- | --- |