1.	Summary	2

1.1	Introduction	2

1.2	Existing Situation	2

1.3	The ISWM system in arges county	3

1.3.1	Overview	3

1.3.2	Project objectives	3

1.4	Option Analysis	5

1.5	Project description	8

1.5.1	Components	8

1.5.2	Project indicators	11

1.5.3	Project costs	12

1.6	Demand analysis	13

1.6.1	Household and similar waste generation data	13

1.6.2	Population Forecast	14

1.6.3	Connection rate to sanitation services	14

1.6.4	Municipal waste generation indicator	14

1.6.5	Municipal waste projection	15

1.6.6	Special Waste Streams	16

1.7	Biodegradable waste & Recycling targets	17

1.8	Economic Analysis and Financial Analysis	19

1.9	Financing Plan	23

# Summary

## Introduction

The present document was prepared as an abstract of the Feasibility Study of Arges County. The summary is structured according to the last requirements of JASPERS, hence the current document could present) some variations, when compared with FS guidelines. The details of the presented information are found in the FS document and Annexes.

## Existing Situation

The project is located in Arges County and represents a top priority according to the National Waste Management Plan, having an important contribution to the implementation of the relevant European legislation in the field of environment in Romania.

Arges County has a surface of 6.862 km2 and a total population of 650.000 inhabitants. Administratively, Arges county comprises 3 municipalities (Campulung, Curtea de Arges & Pitesti), 4 towns (Costesti, Mioveni, Stefanesti & Topoloveni) and 95 communes. Approximately 48 % of the total population is living in urban areas. The proposed project covers the whole territory of Arges County.

The total generation of municipal solid waste in 2009 is estimated to 282.655 tonnes of which approximately 161.475 tonnes was household waste, 88.913 tonnes waste similar to household waste and approximately 32.267 tonnes waste from parks and gardens, markets and street waste. Presently, 100 % of the urban area and from 20% -25 % of the rural areas are covered by organised waste collection provided by a mix of private and public operators. Disposal of waste collected in the County is currently carried out in a number of landfills not compliant with the Landfill Directive. Not separated collection of household waste is yet being implemented.

The current WM infrastructure – in use or implemented (with ISPA/Phare investments, as appropriate) – that will be integrated in the county WM system in Arges county includes the following components:

- a County landfill located in Albota, having a total area of 4.25 ha, the total capacity of the Cell-1 of approximately 750.000 m³ and an estimated lifetime until 2014

- a 60 TPD sorting station for recyclable waste commissioned in Albota landfill to start operations in December 2010.

- 2 composting plants for green waste with 25.000 t/year capacity  in Campulung and Albota, ready to start operations in December 2010.

- a transfer station with capacity of 27.700 t/year in Campulung, ready to start operations in December 2010.

As regards disposal facilities, there is one compliant landfill located in Albota (Cell-1) and 6 non-compliant urban landfills, of which 2 are still active (Curtea de Arges & Costesti). The 2 non-compliant urban landfills have certain deadlines for ceasing their activity, no later than 2017.

The main problems in the current SWM system in Arges County are summarised below:

- low waste collection coverage (average 61%) resulting in illegal or unprotected dumping of waste resulting in risks for environmental pollution- (100 % Collection coverage in urban areas and 80% - 90% in rural areas, 2 non compliant urban landfills;

- limited waste recycling in the county

- inefficient waste management system at county level due to excessive sector fragmentation; inadequate institutional set up defining the operational aspects

- insufficient waste management infrastructure; many of the existing facilities are not compliant with the environmental legislation; wide use of depreciated equipment

- insufficient financing means needed to update or extend the waste infrastructure; insufficient application of the polluter pays principle on basis of affordability

- insufficient public awareness and communication as regards adequate waste management in the county (e.g. separate collection of waste is not an usual practice in the project area and no clear programme is in place for this purpose)

## The ISWM system in arges county

### Overview

The project has been split into 2 main Phases, the Phase-I under ISPA Measure covers Five of Eight identified zones in Arges County (73% of the total County population); the Phase-II covers the remaining three zones of Arges County. The Phase-I is totally implemented, except for the tendered procedure for operation that is currently ongoing.

The general approach for the ISWM System in Arges County comprises separated storage for solid waste, through collection platforms; the improvement of waste collection service in urban areas and the implementation of waste collection in rural areas. The solid waste is collected and delivered to transfer stations for its transport to the regional landfill in Albota.

The main components at every Phase are as follows:

Phase-I (ISPA Funds and in implementation stage):

- Separated waste collection system (Bring system)

- Closure of 4 urban landfills in Albota, Campulung, Mioveni and Topoloveni

- Closure of rural dumpsites

- Construction of Cell-1 at Albota landfill (750.000 m3 capacity)

- One transfer station at Campulung (including composting plant + green point)

- Composting plant at Albota landfill

- Composting plant at Campulung

- Sorting plant at Albota landfill

Phase-II (For Application):

- Separated waste collection system (Combined system of domiciliary + bring system)

- Closure of 2 urban landfills in Curtea de Arges and Costesti

- Construction of Cell-2 at Albota landfill (1.100.000 m3 capacity)

- Two transfer stations at Curtea de Arges and Costesti (including composting plants + civic amenity areas)

- Extension of sorting plant at Albota landfill and improvement of the composting plants from Albota and Campulung

Every transfer station comprises a weight bridge; a “civic amenity area”, which is intended to collect the separated waste streams for further recycling and special waste streams (WEEE, HHW, CW and Bulky waste); a compost facility to process biodegradable fraction of solid waste; and a tipping floor to transfer residual waste to trucks for transporting to Albota landfill.

The second element of the strategy consists in the closure of the final disposal site in Curtea de Agres (as required by ISPA), and the re-allocation and remediation of Costesti disposal site into Albota landfill Cell 2.

The Construction of the Cell-2 in the regional landfill in Albota emerges as obliged measure, as the Cell-1 will run out its capacity by 2014. After this time, there will be no place to dispose the waste generated by the entire Arges County.

### Project objectives

The general objective of the project is to develop an integrated waste management system in Arges County (for Phase-II) to enable the waste management arrangements in the Project Zones to be in compliance with the relevant EU Directives and Romanian legislation, and thus to significantly improve environmental quality and living conditions.

The specific objectives of the Project that will contribute to the general objective are to:

- Assure 100% collection coverage in Arges County;

- Develop an efficient collection and transportation scheme at county level, fully compliant with the EU Directives and Romanian legislation;

- Optimize resources and waste management at county level and improve service standards, through introduction of revised collection services and development of transfer stations;

- Provide essential landfill capacity sufficient to meet the medium term needs of the County;

- Implement separate waste collection practices and facilities for the recovery of packaging material and bio-waste, thus reducing the proportion of material requiring landfill;

- Clean up historically contaminated sites by removing/closing/remediating 2 non-compliant urban landfills;

- Improve the institutional capacity of the local authorities in order to ensure sound implementation of the project and subsequent operation of the facilities and services;

- Engage behavioural change strategies and conduct awareness raising campaigns to communicate with the public about new systems being implemented and how to adapt to them.

- Develop an integrated waste management system at county level by taking into account economies of scale and environmental benefits;

- Development of solid waste treatment facilities in order to minimise the resources usage and to decrease the quantity of waste to be disposed, i.e. Transfer stations and Composting plants;

- Promote, creation in the tertiary sector related with the collection, transport, sorting, recycling and elimination of MSW; to stimulate the market of by-products.

Moreover, the project will support the achievement of the following socio-economic objectives:

- To protect the environment and the human health in Arges county and in the neighbouring counties. This objective is related to the water and soil contamination caused by the present improper solid waste management.

- To improve the landscape in the county, which will implicitly lead to a higher population comfort and help in creating an attractive area for tourism.

- To reduce resources usage by better facilitating sorting and recycling of municipal wastes.

- To reduce the greenhouse gas emissions, due to the avoidance (or proper collection) of methane and carbon dioxide emissions, of all gas generated from decomposing waste.

## Option Analysis

The table below is a summary table comparing the main options integrated at project level, including the achievement of targets, the investment and operational costs for the main scenarios.

| Phases | Description/ main investments |  | Estimated amounts | O&M/year | Targets/Expected Results |
| --- | --- | --- | --- | --- | --- |
| Phase I (ISPA) | - Separated waste collection system (Bring system) - Closure of 4 urban landfills in Albota, Campulung, Mioveni and Topoloveni - Closure of rural dumpsites - Construction of Cell-1 at Albota landfill (750.000 m3 capacity) - One transfer station at Campulung (including composting plant + green point) - Composting plant at Albota landfill - Sorting plant at Albota landfill |  | 34,792 Mio Euro | 8,770 Mio Euro | - To provide 100% collection coverage in 5 Areas of the county - To comply with the EU Landfill Directive - To improve the separate waste collection and disposal; - To promote the recycling of packaging and bio-waste in accordance with the national and EU legislation, thus reducing the proportion of waste to be landfilled - To ensure closure/remediation of 4 non-compliant urban landfills and rural dumpsites - To optimize the waste management at county level and to improve service standards; to improve the institutional capacity of the local authorities in order to have a sound implementation of the project and subsequent operation. - To raise the awareness of the population about sound waste management and disposal practices by the introduction of publicity measures. |
| Phase II (Application Phase) | Option 1 | - Domiciliary mixed tempory astorage with bins of 120 L - Domiciliary waste collection - Closure of 1 urban landfill in Curtea de Arges - Relocation of Costesti disposal site - Construction of Cell-2 at Albota landfill (1.100.000 m3 capacity) - Two transfer stations at Curtea de Arges and Costesti (including composting plants + civic amenity areas) - Two new sorting plants at TS in Curtea de Arges and Costesti | 20,040 Mio Euro | 3,266 Mio Euro | - To comply with the EU Landfill Directive - To improve the separate waste collection and disposal; - To promote the recycling of packaging and bio-waste in accordance with the national and EU legislation, thus reducing the proportion of waste to be landfilled - To optimize the waste management at county level and to improve service standards; - to improve the institutional capacity of the local authorities in order to have a sound implementation of the project and subsequent operation. - To raise the awareness of the population about sound waste management and disposal practices by the introduction of publicity measures. - To develop an integrated waste management system at county level by taking into account economies of scale and environmental benefits; - to ensure the coverage of waste collection in the county from 75% to 100%; - develop an efficient collection and transportation scheme at county level, fully compliant with the EU Directives and Romanian legislation; - to implement a selective pre-collection system for paper, glass, organic matters and packaging waste so as to comply with the EU legislation and with the principles of maximum recycling and composting and minimum residual waste disposal; - to develop solid waste treatment facilities in order to minimise the resources usage and to decrease the quantity of waste to be disposed; this includes transfer stations, sorting plant and composting plants; - to ensure the closure of 1 urban disposal site at Curtea de Arges - to ensure the relocation and remediation of urban disposal site at Costesti - to provide safe waste disposal facilities for the whole population residing in Arges County, in compliance with the requirements of the Landfill Directive 1999/31/EC; - to promote job creation in the tertiary sector related with the collection, transport, sorting, recycling and elimination of MSW; to stimulate the market of by-products. |
|  | Option 2 | - Bring system with collection platforms - Waste collection to collection platforms - Closure of 1 urban landfill in Curtea de Arges - Relocation of Costesti disposal site - Construction of Cell-2 at Albota landfill (1.100.000 m3 capacity) - Two transfer stations at Curtea de Arges and Costesti (including composting plants + civic amenity areas) - Extension of sorting plant in Albota facilities | 18,320 Mio Euro | 2,218 Mio Euro |  |
|  | Option 3 | - Domiciliary separated tempory astorage with bins of 120 L - Domiciliary waste collection - Closure of 1 urban landfill in Curtea de Arges and - Relocation of Costesti disposal site - Construction of Cell-2 at Albota landfill (1.100.000 m3 capacity) - One transfer stations at Curtea de Arges (including composting plants + civic amenity areas) - Extension of TS in Campulung (including composting plants + civic amenity areas) - Extension of sorting plant in Albota facilities | 18,300 Mio Euro | 3,290Mio Euro |  |
|  | Option 4 | - Combination system of domiciliary and bring system (collection platforms+ communal points) - Combined waste collection (Domiciliary + bring system) - Closure of 1 urban landfill in Curtea de Arges - Relocation of Costesti disposal site - Construction of Cell-2 at Albota landfill (1.100.000 m3 capacity) - Two transfer stations at Curtea de Arges and Costesti (including composting plants + civic amenity areas) - Extension of sorting plant in Albota facilities | 18,565 Mio Euro | 2,312 Mio Euro |  |

An option analysis was carried out in order to identify the most suitable components to be implemented by the Phase-II (Zones 1, Zone-3 & Zone-8) in Arges County.  Different options for waste storage, collection, transport and treatment were analyzed and compared on basis of NPV and external benefits. The most interesting analysis was regarding the analysis of 4 different options for Phase-II integrated and working together with the existing system of Phase-I, in this way the IWMS-Arges could be compared in 4 variants, as follows:

| Stage | Phase-I (Zones 3,4,5,6&7) | Phase-II (Zones 1,2&8) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Option1 | Option 2 | Option 3 | Option 4 |
| Storage | -Bring system | -Domiciliary | -Bring system | -Domiciliary | -Domiciliary -Communal |
| Collection | -Separated | -Mixed | -Separated | -Separated | -Separated |
| Transport | - 1 TS for Z3&Z4 | -1 TS for Z1&Z2 -1 TS for Z8 | -1 TS for Z1&Z2 -1 TS for Z8 | -1 Extension in Z-3 for Z1&Z2 -1 TS for Z8 | -1 TS for Z1&Z2 -1 TS for Z8 |
| Treatment | -1 CP-Z3&Z4 -Home composting | -1 CP for Z1&Z2 -1 CP for Z8 -Home composting | -1 CP for Z1&Z2 -1 CP for Z8 -Home composting | -1 Extension in Z-3 for Z1&Z2 -1 CP for Z8 -Home composting | -1 CP for Z1&Z2 -1 CP for Z8 -Home composting |
| Sorting | -1 Unit 20.000 t/y | -1 SP for Z1&Z2 -1 SP for Z8 | -1 Unit 20.000 t/y (Extension) | -1 Unit 20.000 t/y (Extension) | -1 Unit 20.000 t/y (Extension) |
| Landfill | -Cell-1 | -Cell-2 | -Cell-2 | -Cell-2 | -Cell-2 |
| Investment |  | 20.940.000 | 18.320.000 | 18.300.000 | 18.329.179 |
| O&M Costs |  | 3.266.226 | 2.218.617 | 3.290.331 | 2.312.743 |
| NPV (base year 2010) |  | -232.260 | -211.950 | -231.768 | -213.693 |
| IRR (base year 2010) |  | 1,13% | 1,21% | 1,44% | 1,23% |

Economic and Financial Analysis of Options

According to standard practice, the calculation of ”dynamic unit cost” is based on a present value approach, according to which the present value of the cost cash flow related to a particular option is to be divided by the present value of the corresponding financial and economic flow over a determined period of evaluation.

The calculation of the “financial dynamic unit cost” is based on and takes into account:

- The actual value of the existing assets related to the particular option in 2010; (which are in this case zero for all options);

- The investment cost related to the particular option; allocated over the period of implementation in line with the respective implementation schedule and split by main works;

- Appropriate O&M costs for each option presented in detail in the feasibility study;

- The anticipated quantities of solid waste collected in each particular option.

For Option 2, which considers that the collection of waste will be performed by building platforms and not performing door-to-door collection into the rural area, we have a different trend for collected waste in rural areas. In our opinion this approach will lead to non-collection of a part of the generated waste from rural areas because in some villages the door-to-door collection already exists and there is difficult to convince the inhabitants to bring the waste to the platform which will be located quite far from the houses (in order to respect all legal and environmental requirements).  Considering the reasoning presented above, we estimated that approximately 30% of the waste generated in rural areas will not be collected and disposed properly.

The results obtained are the following:

FNPV and Dynamic Prime Costs for each Option

|  | FNPV (Euro) | DPC (Euro/t) |
| --- | --- | --- |
| Option 1 | 48,398,023 | 78.7 |
| Option 2 | 36,416,184 | 66.7 |
| Option 3 | 46,339,255 | 75.4 |
| Option 4 | 37,297,161 | 60.7 |

The analysis show that Option 2 and Option 4 are the most feasible one from financial point of view. The NPV for Option 2 is the lowest while the DPC for Option 4 is the lowest. Considering this close results for the 2 options we decided to perform also an economic analysis of these 2 options.

In general, the economic benefits of each option are equal for each option. The Consultant has considered the following additional elements for Option 2 and 4 for the economic analysis:

- For option 2, considering the reasoning presented above, we estimated that approximately 30% of the waste generated in rural areas will not be collected and disposed properly. For this quantity of waste we have calculated the externalities regarding the CO2 emission according to the provisions of the CBA Guide.

- We have included in the analysis also the value of land acquisition both for Option 2 and Option 4:

  - Land needs for Option 2:

  - Land needs for Option 4:

  - We have considered an average price of land of 8 Euro/m2 for both options;

- The construction of platforms and other solid waste facilities will affect the value of the land in the immediate surroundings of these facilities. We have considered also this economic additional costs of the land value decrease based on the following assumptions:

  - The value of the surrounding land will decrease by 25% (2 Euro/m2);

  - The size of the affected land represents six times the value of the land needed for the facilities.

- All the other economic benefits and costs are identical for both options so they were included in the analysis considering all benefits calculated in the economic benefits sheet of the financial model.

The results obtained are the following:

Economic Net Present Value (ENPV) for Option 2 and 4

|  | ENPV (Euro) |
| --- | --- |
| Option 2 | 1,474,190 |
| Option 4 | 3,046,197 |

The analysis shows that the Economic NPV for Option 4 is higher than the Economic NPV for Option 2. Considering this, Option 4 is the selected option.

The detailed calculations of the Dynamic Prime Cost and Economic Net Present Value for each option are presented in Annex 2 to the CBA Report.

According to the above, the option analysis was carried out in two ways, the first analysing components options at different stages of WM, i.e. storage, collection, transfer, treatment and final disposal. The investment costs for every component were set out, as along with O&M Costs.

The external benefits were also evaluated in a qualitative manner for every component at every stage. The results favoured the following components:

Storage: 	Separate storage at source through Domiciliary + Communal through collection platforms (Mixed system)

Collection: 	Separate collection through collection platforms (Bring System)

Transfer: 	2 new Transfer Station at Curtea de Arges and Costesti with separated collection

Treatment of Biodegradable: 	2 new Transfer Station at Curtea de Arges and Costesti with separated collection

Sorting: 	Extension of Albota sorting plant

In the second part of this option analysis, an option analysis for Integrated WMS was carried out in order to identify the most suitable components to be implemented in the Phase-II (Zones 1, Zone-3 & Zone-8) in Arges County.

Four different variants were assembled, considering the components analyzed in the first part of this analysis and considering the ENPV comparison as a basis for the Option differentiation.

According to the qualitative analysis performed in Section 8.3 and the Quantitative Analysis in Section 8.4, the main conclusions can be summarized in the following table.

Results of Option Analysis

| RANKING | CRITERIA |  |  |
| --- | --- | --- | --- |
|  | External Benefits | ENPV | DPC |
| 1 | Option 4 | Option 4 | Option 4 |
| 2 | Option 2 | Option 2 | Option 2 |
| 3 | Option 3 | Option 1 | Option 3 |
| 4 | Option 1 | Option 3 | Option 1 |

According to above table results, Option 4 is chosen as the best alternative to be implemented for Arges County. This can be described as follows:

| - Mixed (Domiciliary-Communal) - separated waste streams - Residual - Mixed recyclables - Organic (only urban areas) - Containers 1.1 m3 on streets - Bins 120 L - Separated Collection - Mixture of Door-to Door & communal waste collection - Frequency 1 per week/stream - 16 m3 & 24m3 compactor trucks | - 2 Transfer Stations - TS in Curtea de Arges - TS in Costesti - Central Composting - 2 Plants at TS - Individual composting at rural areas - composting of organic waste from urban areas - Sorting plant - Dry recyclables separation - Extension of Albota sorting plant - Regional landfill Albota - Cell-2 of Albota |
| --- | --- |

## Project description

### Components

This section presents the main features, related to the chosen Option 4, for the IWMS of Phase-II in Arges County. The long term investment of the IWMS-Arges in Phase-I&II was prepared. Having in view the long term objectives, the targets set by the RWMP and the financing resources needed, 2 investment phases are included in the County MP.

| Phases | Description/ main investments | Estimated amounts (Mio Euros) | O&M/year | Targets/Expected Results |
| --- | --- | --- | --- | --- |
| Phase I (ISPA) | - Separated waste collection system (Bring system) - Closure of 4 urban landfills in Albota, Campulung, Mioveni and Topoloveni - Closure of rural dumpsites - Construction of Cell-1 at Albota landfill (750.000 m3 capacity) - One transfer station at Campulung (including composting plant + green point) - Composting plant at Albota landfill - Sorting plant at Albota landfill | 34,792 | 8,770 | - To provide 100% collection coverage in 5 Areas of the county - To comply with the EU Landfill Directive - To improve the separate waste collection and disposal; - To promote the recycling of packaging and bio-waste in accordance with the national and EU legislation, thus reducing the proportion of waste to be landfilled - To ensure closure/remediation of 4 non-compliant urban landfills and rural dumpsites - To optimize the waste management at county level and to improve service standards; to improve the institutional capacity of the local authorities in order to have a sound implementation of the project and subsequent operation. - To raise the awareness of the population about sound waste management and disposal practices by the introduction of publicity measures. |
| Phase II (Application Phase) | - Separate temporal storage through a mixed system, including: domiciliary, collection platforms & container groups on main streets. - Separate waste collection with compactor trucks of 16 m3 and 24m3 - Implementation of 2 New Transfer Stations in Curtea de Arges and Costesti - Implementation of 2 civic amenity facilities (green points) at TS to storage recyclables and specific waste streams as CW, WEEE, Bulky Waste and HHW. - 2 new composting plants with windrow compost process at TS, aiming the organic fraction, from Households and commercial entities in the zones - 1 extension of sorting plant at Albota facility (20.000 tons/year) - Cell-2 construction for final disposal, to serve all County (Phase-I & Phase-II areas) | 18,565 | 2,312 | - To comply with the EU Landfill Directive - To improve the separate waste collection and disposal; - To promote the recycling of packaging and bio-waste in accordance with the national and EU legislation, thus reducing the proportion of waste to be landfilled - To optimize the waste management at county level and to improve service standards; - to improve the institutional capacity of the local authorities in order to have a sound implementation of the project and subsequent operation. - To raise the awareness of the population about sound waste management and disposal practices by the introduction of publicity measures. - To develop an integrated waste management system at county level by taking into account economies of scale and environmental benefits; - to ensure the coverage of waste collection in the county from 75% to 100%; - develop an efficient collection and transportation scheme at county level, fully compliant with the EU Directives and Romanian legislation; - to implement a selective pre-collection system for paper, glass, organic matters and packaging waste so as to comply with the EU legislation and with the principles of maximum recycling and composting and minimum residual waste disposal; - to develop solid waste treatment facilities in order to minimise the resources usage and to decrease the quantity of waste to be disposed; this includes transfer stations, sorting plant and composting plants; - to ensure the closure of 1 urban disposal site at Curtea de Arges - to ensure the relocation and remediation of urban disposal site at Costesti - to provide safe waste disposal facilities for the whole population residing in Arges County, in compliance with the requirements of the Landfill Directive 1999/31/EC; - to promote job creation in the tertiary sector related with the collection, transport, sorting, recycling and elimination of MSW; to stimulate the market of by-products |

The priority investments in medium term (SOP project) include the following main components, as listed below. It is worthy to mention that the following components refer only those intended for Phase-II Zones in Arges County.

Component 1 	Municipal waste collection:

This will include supply and installation of municipal waste collection facilities as follows:

- waste collection for residual waste (combined system = Domiciliary + bring system)

Domiciliary Temporal Storage. The domiciliary waste storage consists in the procurement of specialized bins and/or containers to households in urban and rural areas of Arges County. The stored waste, from the bins and/or containers, are typically associated with door-to-door collection by compactor trucks. This procedure can be performed through mechanical and/or manual operations.

The required collection frequency is of 2 times per week with Domiciliary storage alternative. The minimal number of procured bins per household is one with a capacity of 120 L.

The waste collection for residual waste in rural area will be combined system = Door - to -  door + bring system, as follows:

- for Zone 1 about 70% from households will be provided with one bin of 120 l per household for residual waste collection, and for 30% from households the residual waste collection will be achieved through the bring system (Group of Containers on Streets, Collection Platforms);

- for Zone 2, the following localities Corbi, Domnesti and Pietrosani will be provided with one bin of 120 l per household for residual waste collection; about 18% from the remaining localities will be provided with one bin of 120 l per household for residual waste collection, and 82% from households the residual waste collection will be achieved through bring system (Group of Containers on Streets, Collection Platforms);

- for Zone 8 about 22% of the households will be provided with one bin of 120 l for residual waste collection, and 78% of the households the residual waste collection will be achieved through bring system (Group of Containers on Streets, Collection Platforms).

rste collection fidual be combinedng system, as following: for zone 1 about 70% from households will have for residual waste collection one bin to 120 l per household, and for 30% from households the residual waste collection will be achieved through the bring system (Group of Containers on Streets, Collection Platforms);

- one 2ties Corbi, Domnesti aani will hv0 l) per household, for about 18% from the other localitiual waste collection one bin to household,e residual waste collection wile bring system (Group ofets, Collection Plafor zone 8 about 22% from households will have for residual waste collection one bin to 120 l per household, and for 78% from households the residual waste collection will be achieved through the bring system (Group of Containers on Streets, Collection Platforms).

The system consists in to allocate groups of containers (for separate waste streams), along the main streets of urban and/or rural area, or in defined areas for this purpose. The location of these groups of containers is registered by the collection operators that will empty the containers with a regular frequency. The minimal amount of containers is 2 of 1.1 cubic meter capacity (Recyclables & Residual) in rural areas. In urban areas the minimal number of containers to place is 3 of 1.1 cubic meter capacity (Recyclables, Residual & Organic)

Collection Platforms. These facilities consist basically in restricted areas, surrounded by a fence, with a concrete platform and housing waste containers, for the storage of different waste streams (see Annex 9.3-1 and Annex 9.3-2).

The following table presents the required number of containers and bins to implement the separate storage of waste in the zones of Phase-II of the IWMP-Arges.

| Zone | Population 2013 |  | Domiciliary | Bring System |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Collection Platforms |  | Communal Points |
|  | Urban | Rural | Bins of 120L | Units | Containers 1.1 m3 | Containers 1.1 m3 |
| Z-1 | 32.649 | 9.2191 | 32.764 | 27 | 90 | 109 |
| Z-2 | 0 | 26.981 | 6.960 | 92 | 184 | 197 |
| Z-8 | 10.604 | 57.945 | 15.669 | 86 | 187 | 1004 |

1 Population no covered by PHARE projects in Zone 1

Component 2	Waste transfer, composting and sorting

In order to contribute to the recycling and biodegradable waste diversion targets set by the CWMP in line with the NWMP transposing the relevant acquis, significant investments are needed in Arges county. The following are included in the project:

- Two new transfer stations in Curtea de Arges (18.719 t/a) and Costesti (9.870 t/a);

- Two new composting plants (windrow composting): Curtea de Arges (12.621 t/a) and Costesti (5.878 t/a);

- Extension of Sorting plant in Albota landfill (20.000 t/a-shift)

Component 3	Closure of 2 non-compliant urban landfills

The current waste disposal facilities do not comply with the requirements of the Landfill Directive. The measure will provide for the closure of 2 non-compliant landfills in Curtea de Arges and Costesti cities. The following measures will be undertaken: for Curtea de Arges reshaping of the landfill body; surface water drainage system, temporary capping system, gas management (gas collection and flare system); ground water monitoring wells, and final cover. For Costesti, the location and features of this place makes the relocation/remediation approach more suitable for the case; the waste will be relocated into Albota landfill.

Component 4	 Technical Assistance and Supervision

This Component includes the following activities:

1. Technical Assistance: Technical assistance is foreseen under the project for assistance to the beneficiary in the management of project implementation and preparation of tender documents and contracts with various operators.

2. Supervision: Assistance is foreseen for supervision of the various works and supply contracts in accordance with the project implementation plan.

3. Public Awareness: Significant efforts are needed for raising the level of public awareness for waste-related and environmental issues. Key issues include the need for reduction of waste at source, source separation of recyclable materials from the general waste stream and payment of charges/user fees to sustain County wide collection, treatment and disposal services. A budget for public awareness is included in the annual cost estimates for the project.

4. PPP Study and Independent evaluation of designs

Component 5	 Landfill Extension with Cell-2

The capacity depletion of the Cell-1 in Albota landfill by 2014 will lead to an unavoidable extension of the Landfill capacity, by means of Cell-2.

At the beginning of the project, the Cell-2 Component was intended to be financed by the revenues of the operation of the IWMS Phase-I of Arges County. However, the results of a FEA for the operation of IWMS-Phase-I shows that this situation is not possible (see Volume III-FEA). Hence, it is recommended that Cell-2 would be financed by a grant component.

Parameter design of Cell-2:

| Item | Cell-2 |
| --- | --- |
| Geology | Natural barrier of more than 2 m of clay protects the aquifers of the area. |
| Topography | Smooth slopes at NW side of the current Cell-1 |
| Area of deposit | 3,6 ha |
| Capacity | 1,1 Mio m3 (App. 10 years) |
| Height | Max. 30 m |
| Coverage | Entire Arges County |
| Compact ratio | Up to 1,2 to 1,4 ton/m3 |
| Daily cover ratio | 1/6 |

The capacity depletion of Cell-1 and the expected for Cell-2 is presented in the following table.

| Year | Final Disposal of “New Waste” |  | Sludge from WWTP | Total | Accumulated | Depletion of Cell-1 | Depletion of Cell-2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (m3) |  | (m3) | (m3) | (m3) | (m3) | (m3) |
|  | PHASE-I | PHASE-II | Arges County |  |  |  |  |
| 2013 | 93.285 | 23.387 | 3.150 | 119.822 | 741.479 | 8.521 | 1.100.000 |
| 2014 | 93.836 | 23.449 | 3.150 | 120.435 | 861.913 | -111.913 | 988.087 |
| 2015 | 94.133 | 22.954 | 3.150 | 120.236 | 982.150 |  | 867.850 |
| 2016 | 89.731 | 21.757 | 3.150 | 114.638 | 1.096.788 |  | 753.212 |
| 2017 | 90.089 | 20.838 | 3.150 | 114.076 | 1.210.864 |  | 639.136 |
| 2018 | 90.451 | 20.988 | 3.150 | 114.589 | 1.325.452 |  | 524.548 |
| 2019 | 90.818 | 21.143 | 3.150 | 115.111 | 1.440.563 |  | 409.437 |
| 2020 | 91.189 | 21.304 | 3.150 | 115.644 | 1.556.207 |  | 293.793 |
| 2021 | 91.489 | 21.419 | 3.150 | 116.061 | 1.672.268 |  | 177.732 |
| 2022 | 91.791 | 21.538 | 3.150 | 116.482 | 1.788.750 |  | 61.250 |
| 2023 | 92.096 | 21.659 | 3.150 | 116.909 | 1.905.659 |  | -55.659 |

### Project indicators

The Project intends to achieve the following key indicators, which contribute to achieve the corresponding target indicators set in line with the Accession Treaty of Romania.

Main physical indicators in Arges County

| Indicator | Unit | Contribution of Phase-I (ISPA investment) | After Project (inc. Contribution of Phase-II) |
| --- | --- | --- | --- |
| No of transfer stations | Facility | 1 | 3 |
| No of composting plants | Facility | 2 | 4 |
| No of sorting stations | Facility | 1 | 1 |
| Non-compliant urban landfills closed and re-habilitated | Site | 3 | 5 |
| Non-compliant rural landfills closed and re-habilitated | Site | 100% | -- |

Main performance indicators in Arges County

| Indicator | Unit | Contribution of Phase-I (ISPA investment) | After Project (inc. Contribution of Phase-II) |
| --- | --- | --- | --- |
| Collection coverage urban areas | % inhabitants | 99 | 100 |
| Collection coverage rural areas | % inhabitants | 20 | 100 |
| Separate collection system | Yes/No | No | Yes |
| Volume of biodegradable waste diverted from the landfill | % | negligible | 20% – 30% |
| Total separate collected biodegradable waste | Tons/year | Negligible | 50.000 |
| Total separate collected packaging waste | Tons/year | Negligible | 34.000 |
| Compliance with the Landfill Directive (regarding diversion of biodegradable waste from landfills) | Yes/No | No | Yes |

### Project costs

The total project cost has been estimated at 20.409.209 EUR excluding VAT, of which 20.325286 Euro is considered as eligible. A detailed cost breakdown is given in the table below.

| EURO (Constant Prices) | TOTAL PROJECT COSTS | INELIGIBLE COSTS | ELIGIBLE COSTS |
| --- | --- | --- | --- |
|  | (A) | (B) | (C)=(A)-(B) |
| 1. Planning/design fees | 634,000 | 0 | 634,000 |
| 2. Land purchase | 0 | 0 | 0 |
| 3. Building and construction | 9,694,028 | 76,250 | 9,617,778 |
| 4. Plant and machinery | 4,401,828 | 0 | 4,401,828 |
| 5. Contingencies | 1,633,323 | 0 | 1,633,323 |
| 6. Price adjustment (if applicable) | 1,844,052 | 7,574 | 1,836,479 |
| 7. Technical assistance | 1,020,978 | 0 | 1,020,978 |
| 8. Publicity | 487,250 | 0 | 487,250 |
| 9. Supervision during construction implementation | 693,750 | 0 | 693,750 |
| 10. Sub-TOTAL | 20,409,209 | 83,824 | 20,325,386 |
| 11. (VAT), commissions, legal taxes | 4,879,716 | 4,879,716 | 0 |

Indicative cost breakdown by main components

| No. | Component | Share | Civil Works | Equipment | Investment |
| --- | --- | --- | --- | --- | --- |
| 1. | Mixed Waste Collection | 17% | 624.834 € | 1.695.223 € | 2.320.057 € |
| 2. | Transfer Stations | 15% | 1.277.938 € | 701.430 € | 1.979.368 € |
| 3. | Landfill | 23% | 3.071.100 € | 77.000 € | 3.148.100 € |
| 4. | Sorting Plants | 1% | 0 € | 120.000 € | 120.000 € |
| 5. | Landfill/Dumpsite Closure | 30% | 3.418.991 € | 713.850 € | 4.132.841 € |
| 6. | Composting Plant | 14% | 801.165 € | 1.094.325 € | 1.895.490 € |
|  | TA |  |  | 19,12% | 2.600.000 € |
|  | Site Preparation |  |  |  | 500.000 € |
|  | Contingencies |  |  | 8,91% | 1.633.323 € |
|  | Total incl. Contingencies, FD, TA |  |  |  | 18.329.179 € |
|  | Fees, taxes, allowances |  |  |  | 125.880 € |
|  | Salaries PIU |  |  |  | 70.098 € |
|  | PIU Furniture |  |  |  | 10.000 € |
|  | Audit Costs |  |  |  | 30.000 € |
|  | TOTAL |  |  |  | 18.565.157 |

The specific costs of the project are reflected in the following table:

| Item* |  | Indicator | Unit | Unit costs |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  | Unit Investment Costs for Collection and transport |  |  |  |
| 1,1 |  | Total per capita investment costs a | € / capita | 13,6 |  |
| 1,2 |  | Investment costs per ton collected a | € / t | 42,6 |  |
| 2 |  | Unit Investment costs for Transfer Stations |  |  |  |
| 2,1 |  | Investment costs per ton transported to landfilla | € /t | 69,2 |  |
| 3 |  | Unit Investment Costs landfill construction |  |  |  |
| 3,1 |  | Total per capita investment costsb | € / capita | 5,0 |  |
| 3,2 |  | Investment costs per surface area used c | € / ha. | 157.405 |  |
| 3,3 |  | Investment costs per ton stored at landfill d | € /t | 2,9 |  |
| 4 |  | Unit investment costs for biological treatment |  |  |  |
| 4,1 |  | Total per capita investment costsa | € / capita | 11,1 |  |
| 4,2 |  | Investment costs per ton Input a,e | €/t | 92,7 |  |
| 5 |  | Unit investment costs for sorting plant |  |  |  |
| 5,1 |  | Total per capita investment costs | € / capita | 0,7 |  |
| 5,2 |  | Investment costs per ton Input | €/t | 6,0 |  |
| 6 |  | Unit investment cost for closure of existing sites |  |  |  |
| 6,1 |  | Costs per m3 closed of landfill | €/ma | 6,3 |  |
|  | a Considering only the data from Zones 1,2 & 8 of Arges County in 2013, when its foreseen a full collection working capacity |  |  |  |  |
|  | b Considering data from the whole Arges County |  |  |  |  |
|  | c The area corresponds only to the Cell-2 |  |  |  |  |
|  | d Total capacity of 1,1 Mio m3, with a specific weight between 1 and 1,4 tons per cubic meter |  |  |  |  |
|  | e Total amount of input in 2013, 20.454 tons |  |  |  |  |

## Demand analysis

Data on current waste generation and composition is based on various sources, including data from the Arges County EPA, municipalities and operators (also reflected in the County Waste Management Plan and the RWMP). The current generation of municipal solid waste (based on 2009 figures) in the County is reported to be about 282.655 t/year.

The projections for future waste generation are based on a yearly growth of 0.8% (from 2008 to 2013) which is in accordance with the national and regional waste management plans.

### Household and similar waste generation data

The estimated quantity of the waste generated and uncollected takes into account the following:

- generation indicator in urban area of 0,21 kg/inhabitant x day;

- generation indicator in rural area of 0,19 kg/inhabitant x day;

- the quantity of similar waste generated in rural area represents about 33 % of the quantity of household waste.

### Population Forecast

The Argeş County is located into the South-Muntenia region of Romania. The total population of this county is around 650 thousand inhabitants, which is spread into three municipalities (Piteşti, Curtea de Argeş and Câmpulung), four cities (Costeşti, Mioveni, Stefanesti and Topoloveni, and 95 rural communes. The Urban population consists in 48% and the rural the 52% of the total. The Population is of  640.376 inhabitants. According the National Institute of Statistics, the population of Arges County will decrease with 585.181 inhabitants by the year 2039, of which 48% in urban area and 52% in rural area.

### Connection rate to sanitation services

The connection rate to sanitation services at the end of the year 2008 in County was of approximately 61 %, of which 100% in urban area and  25%in rural area. The service coverage is estimated at 100% for the whole county by the end of 2011

### Municipal waste generation indicator

At this point is worthy to underline that the IWMP in Arges County has bee split in 2 main Phases (see Annex 3.2-1), i.e.

- Phase I. Considering 5 Zones with a coverage of 73% of county population, financed by ISPA program and currently under implementation

- Phase-II. Considering 3 Zones with a coverage of 27% of county population, to apply for cohesion funds

The table below summarizes the rates considered for waste amount calculations in the Feasibility Study

| Indicator | Arges County |  |  |  |
| --- | --- | --- | --- | --- |
| Year | 2005 | 2010 | 2020 | 2035 |
| Domestic Waste-Urban | 0,94 Kg/hab-day | 0,978 Kg/hab-day | 1,041 Kg/hab-day | 1,092 Kg/hab-day |
| Commercial Waste-Urban | 0,575 Kg/hab-day | 0,599 Kg/hab-day | 0,637 Kg/hab-day | 0,668 Kg/hab-day |
| Domestic Waste-Rural | 0,42 Kg/hab-day | 0,437 Kg/hab-day | 0,471 Kg/hab-day | 0,509 Kg/hab-day |
| Commercial Waste-Rural | 0,178 Kg/hab-day | 0,185 Kg/hab-day | 0,197 Kg/hab-day | 0,207 Kg/hab-day |
| Green Waste Urban | 0,061 Kg/hab-day | 0,063 Kg/hab-day | 0,067 Kg/hab-day | 0,072Kg/hab-day |
| Green Waste Rural | 0,019 Kg/hab-day | 0,02 Kg/hab-day | 0,021 Kg/hab-day | 0,022Kg/hab-day |
| Market Waste Urban | 0,016 Kg/hab-day | 0,017 Kg/hab-day | 0,018 Kg/hab-day | 0,019 Kg/hab-day |
| Market Waste Rural | 0,005 Kg/hab-day | 0,005 Kg/hab-day | 0,006Kg/hab-day | 0,006 Kg/hab-day |
| Street Waste Urban | 0,132 Kg/hab-day | 0,137 Kg/hab-day | 0,146 Kg/hab-day | 0,153 Kg/hab-day |
| Street Waste Rural | 0,041 Kg/hab-day | 0,043 Kg/hab-day | 0,045 Kg/hab-day | 0,047Kg/hab-day |
| Construction Waste Urban | 0,229 Kg/hab-day | 0,239 Kg/hab-day | 0,255 Kg/hab-day | 0,272 Kg/hab-day |
| Construction Waste Rural | 0,071 Kg/hab-day | 0,074Kg/hab-day | 0,079 Kg/hab-day | 0,084Kg/hab-day |
| Population (Inh.) | 646.320 | 638.455 | 619.558 | 592.256 |

### Municipal waste projection

Based on the quantities of municipal waste generated in Arges County in 2008 both in urban and rural areas and the indicators mentioned in the table above, the quantities projected to be generated during the planning period of Phase-II are calculated.

Projection of the total quantity of municipal waste in Arges county

|  | Generated Waste (ton/year) | Code | 2010 | 2013 | 2016 | 2020 | 2025 | 2030 | 2035 | 2039 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,0 | Municipal waste and assimilable waste from commerce, industry and institutions, out of which: | 20 15 01 | 284.061 | 288.323 | 291.145 | 293.941 | 297.478 | 297.486 | 296.610 | 295.914 |
| 1,1 | Municipal waste mixed collected from the population, out of which: | 20 03 01 | 162.279 | 164.713 | 166.407 | 168.171 | 170.406 | 170.623 | 170.335 | 170.107 |
|  | 1.1.1 Urban |  | 109.230 | 110.869 | 111.863 | 112.754 | 113.878 | 113.645 | 113.075 | 112.621 |
|  | 1.1.2 Rural |  | 53.048 | 53.844 | 54.544 | 55.417 | 56.528 | 56.978 | 57.260 | 57.486 |
| 1,2 | Assimilable waste mixed collected from commerce, industry and institutions | 20 03 01 | 89.355 | 90.696 | 91.509 | 92.238 | 93.157 | 92.967 | 92.500 | 92.129 |
|  | 1.2.1 Urban |  | 66.856 | 67.859 | 68.468 | 69.013 | 69.701 | 69.558 | 69.209 | 68.931 |
|  | 1.2.2 Rural |  | 22.499 | 22.837 | 23.041 | 23.225 | 23.456 | 23.408 | 23.291 | 23.197 |
| 1,3 | Municipal and similar waste collected separately (except construction and demolition waste) out of which: | 20 01 15 01 |  |  |  |  |  |  |  |  |
|  | Paper and cardboard | 20 01 01 15 01 01 | 13.387 | 15.497 | 17.598 | 19.807 | 20.818 | 21.879 | 20.807 | 19.587 |
|  | Glass | 20 01 02 15 01 07 | 12.366 | 14.315 | 16.256 | 18.296 | 19.230 | 20.211 | 19.220 | 18.093 |
|  | Metal | 20 01 40 15 01 04 | 5.105 | 5.910 | 6.711 | 7.554 | 7.939 | 8.344 | 7.935 | 7.469 |
|  | Plastic | 20 01 39 15 01 02 | 16.450 | 19.043 | 21.625 | 24.339 | 25.581 | 26.886 | 25.568 | 24.068 |
|  | Wood | 20 01 38 15 01 03 | 6.807 | 7.880 | 8.948 | 10.071 | 10.585 | 11.125 | 10.580 | 9.959 |
|  | Biodegradable | 20 01 08 | 150.028 | 173.585 | 168.950 | 163.155 | 158.860 | 151.775 | 147.414 | 144.993 |
|  | Others | 20 01 15 01 | 46.630 | 52.140 | 51.914 | 51.442 | 50.883 | 49.761 | 48.613 | 47.700 |
| 1,4 | Bulky waste | 20 03 07 | -- | -- | -- | -- | -- | -- | -- | -- |
| 1,5 | Waste from gardens and parks | 20 02 | 9.407 | 9.548 | 9.652 | 9.768 | 9.915 | 9.944 | 9.943 | 9.943 |
|  | Urban |  | 7.038 | 7.144 | 7.222 | 7.309 | 7.418 | 7.440 | 7.440 | 7.439 |
|  | Rural |  | 2.369 | 2.404 | 2.430 | 2.460 | 2.496 | 2.504 | 2.504 | 2.504 |
| 1,6 | Market waste | 20 03 02 | 2.514 | 2.552 | 2.575 | 2.595 | 2.621 | 2.616 | 2.603 | 2.592 |
|  | Urban |  | 1.881 | 1.909 | 1.926 | 1.942 | 1.961 | 1.957 | 1.947 | 1.939 |
|  | Rural |  | 633 | 643 | 648 | 653 | 660 | 659 | 655 | 653 |
| 1,7 | Street waste** | 20 03 03 | 20.507 | 20.815 | 21.001 | 21.169 | 21.380 | 21.336 | 21.229 | 21.143 |
|  | Urban |  | 15.343 | 15.574 | 15.713 | 15.838 | 15.996 | 15.964 | 15.884 | 15.820 |
|  | Rural |  | 5.164 | 5.241 | 5.288 | 5.330 | 5.383 | 5.372 | 5.345 | 5.324 |
|  | Kg/inh-year |  | 445 | 456 | 464 | 474 | 487 | 495 | 501 | 506 |
|  | Kg/inh-day |  | 1,22 | 1,25 | 1,27 | 1,30 | 1,34 | 1,36 | 1,37 | 1,39 |
| 1,8 | Generated and uncollected waste, out of which: | 20 01 15 01 | 32.284 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Urban |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Rural |  | 32.284 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2,0 | Municipal and assimilable waste sorted collected (exclusively waste from demolitions and constructions)* |  | 35.630 | 36.164 | 36.561 | 36.999 | 37.554 | 37.665 | 37.663 | 37.662 |
|  | Urban |  | 26.658 | 27.058 | 27.356 | 27.683 | 28.098 | 28.181 | 28.180 | 28.179 |
|  | Rural |  | 8.971 | 9.106 | 9.206 | 9.316 | 9.456 | 9.484 | 9.483 | 9.483 |
| 3,0 | Sludge from WWTP (ton) |  | 10.000- | 16.000 | 16.000 | 16.000 | 16.000 | 16.000 | 16.000 | 16.000 |

### Special Waste Streams

The special waste streams to be generated in Arges County are as summarized by the next table.

| Year | 2010 | 2013 | 2016 | 2020 | 2025 | 2030 | 2035 | 2039 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Household Hazardous Waste (tons) |  |  |  |  |  |  |  |  |
| Urban | 765 | 758 | 751 | 742 | 731 | 720 | 709 | 701 |
| Rural | 499 | 494 | 490 | 484 | 477 | 470 | 463 | 457 |
| Index Urban (kg/inh-year) | 2,5 |  |  |  |  |  |  |  |
| Index Rural (kg/inh-year) | 1,5 |  |  |  |  |  |  |  |
| Municipal and assimilable waste sorted collected (exclusively waste from demolitions and constructions)* |  |  |  |  |  |  |  |  |
| Generation County (tons) | 35.630 | 36.164 | 36.561 | 36.999 | 37.554 | 37.665 | 37.663 | 37.662 |
| Generation Urban | 26.658 | 27.058 | 27.356 | 27.683 | 28.098 | 28.181 | 28.180 | 28.179 |
| Generation Rural | 8.971 | 9.106 | 9.206 | 9.316 | 9.456 | 9.484 | 9.483 | 9.483 |
| County Index (Kg/inh-y) | 55,81 | 57,16 | 58,31 | 59,72 | 61,53 | 62,65 | 63,59 | 64,36 |
| Index Urban (Kg/inh-y) | 87,14 | 89,25 | 91,04 | 93,25 | 96,08 | 97,82 | 99,30 | 100,50 |
| Index Rura (Kg/inh-y)l | 26,98 | 27,63 | 28,19 | 28,87 | 29,75 | 30,29 | 30,74 | 31,11 |
| Bulky waste |  |  |  |  |  |  |  |  |
| Generation County (tons) | 1.302 | 1.290 | 1.274 | 1.259 | 1.240 | 1.218 | 1.199 | 1.185 |
| Generation Urban | 974 | 965 | 953 | 942 | 928 | 911 | 897 | 887 |
| Generation Rural | 328 | 325 | 321 | 317 | 312 | 307 | 302 | 298 |
| County Index (Kg/inh-y) | 2,04 | 2,04 | 2,03 | 2,03 | 2,03 | 2,03 | 2,03 | 2,03 |
| Index Urban (Kg/inh-y) | 3,18 | 3,18 | 3,17 | 3,17 | 3,17 | 3,16 | 3,16 | 3,16 |
| Index Rura (Kg/inh-y)l | 0,99 | 0,99 | 0,98 | 0,98 | 0,98 | 0,98 | 0,98 | 0,98 |
| End-of-Life-Vehicles |  |  |  |  |  |  |  |  |
| Arges County (Unit) | 3.714 | 3.831 | 5.315 | 7.907 | 9.941 | 12.499 | 15.714 | 18.873 |
| County Index (ELV/1000inh) | 6 | 8,4 | 10,5 | 12,8 | 16,3 | 20,8 | 26,5 | 32,3 |
| Waste from Electric and Electronic Equipment (WEEE) |  |  |  |  |  |  |  |  |
| Arges County (Tons) | 2.554 | 3.796 | 3.762 | 3.717 | 3.662 | 3.607 | 3.554 | 3.511 |
| County Index (Kg/inh-y) | 4 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |

## Biodegradable waste & Recycling targets

As regards the diversion of the biodegradable waste from the landfill, the project is in line with the targets set by the legislation, which represents a reduction in disposal of biodegradable waste up to 50% by 2013 (compared to the quantity reported in 1995). This target will be met thorough a mix of measures provided by SOP project and other programmes implemented by the county. The Phase-II project contribution represents 27% of total biodegradable waste to be disposed off in landfill per year by 2013. Additional measures to be implemented beyond 2013 are expected to reduce the disposal rate up to 35% by 2016.

According to the County Waste Management Plan, the quantity of biodegradable waste allowed to be disposed off in landfill will be decrease gradually in Arges County:

- 107.756 tonnes in 2010;

- 71.837 tonnes in 2013;

- 50.286 tonnes in 2016.

The reduction of the quantity of biodegradable waste is to be achieved by implementation of a mix of measures as follows:

- Individual composting;

- Composting in the 4 composting plants (2 in Phase-I & 2 in Phase-II);

- Recycling and recovery of paper, cardboard and wood waste;

- Home composting in rural areas

- Publicity, awareness campaign

Quantifying targets for biodegradable municipal waste in Arges County

| Biodegradable municipal waste | Unit | 2010 | 2013 | 2016 |
| --- | --- | --- | --- | --- |
| Quantity of biodegradable waste generated in 1995 | ton | 143.635 |  |  |
| Maximum amount of biodegradable waste that can be disposed in landfill |  | 107.726 | 71.817 | 50.272 |
| Achieved amount of biodegradable waste that can be disposed (Phase-I + Phase-II) |  | 86.180 | 63.907 | 49.714 |
| Quantity of biodegradable waste to be reduced from landfill |  | 48.278 | 93.444 | 101.130 |
| Quantity of generated biodegradable waste |  | 134.458 | 157.351 | 150.844 |

Source: CWMP- Arges & FS-Arges

In regard the targets for packaging waste and based on the amount of packaging waste by type of material expected to be generally quantified targets for recycling and recovery.

The recovery amount is presented in the following table.

Amounts of recovered packaging waste in Arges  County

| Year | Households |  |  |  |  | Commerce & Institutions |  |  |  |  | Sum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (ton) |  |  |  |  | (ton) |  |  |  |  |  |
|  | Paper | Glass | Metal | Plastic | Wood | Paper | Glass | Metal | Plastic | Wood |  |
| 2010 | 7.229 | 7.012 | 4.166 | 8.883 | 3.676 | 4.819 | 4.452 | 2.777 | 5.922 | 2.451 | 51.387 |
| 2011 | 8.012 | 7.771 | 4.617 | 9.846 | 4.074 | 5.341 | 4.934 | 3.078 | 6.564 | 2.716 | 56.954 |
| 2012 | 8.856 | 8.589 | 5.103 | 10.882 | 4.503 | 5.904 | 5.453 | 3.402 | 7.255 | 3.002 | 62.949 |
| 2013 | 9.298 | 9.019 | 5.358 | 11.426 | 4.728 | 6.199 | 5.726 | 3.572 | 7.617 | 3.152 | 66.096 |
| 2014 | 9.763 | 9.470 | 5.626 | 11.997 | 4.964 | 6.509 | 6.012 | 3.751 | 7.998 | 3.310 | 69.401 |
| 2015 | 10.252 | 9.754 | 5.908 | 12.597 | 5.213 | 6.834 | 6.313 | 3.938 | 8.398 | 3.475 | 72.682 |
| 2016 | 10.559 | 10.046 | 6.085 | 12.975 | 5.369 | 7.039 | 6.502 | 4.057 | 8.650 | 3.579 | 74.862 |

The amounts material from Packaging waste that is recycled by the IWM System in Arges County (Phase-I + Phase-II) are as follows:

Amounts of recycled packaging waste in Arges  County

| Year | Amounts |  |  |  |  | Sum |
| --- | --- | --- | --- | --- | --- | --- |
|  | (ton) |  |  |  |  |  |
|  | Paper | Glass | Metal | Plastic | Wood |  |
|  | 80% | 60% | 50% | 23% | 15% |  |
| 2010 | 10.710 | 7.642 | 3.857 | 3.701 | 1.021 | 26.932 |
| 2011 | 11.245 | 8.024 | 4.050 | 3.886 | 1.072 | 28.278 |
| 2012 | 11.808 | 8.426 | 4.253 | 4.081 | 1.126 | 29.692 |
| 2013 | 12.398 | 8.847 | 4.465 | 4.285 | 1.182 | 31.177 |
| 2014 | 13.018 | 9.289 | 4.689 | 4.499 | 1.241 | 32.736 |
| 2015 | 13.669 | 9.640 | 4.923 | 4.724 | 1.303 | 34.259 |
| 2016 | 14.079 | 9.929 | 5.071 | 4.866 | 1.342 | 35.287 |

In order to verify the achievement of the targets, the following table shows the targeted quantified amounts of packaging waste in the Local Waste Management Plan of Arges County.

Quantifying targets for packaging waste, total and by type of material (Source: LWMP Arges)

| Amounts | 2010 | 2011 | 2012 | 2013 |
| --- | --- | --- | --- | --- |
| H+C | 9.639 | 10.683 | 11.512 | 12.398 |
| Plastic | 2.303 | 2.764 | 3.265 | 4.189 |
| Glass | 5.441 | 6.232 | 7.362 | 8.589 |
| Metal | 3.267 | 3.860 | 4.165 | 4.492 |
| Wood | 817 | 1.072 | 1.276 | 1.497 |
| Total recycled | 21.467 | 24.611 | 27.580 | 31.165 |
| Total Collected | 27.228 | 31.567 | 35.647 | 39.400 |

When comparing the last 3 tables, the achievements for recovery and recycling of packaging waste are fully achieved, according to the targets set in the LWMP.

## Economic Analysis and Financial Analysis

The purpose of the summary is to provide a general overview of the approach and methodology applied and to highlight the main conclusions of the financial and economic analysis.

The objective of the Financial Analysis (FA) was to assess the financial viability and sustainability of the Project over the entire project lifetime.

In general terms the FA takes into account all relevant data and information made available from the various sources and especially the reports, financial statements and production / service data provided by the local stakeholders. It takes further into account the socio-economic data and background information presented in the Master Plan Report and the technical concepts, demand projections and cost estimates, as detailed in the respective chapters of the Feasibility Study.

According to EU standards of the CBA, and thus also the financial analysis uses the “incremental method”: that means, the project was evaluated on the basis of the differences between the scenario “with the project” and an alternative scenario “without the project”. For the “with project” scenario cost and revenues considered are those of a scenario of efficient operation. For the “without project” scenario cost and revenues considered are those of a “business as usual” without any major new investments or replacements.

The eligible investment project considered for the analysis is presented in the following table (in Euro, nominal terms):

Eligible investment costs in current prices

| Investment costs |  | Total | 2010 | 2011 | 2012 | 2013 |
| --- | --- | --- | --- | --- | --- | --- |
| Planning/design fees | EUR | 696,974 |  | 226,166 | 232,498 | 238,311 |
| Land purchase | EUR |  |  |  |  |  |
| Building and construction | EUR | 10,573,099 |  | 3,430,931 | 3,526,997 | 3,615,172 |
| Plant and machinery | EUR | 4,839,056 |  | 1,570,255 | 1,614,222 | 1,654,578 |
| Contingencies | EUR | 1,795,559 |  | 582,652 | 598,966 | 613,941 |
| Technical assistance | EUR | 1,122,390 |  | 364,211 | 374,409 | 383,770 |
| Publicity | EUR | 535,648 |  | 173,816 | 178,683 | 183,150 |
| Supervision during construction implementation | EUR | 762,659 |  | 247,480 | 254,409 | 260,770 |
| TOTAL | EUR | 20,325,386 |  | 6,595,511 | 6,780,185 | 6,949,690 |

A projection of the cash flows of the consolidated waste management system from the present up until 2040 was carried out. These flows include:

- capital costs, including the proposed project expenditures and future replacement investments;

- recurrent costs of the various waste management facilities: salaries, energy, maintenance, etc., plus loan service;

- recurrent revenues: basically revenues from charges levied on waste producers and revenues from recyclables and compost. The tariffs were set considering the affordability limitation of 1.80% for the lowest decile.

- capital revenues: the various funds which will finance the project: ERDF grant, grants from central, county and local government in Arges County, bank loans.

The projection took into consideration that in Arges County the solid waste management strategy consisted of 2 investment projects:

- Investment cost for Phase I financed by an ISPA project and covering the areas of Pitesti, Campulung, Rucar, Mioveni and Topoloveni;

- Investment cost for Phase II subject of the analysis of the current Cohesion Funds Application and covering the areas of Curtea de Arges, Domnesti and Costesti.

Due to the investment implementation in 2 Phases, all forecast are prepared separately for each of the 2 areas (demand, revenues, O&M, etc).

Inclusion of cell 2 in the investments to be financed from the SOP was deemed necessary due to the following reasons:

- Total amount of investments for Phase I  have increased significantly, which might affect total cost of the system to the end customer:

  - the initial amount has been EUR 24,5 million;

  - the final amount has been  EUR 35 million;

- The population development for the ISPA Application was considered with an increase of +0.05% annually. The current assumption for population development is a negative rate of -0.30%. Therefore, the numbers of inhabitants to be charge the tariffs are fewer than previous envisaged.

- The O&M costs recorded a significantly higher increase in real terms that initially envisaged in the ISPA Application.

In order verify whether the investment from own funds of the second cell was affordable, we calculated the Dynamic Prime Costs (DPC) as being a long term average tariff and we compare it with the affordable tariff. The results are presented in the following chart:

Comparison of Dynamic Prime Costs and Affordable tariff

The chart clearly shows that if we compare the DPC only for Phase I (without the financing of Cell II) with the affordable tariff we obtain the following results:

- For 2010, 2011 and 2012 the DPC is higher than the affordable tariff showing that the system is not assuring a full cost recovery;

- For 2013, 2014 and 2015 the affordable tariff is exceeding the DPC showing the possibility to generate additional financial resources. However, considering the fact that the construction of cell 2 should start latest in 2014 to be functional in 2015, the amount of money generated during the 2013 and 2014 are not sufficient to cover the investment costs.

Also if we compare the Dynamic Prime Costs for the entire county (Phase I and Phase II) we notice that until 2015, the DPC is above the affordable tariff mainly due to the higher level of operating costs from Phase II (the density of population is lower leading to high collection costs).

Considering the arguments presented above, we recommended the construction Cell 2 as part of the investments proposed for the EU Funds Application.

Considering all of the above, we obtained the following financial ratios from the project analysis:

- The level of the financing gap calculated with a discount rate of 5% is 91.25%.

- The financial NPV and the Financial Rates of Return with and without community assistance are as follows:

| Before community assistance |  | After community assistance |  |  |  |
| --- | --- | --- | --- | --- | --- |
| FNPV/C | FRR/C | FNPV/C | FRR/C | FNPV/K | FRR/K |
| -15,439,048 | -7.84% | -1,969,698 | -0.44% | -2,001,945 | -1.63% |

Taking into account the detailed analysis from the previous chapter, the Consultants propose the following financing scheme for the project (as percentage of total eligible costs):

- EU Grant: 73.00%;

- State Budget subsidy: 16.43%;

- Local Budgets contribution: 1.83%;

- Loan contracted by the County Council: 8.75%

The financing plan for the eligible cost is presented in the following table:

| Source of financing | EUR |
| --- | --- |
| EU grant | 14,838,182 |
| Central Government grant | 3,338,591 |
| Co-financing loan | 1,777,658 |
| Local Authority Grant | 370,955 |

The contribution of the Local Authority Grant will be assured from the budget of the County Council. The non-funding gap will be financed by the County Council by contracting a loan.

In order to prove that the County Council can assure the co-financing of the investment, the Consultant has performed an analysis of the financial status of local authority. The analysis revealed the following conclusions:

- The own revenues of the local authority showed a stable trend in 2009 compared with 2008 while in 2010 there is expected a small decrease mainly due to the financial crisis;

- The expenditures showed a decreased mainly in the category of materials and works showing the commitment of the local authority to keep its expenditures under control;

In the following chart we show the evolution of the debt service capacity of the County Council considering all the contracted loans:

County Council Debt Service Level forecast

The chart shows that the debt service level of the County Council will reach a maximum in 2012 (around 17% from the own revenues) and will decrease afterwards. These levels are below the maximum ceiling of 30% of the own revenues showing that the County Council can contract without any problems the co-financing loan for this project.

In order to prove that the project is advantageous to the society of the country the Consultant has performed a detailed economic analysis. The economic analysis is based on the following assumptions:

- The period for the economic evaluation is 2011 to 2040;

- The base year for evaluation is 2010;

- All cost and benefit figures are stated in constant prices (Euro);

- Discount rate used for calculation of NPV is 5.5%.

The Benefit/Cost Ratio is 1.223, the ERR 15.81 %, and the NPV calculated at a discount rate of 5.5% is EURO 9.17 million.

The project shows satisfactory economic indicators with economic benefits exceeding economic cost.

Considering the need to respond the current needs of the system, the collection systems has been adapted similary with the bring system for part of the household where the access of the collection trucks is difficult. The necessity result from the fact that there are collection platforms and bring system because the collection vehicles can not access every location.

As a result, 10% of the collection bins have been transformed into collection containers placed on the street. Following this modification in the collection system in order to improve and assure the 100% coverage with sanitation services, the financial indicators of the project will not suffer any changes, the collection costs being calculated from the beginning for a combined collection system (door to door and bring system).

result from th that hre coorms and bringm beca th  can not access evey

The change will only impact on the technical efficiency of the system for collecting residual waste, which will become more flexible. This will enable a better response to inhabitant’s demand from the sanitation operators responsible for waste collection, transport and operation of the two transfer stations (Curtea de Arges and Costesti) to be appointed after the Concession Contract award procedure for the services  mentioned above

## Financing Plan

The total project cost in current prices has been estimated at 20.409.209 EUR excluding VAT, of which 20.325.386 Euro is considered as eligible for consideration under the calculations for determining the EU’s projected funding. A detailed cost breakdown is given in the table below.

Cost breakdown (in Euro constant prices, excluding VAT), is presented below:

| No. | Component | Share | Civil Works | Equipment | Investment | O&M Costs | Amortisation | Running Costs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Mixed Waste Collection | 17% | 624.834 € | 1.695.223 € | 2.320.057 € | 1.490.652 € | 565.557 € | 2.056.209 € |
| 1.1 | Curtea de Arges |  | 82.215 € | 765.466 € | 847.681 € | 114.820 € | 112.641 € | 227.461 € |
| 1.2 | Domnesti |  | 280.140 € | 242.724 € | 522.864 € | 36.409 € | 45.880 € | 82.289 € |
| 1.3 | Costesti |  | 262.479 € | 617.033 € | 879.512 € | 92.555 € | 98.647 € | 191.202 € |
| 1.4 | Residue Collection Trucks |  | 0 € | 0 € | 0 € | 331.855 € | 87.820 € | 419.675 € |
| 1.5 | Recyclables Coll.-Trucks |  | 0 € | 0 € | 0 € | 824.815 € | 213.569 € | 1.038.384 € |
| 1.5 | HHW Coll. Trucks |  | 0 € | 70.000 € | 70.000 € | 90.198 € | 7.000 € | 97.198 € |
| 2. | Transfer Stations | 15% | 1.277.938 € | 701.430 € | 1.979.368 € | 266.391 € | 128.094 € | 394.486 € |
| 2.1.2 | Curtea de Arges |  | 649.103 € | 354.568 € | 1.003.671 € | 155.786 € | 71.565 € | 227.351 € |
| 2.1.3 | Costesti |  | 628.835 € | 346.863 € | 975.697 € | 110.605 € | 56.529 € | 167.134 € |
| 3. | Landfill | 23% | 3.071.100 € | 77.000 € | 3.148.100 € | 177.355 € | 0 € | 177.355 € |
| 3.2 | Albota Cell-2 |  | 3.071.100 € | 77.000 € | 3.148.100 € | 177.355 € | 0 € | 177.355 € |
| 4. | Sorting Plants | 1% | 0 € | 120.000 € | 120.000 € | 141.200 € | 10.909 € | 152.109 € |
| 4.1 | Extension at Albota |  | 0 € | 120.000 € | 120.000 € | 141.200 € | 10.909 € | 152.109 € |
| 4.2 |  |  | 0 € | 0 € | 0 € | 0 € | 0 € | 0 € |
| 5. | Landfill Closure | 30% | 3.418.991 € | 713.850 € | 4.132.841 € | 14.396 € | 27.525 € | 41.921 € |
| 5.1 | Curtea de Arges |  | 2.115.563 € | 713.850 € | 2.829.413 € | 14.396 € | 27.525 € | 41.921 € |
| 5.2 | Costesti |  | 1.303.428 € | 0 € | 1.303.428 € | 0 € | 0 € | 0 € |
| 6. | Composting Plant | 14% | 801.165 € | 1.094.325 € | 1.895.490 € | 222.749 € | 102.554 € | 325.303 € |
| 6.1 | Curtea de Arges Plant |  | 429.514 € | 521.643 € | 951.157 € | 111.520 € | 51.016 € | 162.536 € |
| 6.2 | Costesti Plant |  | 371.651 € | 572.682 € | 944.333 € | 111.229 € | 51.537 € | 162.767 € |
|  | Sub-total: | 100% | 9.194.028 € | 4.401.828 € | 13.595.856 € | 2.312.743 € | 834.639 € | 3.147.382 € |
|  | TA |  |  | 19,12% | 2.600.000 € |  |  |  |
|  | Site Preparation |  |  |  | 500.000 € |  |  |  |
|  | Contingencies |  |  | 8,91% | 1.633.323 € |  |  |  |
|  | Total incl. Contingencies, FD, TA |  |  |  | 18.329.179 € |  |  |  |
|  | Fees, taxes, allowances |  |  |  | 125.880 € |  |  |  |
|  | Salaries PIU |  |  |  | 70.098 € |  |  |  |
|  | PIU Furniture |  |  |  | 10.000 € |  |  |  |
|  | Audit Costs |  |  |  | 30.000 € |  |  |  |
|  | TOTAL |  |  |  | 18.565.157 |  |  |  |

Following the decrease with 10% of the individual bins and increased number of 1.1 cm containers, the investments costs for the collection component is unchanged.

The non-funding gap is financed by the Beneficiary from own sources or by contracting a loan. The financing plan for the eligible costs is presented in the following table:

Financing plan (in Euro)

| Financing Plan | Total | 2011 | 2012 | 2013 |
| --- | --- | --- | --- | --- |
| Eligible expenditures | (20,325,386) | (6,595,511) | (6,780,185) | (6,949,690) |
| EU Grant | 14,838,182 | 4,814,934 | 4,949,752 | 5,073,496 |
| State Budget Contribution | 3,338,591 | 1,083,360 | 1,113,694 | 1,141,537 |
| Local Budget Contribution | 370,955 | 120,373 | 123,744 | 126,837 |
| Loan | 1,777,658 | 576,843 | 592,995 | 607,820 |

For the local contribution part the accepted solution is:

- Contribution by the County Council involved in the project

The contribution of the County Council adds up to the amount of EUR 0.370 million.