7	Design parameters	113

7.1	Population Growth	113

7.2	Collection and Transport	113

7.2.1	Waste Storage for Residual Waste (domiciliary + bring system)	113

7.2.2	Waste Separate Storage of Recyclable Streams (domiciliary + bring system)	114

7.2.3	Waste Separate Storage of Household Biodegradable	114

7.2.4	Waste Collection	115

7.2.5	Waste transfer	117

7.3	Landfill Design	118

7.3.1	Size of the Cell-2	118

7.3.2	Bottom Lining Construction	119

7.3.3	Leachate Drainage, Collection and Treatment	119

7.3.4	Landfill Gas Treatment	120

7.3.5	Capping System Closure of Landfill	121

7.4	Closure of Disposal Sites in Urban Areas	121

7.4.1	General	121

7.4.2	Curtea de Arges Site	122

7.4.3	Costesti	123

7.5	Biological Treatment	124

7.6	Sorting Plant	124

7.7	Bring Centres	124

7.8	Sludge treatment and Disposal	125

# Design parameters

## Population Growth

According to INSSE 2007, the population in the urban area of the Arges County will be decreased from 308.698 inhabitants in 2007 to 280.401 inhabitants by 2039, as well as the population of rural areas from 335.538 inhabitants in 2007 to 304.780 inhabitants by 2039. The total size of the population in the County is going to be reduced from 644.236 inhabitants in year 2007 to 585.181 inhabitants by 2039. The Figure 7.1-1 describes the demographic evolution in the Arges County in the period of project.

Figure 7.1-1 Population forecast for Arges County

## Collection and Transport

At this point of the work, a differentiation between Project Phases should be done. The measures corresponding to solve waste collection and transport in Zones of Phase-I of the project, i.e. Zones 3,4,5,6,& 7, have been already designed and tendered, and they are currently in the implementation stage. Therefore, the pre-design of these measures in Zones 3,4,5,6 & 7 is not taken into a consideration in this work, and only the Zones corresponding the Phase-II of the project, i.e. Zones 1,2 & 8, are pre-designed.

### Waste Storage for Residual Waste (domiciliary + bring system)

Urban and rural areas will implement a combined system, implementing the domiciliary, collection platforms and communal points (group of containers), in order to serve of waste collection to the appointed Zones. Therefore, the following arrangements will be implemented:

| Domiciliary | Collection Platforms |  | Communal points |
| --- | --- | --- | --- |
|  | Type A | Type B |  |
| - for urban area 1 bin of 120 l for residual waste; - for rural area 1 bin of 120 l for residual waste, as follows: - In Zone 1 for about 70% of households; - In Zone 2 for about 18% of households*; - In Zone 8 for about 22% of households. | - 1 x 1,1 m3 Container for residual waste | - 1 x 1,1 m3 Container for residual waste | - for urban area 1 x 1,1 m3 Container for residual waste; - for rural area about between 2 and 3 x 1,1 m3 Container for residual waste. |

* in zone 2 for the following localities Corbi, Domnesti and Pietrosani the residual waste collection at domiciliary will remain 100%, and for the 18% from the remaining localities will be provided with one bin of 120 l per household

In the case of Collection platforms and communal points, they will be situated at about every 150 to 200 m to serve on average app. 35 people per container. There will be app. 104 collection points for residual waste for urban area and app. 795 in rural areas.

### Waste Separate Storage of Recyclable Streams (domiciliary + bring system)

Urban and rural areas will implement a combined system, implementing the domiciliary, collection platforms and communal points (group of containers), in order to serve of waste collection to the appointed Zones. Therefore, the following arrangements will be implemented:

| Domiciliary | Collection Platforms |  | Communal Points |
| --- | --- | --- | --- |
|  | Type A | Type B |  |
| - 1 bin of 120 L for recyclables | - 1 x 1,1 m3 Container for plastic/metal - 1 x 1,1 m3 Container for paper/cardboard - 1 x 1,1 m3 Container for glass | - 1 x 1,1 m3 Container for recyclables | - 1 x 1,1 m3 Container for plastic/metal - 1 x 1,1 m3 Container for paper/cardboard - 1 x 1,1 m3 Container for glass |

There will be app. 104 collection points (collection platforms + communal points) for recyclable waste streams for urban area and app. 795  in rural areas.

Table 7.2-1 Endowments for waste storage in Phase-II

|  | Type | Bring system |  |  | Criteria for System Implementation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Storage type: | Separated |  |  | Zone | Domiciliary | Platforms | No Platforms | Sum |  |  |  |
|  | Container: | 1,1 | m3 |  | Z11 | 78% | 15% | 7% | 100% |  |  |  |
|  | Bins | 0,12 | L |  | Z22 | 26% | 50% | 24% | 100% |  |  |  |
|  | Density | 0,35 | ton/m3 |  | Z83 | 23% | 20% | 57% | 100% |  |  |  |
|  | Households | 3 | Inh/house |  |  |  |  |  |  |  |  |  |
|  | Coverage | 100 | Inh/container |  |  |  |  |  |  |  |  |  |
|  | HH Bins | 3 | Bins (Reciclables, Residual, Urban organic ) |  |  |  |  |  |  |  |  |  |
| Amount of waste |  |  | Ton/year | m3/year | Generated volume | Domiciliary | Volume for Communal |  | Volume for Domiciliary |  |  |  |
|  | Population 2013 |  |  |  |  |  | Platforms | No Platforms |  | Platforms | No Platfroms |  |
|  | Urban | Rural |  |  | m3/Week | m3/Week | m3/Week | m3/Week | Bins | Units | Units | Containers |
| Z-1 | 32.649 | 9.219 | 18.546 | 52.989 | 1.016 | 813 | 152 | 51 | 32.764 | 27 | 90 | 109 |
| Z-2 | 0 | 26.981 | 6.957 | 19.876 | 381 | 114 | 191 | 76 | 6.960 | 92 | 184 | 197 |
| Z-8 | 10.604 | 57.945 | 22.053 | 63.008 | 1.208 | 363 | 242 | 604 | 15.669 | 86 | 187 | 1004 |

*Population without PHARE Projects

1 The current system around Curtea de Arges City has already domiciliary

2 The mountain terrain of Domensti make necesary the platforms to hold containers

3 The terrain is flat, thus not big number of platforms are foressen

### Waste Separate Storage of Household Biodegradable

The collection of biodegradable waste in urban and rural areas will be as follows:

- Urban area: organic waste will be collected from Collection platforms, communal points and individual houses (only in urban areas), waste from parks and gardens as well as markets;

- Rural area: Implementation of home composting for individual Households. For this purpose, a number of 20,500 composters will be distributed in the rural area. waste from parks and gardens as well as markets;

- Supply of transport means for the transfer stations to the landfill

| Domiciliary | Collection Platforms |  | Communal Points |
| --- | --- | --- | --- |
|  | Type A | Type B |  |
| - 1 bin of 120 L for organic (only in urban areas) | - 1 x 1,1 m3 Container for organic (only in urban areas) | - 1 x 1,1 m3 Container for organic only in urban areas | - 1 x 1,1 m3 Container for organic (only in urban areas) |

### Waste Collection

For the implementation of a waste collection system, it has to be considered that in Romania there is a restriction for 2-axle trucks of between 15 and 16 tons and for 3-axle trucks of between 19.5 and 22.5tons, because for the collection of waste generally the requirements for unpaved and upgraded road types will be taken. The use of these truck types is therefore feasible. The specifications applying to the trucks are given in the following table

Table 7. 2-2	Design criteria for trucks in Phase-II

| Criteria | Comment |
| --- | --- |
| Average speed | 40 Km/h |
| Shift | 1 |
| Working days | 300 |
| Availability | 95% |
| Waste collected | - 24m³ trucks: 71.82 ton/week 16m³ trucks: 47,88 ton/week |
| gross weight | - 24m³ trucks: max. gross weight of 22.5t - 16m³ trucks max. gross weight of 19.5t |
| Payload | - 24m³ trucks- max. payload of 12t - 16m³ trucks max. payload of 8t |
| Emission control | Minimum Euro 4 filter |
| Compaction | Minimum compaction of 1:2 is required Compaction increases the density of collected residues from about 0.25 kg/l to about 0.50 kg/l. |
| Bin lifts | Lift devices able to hook standardized containers of 1,100l, 240l and 120l (240l and 120l are used in the existing waste collection system, need also to be hooked up) |
| Propulsion | Recommendation: in rural areas use 6x2*4 or 4x2*4 trucks, in urban areas one axle driven trucks (6*2) |
| installed CB radio device | For communication between the headquarter and the drivers |
| Drivers cabin | Air conditioned |
| Rear- end- Loader - catch basin at back end | - deep catch basin needed, so that bulky waste can be thrown by hand into the truck - when driving, the collection body has to be closed, to avoid losing collected waste |

Note: These trucks have to meet the requirements of the Government Ordinance No. 43/1997 regarding road status

Table 7.2-3 Number of collection trucks to be used in Phase-II

| Zone 1-Curtea de Arges |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coverage Time/trip | 12,00 | hrs |  |  |  |  |  |  |
| Trips/truck-shift | 0,67 | Trips |  |  |  |  |  |  |
| Km travelled | 88.350 | km/year-truck |  |  |  |  |  |  |
| Emission CO2 | 33 | ton CO2/year-truck |  |  |  |  |  |  |
| Population 2013 |  | Amounts (ton/week) |  |  |  |  |  |  |
| Lenght (km) | 465 | Organic | Glass | Plastic | Paper | Residues | CW | HHW |
| Total | 74.925 |  |  |  |  |  |  |  |
| Urban | 32.649 | 169 | 22 | 29 | 23 | 64 | 44 | 2 |
| Rural | 42.276 | 34 | 11 | 15 | 12 | 30 | 23 | 1 |
| Trucks (Units) |  |  |  |  |  |  |  |  |
| Type | Organic | Recyclables |  |  | Residues | Total | HHW |  |
| 24 m3 | 4 | 2 |  |  | 2 | 8 | 1 |  |
| 16 m3 | 2 | 2 |  |  | 1 | 5 | 0 |  |
|  |  |  |  |  |  | 13 | 1 |  |
| Emissions CO2 (ton/year) |  |  |  |  |  | 425 | 25 |  |
| Zone 2-Domnesti |  |  |  |  |  |  |  |  |
| Coverage Time/trip | 7,00 | hrs |  |  |  |  |  |  |
| Trips/truck-shift | 1,14 | Trips |  |  |  |  |  |  |
| Km travelled | 85.533 | km/year-truck |  |  |  |  |  |  |
| Emission CO2 | 32 | ton CO2/year-truck |  |  |  |  |  |  |
| Population 2013 |  | Amounts (ton/week) |  |  |  |  |  |  |
| Lenght (km) | 263 | Organic | Glass | Plastic | Paper | Residues | CW | HHW |
| Total | 26.981 |  |  |  |  |  |  |  |
| Urban | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Rural | 26.981 | 21 | 7 | 9 | 7 | 18 | 14 | 1 |
| Trucks (Units) |  |  |  |  |  |  |  |  |
| Type | Organic | Recyclables |  |  | Residues | Total | HHW |  |
| 24 m3 | 0 | 0 |  |  | 0 | 0 | 0 |  |
| 16 m3 | 1 | 1 |  |  | 1 | 3 | 0 |  |
|  |  |  |  |  |  | 3 | 0 |  |
| Zone 8-Costesti |  |  |  |  |  |  |  |  |
| Coverage Time/trip | 23,00 | hrs |  |  |  |  |  |  |
| Trips/truck-shift | 0,35 | Trips |  |  |  |  |  |  |
| Km travelled | 87.631 | km/year-truck |  |  |  |  |  |  |
| Emission CO2 | 32 | ton CO2/year-truck |  |  |  |  |  |  |
| Population 2013 |  | Amounts (ton/week) |  |  |  |  |  |  |
| Lenght (km) | 884 | Organic | Glass | Plastic | Paper | Residues | CW | HHW |
| Total | 68.549 |  |  |  |  |  |  |  |
| Urban | 10.604 | 12 | 1 | 2 | 2 | 4 | 3 | 1 |
| Rural | 57.945 | 42 | 14 | 19 | 15 | 37 | 28 | 2 |
| Trucks (Units) |  |  |  |  |  |  |  |  |
| Type | Organic | Recyclables |  |  | Residues | Total | HHW |  |
| 24 m3 | 1 | 1 |  |  | 1 | 3 | 1 |  |
| 16 m3 | 3 | 3 |  |  | 3 | 9 | 0 |  |
|  |  |  |  |  |  | 12 | 1 |  |
|  |  |  | Emissions CO2 (ton/year) |  |  | 447 | 32 |  |
|  |  |  |  |  | Type | Trucks | HHW |  |
|  |  |  |  |  | 24 m3 | 11 | 2 |  |
|  |  |  |  |  | 16 m3 | 17 | 0 |  |
|  |  |  |  |  | Total | 28 | 2 |  |

As vehicles (apart from for HHW) are not to be procured within the project, this data is for indicative planning purposes.  Operators may be able to improve the efficiency of vehicle utilisation.

### Waste transfer

In order to contribute to the recycling and biodegradable waste diversion targets set by the CWMP in line with the NWMP transposing the relevant acquis, significant investments are needed in Arges county. The following are included in the project:

- Two new transfer stations: Curtea de Arges (19.000 t/a) and Costesti (10.000 t/a);

- Two new composting plants (windrow composting): Curtea de Arges (13.000 t/a) and Costesti (6.000 t/a);

The transfer station in Curtea de Arges will receive the waste streams from Zones 1 & 2, including all their communes and villages, as this is the central point to distribute the different sub-products to their further treatment, and the residual waste to be disposed off in Albota Landfill. Hence, the maximal amount of material, for recovery and for final disposal, by this transfer station is around 108 tons/day in year 2009 and around 55 tons/day by year 2039, including the amount of sub-products for recovering processes (see Annex 7.2-1).

For this purpose, the Transfer Station in Curtea de Arges is foreseen the requirement of five collection routes to serve the Zones 1 & 2. The Covered population, communes and the share in collection amounts is presented in below.

| TS-Curtea de Arges | TS-Costesti |
| --- | --- |

Figure 7.2-1 Collection Routes for Transfer Station in Curtea de Arges & Costesti

Table 7.2-4 Amounts of waste being handled by transfer stations in Curtea de Arges & Costesti

| Year | TS-1 Curtea de Arges |  | TS-8 Costesti |  | Total (ton) |
| --- | --- | --- | --- | --- | --- |
|  | Population covered | Amounts (ton) | Population covered | Amounts (ton) |  |
| 2010 | 102.829 | 31.404 | 69.170 | 13.761 | 45.165 |
| 2013 | 101.906 | 36.583 | 68.549 | 17.907 | 54.490 |
| 2016 | 100.992 | 37.221 | 67.934 | 18.323 | 55.543 |
| 2020 | 99.785 | 37.956 | 67.122 | 18.828 | 56.784 |
| 2025 | 98.298 | 38.749 | 66.122 | 19.353 | 58.101 |
| 2030 | 96.832 | 39.146 | 65.136 | 19.702 | 58.848 |
| 2035 | 95.388 | 39.283 | 64.165 | 19.872 | 59.155 |
| 2039 | 94.249 | 39.325 | 63.398 | 19.952 | 59.276 |

## Landfill Design

### Size of the Cell-2

In the sight of the findings of the Master Plan of Arges County, regarding the capacity depletion of Cell-1 in Regional Landfill in Albota, the Phase-II of the Integrated Waste Management Project in Arges County takes into a consideration the Cell-2 of the Regional Landfill in Albota. The capacity of the Cell-2 is foreseen around the 1,1 million cubic meters.

At the beginning of the project, the Cell-2 Component was thought to be financed by the revenues of the operation of the IWMS Phase-I of Arges County. However, the results of a FEA for the operation of IWMS-Phase-I shows that this situation is not possible. Hence, it is recommended that Cell-2 would be financed by a grant component. Parameters design of Cell-2:

- Capacity: 1,1 Mio m3 (10 years)

- Location: N-W side of Cell-1

- Zones to be covered:  Entire Arges County (Zones 1,2,3,4,5,6,7&8)

- Type of waste: segregated municipal waste

- Maximal height: 30 m

- Compaction ratio: 1,2 to 1,4 ton/m3

- Daily cover ratio: 1/6

The capacity depletion of Cell-1 and the expected for Cell-2 is presented in the following table (see Annex 7.3-1).

Table 7.3-1 Capacity depletion of Cell-1 & Cell-2 of Albota Landfill

| End of Year | Relocation of “Old Waste” (m3) |  | Final Disposal of “New Waste” 2 |  | Sludge from WWTP | Total | Accumulated | Depletion of Cell-1 | Depletion of Cell-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m3) |  | (m3) | (m3) | (m3) | (m3) | (m3) |
|  | PHASE-I | PHASE-II | PHASE-I | PHASE-II |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 750.000 |  |
| 2009 | 20001 | 0 | 0 | 0 |  | 2.000 | 2.000 | 748.000 |  |
| 2010 | 1780001 | 0 | 98.886 | 0 |  | 276.886 | 278.886 | 471.114 |  |
| 2011 | 0 | 103.900 | 102.923 | 0 | 3.150 | 209.973 | 488.860 | 261.140 |  |
| 2012 | 0 | 0 | 104.623 | 25.024 | 3.150 | 132.797 | 621.657 | 128.343 |  |
| 2013 | 0 | 0 | 93.285 | 23.387 | 3.150 | 119.822 | 741.479 | 8.521 | 1.100.0004 |
| 2014 | 0 | 0 | 93.836 | 23.449 | 3.150 | 120.435 | 861.913 | -111.913 | 988.087 |
| 2015 | 0 | 0 | 94.133 | 22.954 | 3.150 | 120.236 | 982.150 |  | 867.850 |
| 2016 | 0 | 0 | 89.731 | 21.757 | 3.150 | 114.638 | 1.096.788 |  | 753.212 |
| 2017 | 0 | 0 | 90.089 | 20.838 | 3.150 | 114.076 | 1.210.864 |  | 639.136 |
| 2018 | 0 | 0 | 90.451 | 20.988 | 3.150 | 114.589 | 1.325.452 |  | 524.548 |
| 2019 | 0 | 0 | 90.818 | 21.143 | 3.150 | 115.111 | 1.440.563 |  | 409.437 |
| 2020 | 0 | 1 | 91.189 | 21.304 | 3.150 | 115.644 | 1.556.207 |  | 293.793 |
| 2021 | 0 | 2 | 91.489 | 21.419 | 3.150 | 116.061 | 1.672.268 |  | 177.732 |
| 2022 | 0 | 3 | 91.791 | 21.538 | 3.150 | 116.482 | 1.788.750 |  | 61.250 |
| 2023 | 0 | 4 | 92.096 | 21.659 | 3.150 | 116.909 | 1.905.659 |  | -55.659 |
| 1 Old waste from rural and urban dumpsites in Phase-I areas of Arges County 2 The disposal density is considered as 1.5 ton/m3 |  |  |  |  | 3 Initial capacity of Cell-1 = 750.000 m3 4 Initial capacity of Cell-2 = 1.100.000 m3 |  |  |  |  |

Table 7.3-2 Share of amounts for landfilling in Arges County

| Year | Zone 1 Curtea de arges | Zone 2 Domnesti | Zone 3 Campulung | Zone 4 Rucar | Zone 5 Mioveni | Zone 6 Pitesti | Zone 7 Tpoloveni | Zone 8-costesti | Total (ton) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2010 | 23.667 | 2.166 | 21.645 | 544 | 21.971 | 111.091 | 7.812 | 11.237 | 200.134 |
| 2013 | 22.483 | 3.327 | 23.275 | 920 | 20.847 | 100.599 | 8.184 | 12.755 | 192.392 |
| 2016 | 20.642 | 3.233 | 22.510 | 963 | 20.140 | 96.306 | 8.048 | 12.002 | 183.843 |
| 2020 | 20.012 | 3.266 | 22.936 | 1.017 | 20.511 | 97.640 | 8.266 | 11.852 | 185.501 |
| 2025 | 20.508 | 3.396 | 23.374 | 1.070 | 20.893 | 99.064 | 8.484 | 12.226 | 189.015 |
| 2030 | 20.810 | 3.502 | 23.555 | 1.115 | 21.043 | 99.328 | 8.617 | 12.497 | 190.467 |
| 2035 | 20.921 | 3.559 | 23.582 | 1.141 | 21.060 | 99.110 | 8.672 | 12.627 | 190.672 |
| 2039 | 20.952 | 3.588 | 23.568 | 1.154 | 21.044 | 98.865 | 8.692 | 12.684 | 190.546 |

### Bottom Lining Construction

In the case of the Cell-2 the base will follow the existing inclination of the terrain, for the side corresponding to natural terrain in the northwest part of the place. In the southeast part, the landfill base will be limited by the slope of the Cell-1.

The existing subsoil in the area of the Landfill consists of material suitable for a geological barrier which therefore may also be used for this adjustment in the slope area.

The base with a flat inclination will be designed with a straight crossfall. The width of the individual fields amounts to 28 m each, whereas the low point is always in the middle of the field. Later on, the leachate collection line will be installed there. Earth has to be excavated and built in again for the levelling. The exceeding earth has to be stored alongside the construction field and to be used for the construction of the landfill embankments and for the adjustment of the level. All excavated and filled areas have to be compacted.

The material for the polymer liner shall be High Density Poly Ethylene (HDPE) with 2.0mm both-sides textured and non textured HDPE.

The protection geotextile and the drainage material shall be installed very carefully starting from bottom to top of the slope. On the top of the slope the liner shall be fixed in an anchoring trench. The protective geotextile shall be a PE non-woven needle-punched product. The material shall comply with the requirements of the following table.

### Leachate Drainage, Collection and Treatment

The leachate drainage layer has a thickness of 50 cm and consists of gravel with a diameter. The drainage layer has to be installed and the material has to be tested in accordance with the requirements of the CQA Plan. The minimum requisite thickness of 50 cm must be proven by tests of 16 – 32 mm complying with the requirements of the following table:

| Property | Test Method (STAS) | Required Value |
| --- | --- | --- |
| Particle Size Distribution | 1913/5-85 | 16 – 32 mm (nominal) >= 10% < 10mm >= 5% < 63mm |
| Crushing Strength | 9850/89 | >= 50 kN |
| Carbonate Content | 7107/3-74 | <= 10 % |

The leachate collection system consists of two parts:

- leachate collection on the base of the new landfill cell,

- leachate collection on the toe of the slope between the old and the new landfill, in order to prevent the infiltration of leachate below the geomembrane sealing.

The leachate in the individual landfill cells will be collected via drain pipes 355 x 32.2 made of PE 100 (PN 16).  All drain pipes may be inspected and cleaned with a high pressure cleaner via the manholes. The manholes (inspection pits) are made of PE, with a nominal diameter of 1 m.

In addition, for those pipes, which due to their big length the drain pipes of the new landfill cell may be accessed via a flushing opening at the upper edge of the slope.

On the slope, a drain pipe 250 x 22.7 made of PE 100 (PN 16) will be installed. Two thirds of the circumference of the pipes is perforated. The drain slots have to meet the following requirements:

Width of drain slots: 12 mm

Length of drain slots: 35 - 50 mm

Distance between slots: min. 40 mm

Area of slots per m: min. 300 cm²/m.

The pump of leachate will be performed through a pumping station, from the toe of the old landfill to the leachate tank and one pumping station to pump the leachate from the storage tank to the leachate treatment plant, which will be already working when Cell-2 will be constructed. Two redundant pumps will be permanently installed in each manhole. The pumps should be corrosion resistant for pumping the leachate.

The tank for the leachate consists of a PE pipe with a diameter of 2.5 m, installed underground along of one side of the landfill. At each junction to a leachate pipe from the landfill area, the tank will be equipped with a manhole with a diameter of 2 m, where the cocks and a ladder are installed. The tank should have negative buoyancy; therefore it gets an uplift protection. The tank for the water from the treatment facility will be designed like the tank for the leachate storage facility. It will be equipped with a manhole at the beginning and at the end of the tank.

### Landfill Gas Treatment

The landfill gas generated by the disposed waste in Cell-2 is going to be treated in the same facility constructed for Cell-1. Therefore, The landfill gas is to be collected and to be treated in an extraction facility with an access gas burner in order to prevent emissions. The methodology to follow for this issue is as follows:

- Condensation. There are two mechanisms of the generation of condensate on the intake side of the protective  degasification facility:

  - Condensation of the super-saturation portion of water vapour generated during the cooling of the gas

  - Precipitation of drops at widening of the cross sections of the pipes, at loop pipes and built-in parts when the flow velocity of the gas is reduced (gravity precipitation).

The gas condensate set free runs on the bottom of the gas pipe in free drop to the next low point, at which the condensate is precipitated from the gas system. Since there is always low pressure on the intake side of the degasification facility, the precipitator has to be vacuum-tight.

The landfill gas condensate is composed of (according to the two mechanisms mentioned above) water (main component), steam distillable components (e. g. ammonium), condensable elements (e.g. organo-chlorines, chlorofluorohydrocarbon) and landfill leachate (if entrained during the extraction).

Due to its components, the precipitated condensate is conveyed to the leachate and treated at the leachate treatment plant, already in function in the regional landfill of Albota

### Capping System Closure of Landfill

The final covering layer is realized in accordance with the following:

a   temporary covering layer (20 cm)

b  drainage layer (30 cm)

c   mineral layer (clay, alluvial land)(50 cm)

d   protective geotextile

e   drainage layer 30 cm

f    separation geotextile

g   un-compacted sand/gravel (70 cm)

h   top soil (30 cm)

The proposed solution for the replacement, proposed by the Designer, foresaw the execution of the final system in two steps:

    - First step – temporary sealing through the laying of two layers, part of the final sealing (a+b);

    - Second step – completion of the sealing system with the c, d, e, f, g, and h layers. The execution of the second step takes place after the significant settlement stops. From experience with settlement measurements in international projects, this period is to be between 3-5 years, in minimum.

## Closure of Disposal Sites in Urban Areas

### General

Old landfills with the size / surface area exceeds, or will exceed at the date of closure, the values given in Ordinance No. 1274/2005, will be under Romanian Landfill Directive (Order of the Ministry of Environment and Water Management no. 757 / 2004 approving technical standards for the storage of waste (Official Gazette no. 86 26.01. 2005), including provision of the monitoring system, within this project. In Arges this will apply to Curtea de Arges, Campulung, Mioveni, Topoloveni, Albota and Costesti. In Phase-I of the project, the landfills from Campulung, Mioveni, Topoloveni and Albota will be closed. The Phase-II of the project will full closure Curtea de Arges landfill and relocate the Costesti landfill.

### Curtea de Arges Site

#### Curtea de Arges

The main disposal site in Zone 1-Curtea de Arges is the one located in the west part, at the fringes of the Curtea de Arges city, on the 73 C national road Curtea de Arges – Ramnicu Valcea. The site is owned by the municipality of Curtea de Arges and operated by S.C. TRANSARG CAG '98 Company.

The site started operations in year 1952, in a total uncontrolled manner, thus there was no register of the amount of disposed waste and no works for environmental protection infrastructure were done. Currently, the final disposal site has a total surface of 4 ha., from which 2,1 hectares is placed the current depositing platform. It is estimated a volume of disposed waste around 450.000 m3. In the surroundings of the site there are no habitations or economic sites, just forest, orchards and infields surround the deposit.

The site has in the lowest point of the platform a collection structure for leachates. The leachate is conducted to the Waste Water Treatment Plant of Curtea de Arges. The estimated amount of collected leachate is 87 m3/year.

Figure 7.4-1 Final disposal site in Curtea de Arges (see Annex 7.4-1)

In 1996 took place some works to modernize the final disposal site in Curtea de Arges. The works included an access road construction, stockpiling of recyclable waste of 18 m x 6 m, a support wall at the base of the dust hole and a tank for leachate pre-treatment. The works were stopped as a result of lack of funds.

### Costesti

The final disposal site for the solid waste generated in Costesti locality started operating in year 1994. It is located at the fringes of the town, with a distance of approximately of 5 km to the habitation zone. The site has an approximately surface of 3 ha. and the amount of waste is around of 103.900 m3.

Figure 7.4-2 Final disposal site of Costesti city (see Annex 7.4-2)

The site is owned by the municipality, but there is performed no operations related to the final disposal of waste.  This lack of control leads to the scattering of waste within a extended area.

## Biological Treatment

The main design criteria are the amount of separated biodegradable waste, which reaches the transfer stations. These amounts correspond to the evolution of Waste composition during the project period that is presented in Section 5.2. The resulting amounts are presented in the next table and detailed data in Annex 7.5-1.

Table 7.5-1 Potential Composting production in Zones of Arges County

| Year | Total inhab | Input for individual Composting in Rural areas (ton / year) |  |  |  |  |  |  |  | Biodegradable for Compost Plants (ton / year) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Phase-II |  |  | Phase-I |  |  |  |  | Phase-II | Phase-I | Total |
|  |  | Z-1 | Z-2 | Z-8 | Z-3 | Z-4 | Z-5 | Z-6 | Z-7 | Z1, Z2 & Z8 | Z3, Z4, Z5, Z6 & Z7 |  |
| 2010 | 638.455 | 2.887 | 1.842 | 3.957 | 2.968 | 646 | 2.502 | 5.697 | 2.005 | 18.003 | 23.974 | 41.978 |
| 2013 | 632.726 | 4.884 | 3.117 | 6.694 | 5.020 | 1.093 | 4.232 | 9.638 | 3.391 | 20.454 | 30.512 | 50.965 |
| 2016 | 627.049 | 4.774 | 3.047 | 6.543 | 4.907 | 1.068 | 4.137 | 9.421 | 3.315 | 19.499 | 37.220 | 56.719 |
| 2020 | 619.558 | 4.615 | 2.945 | 6.326 | 4.744 | 1.033 | 3.999 | 9.108 | 3.205 | 18.187 | 35.266 | 53.454 |
| 2025 | 610.320 | 4.499 | 2.871 | 6.167 | 4.625 | 1.007 | 3.899 | 8.879 | 3.124 | 17.108 | 33.698 | 50.806 |
| 2030 | 601.220 | 4.291 | 2.738 | 5.881 | 4.411 | 960 | 3.718 | 8.468 | 2.980 | 15.669 | 31.510 | 47.179 |
| 2035 | 592.256 | 4.158 | 2.654 | 5.699 | 4.274 | 930 | 3.603 | 8.206 | 2.888 | 14.720 | 30.044 | 44.764 |
| 2039 | 585.181 | 4.094 | 2.613 | 5.611 | 4.208 | 916 | 3.547 | 8.079 | 2.843 | 14.201 | 29.234 | 43.435 |

## Sorting Plant

The Phase-II of the Waste Management Project in Arges County includes the extension of the sorting plant at Regional Landfill in Albota. The extension consists in a capacity of approx. 60 tons/day-shift (equalizes to approx. 8 tons/hour) of domestic waste to be sorted. At the sorting plant paper & cardboard and plastic & metal (light fraction) are sorted separately to increase the material quality and to fulfil recycler’s demands. The quantity of the input material is the main design criteria for the capacity and number of shifts of the sorting plant.

Table 7.6-1 Amounts of waste streams arriving to Transfer stations in Phase-I & Phase-II in Arges

| Year | Organic for Compost |  | Paper |  | Glass |  | Metal |  | Plastic |  | Wood |  | Other |  | Residue |  | CW |  | Total |  | Total County |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ph-I | Ph-II | Ph-I | Ph-II | Ph-I | Ph-II | Ph-I | Ph-II | Ph-I | Ph-II | Ph-I | Ph-II | Ph-I | Ph-II | Ph-I | Ph-II | Ph-I | Ph-II | Ph-I | Ph-II |  |
| 2010 | 93.951 | 18.003 | 13.689 | 3.090 | 12.645 | 2.854 | 7.889 | 1.780 | 16.821 | 3.797 | 6.960 | 1.571 | 24.074 | 5.321 | 12.760 | 2.868 | 26.160 | 5.881 | 214.950 | 45.165 | 260.115 |
| 2013 | 98.829 | 20.454 | 15.083 | 3.883 | 13.932 | 3.587 | 8.692 | 2.238 | 18.534 | 4.772 | 7.669 | 1.974 | 26.360 | 6.610 | 14.042 | 3.597 | 28.789 | 7.375 | 231.930 | 54.490 | 286.419 |
| 2016 | 94.134 | 19.499 | 16.876 | 4.348 | 15.589 | 4.016 | 9.725 | 2.506 | 20.737 | 5.343 | 8.581 | 2.211 | 26.036 | 6.535 | 14.169 | 3.630 | 29.105 | 7.456 | 234.952 | 55.543 | 290.495 |
| 2020 | 87.626 | 18.187 | 19.185 | 4.950 | 17.722 | 4.572 | 11.056 | 2.853 | 23.575 | 6.083 | 9.755 | 2.517 | 25.524 | 6.418 | 14.283 | 3.659 | 29.454 | 7.545 | 238.181 | 56.784 | 294.965 |
| 2025 | 82.243 | 17.108 | 21.331 | 5.513 | 19.704 | 5.093 | 12.292 | 3.177 | 26.211 | 6.775 | 10.846 | 2.803 | 24.910 | 6.277 | 14.428 | 3.696 | 29.896 | 7.659 | 241.860 | 58.101 | 299.962 |
| 2030 | 75.039 | 15.669 | 23.458 | 6.075 | 21.669 | 5.611 | 13.518 | 3.501 | 28.826 | 7.465 | 11.928 | 3.089 | 24.034 | 6.069 | 14.400 | 3.689 | 29.984 | 7.681 | 242.855 | 58.848 | 301.703 |
| 2035 | 70.303 | 14.720 | 24.770 | 6.426 | 22.881 | 5.936 | 14.274 | 3.703 | 30.438 | 7.897 | 12.595 | 3.268 | 23.131 | 5.853 | 14.330 | 3.671 | 29.983 | 7.681 | 242.705 | 59.155 | 301.859 |
| 2039 | 67.730 | 14.201 | 25.479 | 6.620 | 23.536 | 6.115 | 14.683 | 3.815 | 31.309 | 8.135 | 12.956 | 3.366 | 22.441 | 5.688 | 14.274 | 3.657 | 29.982 | 7.681 | 242.391 | 59.276 | 301.667 |

## Bring Centres

The Phase-II of the Integrated Waste Management Project in Arges County takes into a consideration the Zones 1,2 & 8, with a population of around 76, 27 and 69 thousands inhabitants. Therefore there is no city over 100 thousand inhabitants, which needs “Bring Centres”

For the Phase-II, the Green Collection points are though to work as complementary facility of the transfer stations. These collection points will receive separated waste streams, which are able for recycling processes. Such waste streams are going to be processed and handled as indicated by corresponding Directives as 75/439/CEEE, 2002/96/EC, 94/62/EC, etc. The waste streams foreseen for this facility are:

| - Paper - Plastic - Glass | - Household Hazardous Waste - Electric and Electronic Waste - Bulky Waste |
| --- | --- |

Table 7.7-1 Specific Waste streams for Civic Amenity facilities

| Year | 2010 | 2013 | 2016 | 2020 | 2025 | 2030 | 2035 | 2039 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HHW |  |  |  |  |  |  |  |  |
| Generation County (tons) | 1264 | 1252 | 1241 | 1226 | 1208 | 1190 | 1172 | 1158 |
| Generation Urban | 765 | 758 | 751 | 742 | 731 | 720 | 709 | 701 |
| Generation Rural | 499 | 494 | 490 | 484 | 477 | 470 | 463 | 457 |
| County Index (Kg/inh-y) | 1,98 | 1,98 | 1,98 | 1,98 | 1,98 | 1,98 | 1,98 | 1,98 |
| Index Urban (Kg/inh-y) | 2,5 | 2,5 | 2,5 | 2,5 | 2,5 | 2,5 | 2,5 | 2,5 |
| Index Rura (Kg/inh-y)l | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 |
| Bulky waste |  |  |  |  |  |  |  |  |
| Generation County (tons) | 1.302 | 1.290 | 1.274 | 1.259 | 1.240 | 1.218 | 1.199 | 1.185 |
| Generation Urban | 974 | 965 | 953 | 942 | 928 | 911 | 897 | 887 |
| Generation Rural | 328 | 325 | 321 | 317 | 312 | 307 | 302 | 298 |
| County Index (Kg/inh-y) | 2,04 | 2,04 | 2,03 | 2,03 | 2,03 | 2,03 | 2,03 | 2,03 |
| Index Urban (Kg/inh-y) | 3,18 | 3,18 | 3,17 | 3,17 | 3,17 | 3,16 | 3,16 | 3,16 |
| Index Rura (Kg/inh-y)l | 0,99 | 0,99 | 0,98 | 0,98 | 0,98 | 0,98 | 0,98 | 0,98 |
| Waste from Electric and Electronic Equipment (WEEE) |  |  |  |  |  |  |  |  |
| Arges County (Tons) | 2.554 | 3.796 | 3.762 | 3.717 | 3.662 | 3.607 | 3.554 | 3.511 |
| County Index (Kg/inh-y) | 4 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |

## Sludge treatment and Disposal

According to the above information, the Master Plan for WWTP of Arges County is not considering the landfill as an option for final disposal of Sludge from Water Treatment. However, recently three cities have approached the PIU, notifying that 6.100 ton/year of sludge is planned to be disposed in Albota landfill (see Annex 6.6-1). The amounts are the following:

Mioveni - 			3.000 ton/year

Curtea de Arges -		 3.000 ton/year

Campulung 		100 ton/years

Moreover, from the area that will be Operated by the Regional water Operator (Pitesti and surroundings, Costesti, Topoloveni, etc), there will be disposed 9.900 tons/year. So in total there will have 16.000 t/year of sludge to be disposed at landfill. For this reason, this amount is considered for waste landfill as follows:

Sludge: 			14.545.000 m3/year (35% TS)

Tons: 			16.000 tons/year (considering a density of 1,1 ton/m3)

Volume for disposal: 	5.090 m3/year (considering the 35% of TS)

The amount to be registered at Albota WMC is 16.000 ton/year (through weighbridge) for invoicing purposes. However, the volume to be able for disposal is 14.545 m3/year.

In theory the total mentioned volume of sludge may be used for composting purposes; this will depend on chemical composition of sludge as high concentrations of some components can danger the composting processes. In order to be conservative, the considered amount for using in Composting plants is 40%, thus for disposal will be only 60% of the total volume of sludge that is around 3.150 m3/year.

This volume for disposal represents approximately less than 3%, of annual volume of waste being disposed in regional landfill Albota, which is a very small volume and do not overpass the limit set by the corresponding legislation.

Surface fo transfer stations:

-Curtea de Arges: 6.725 m2 (Inlcuding composting plant, civic amenity areas and suplementary areas)

- Costesti: 5.187 m2 (Inlcuding composting plant, civic amenity areas and suplementary areas)

The Romanian Legislation by Order 757/2004 allows mixing sludge with household waste in a rate of 1:10 from the deposit rate